Code No. 221802

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

BASIC ELECTRONICS

(Common to Production Engineering, Mechanical Manufacturing Engineering and Metallurgy & Material Technology)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain operation of PN junction diode and plot the V-I curves for Ge and Si to the same scale showing the cut in value for each?

b)	Draw the circuit of a Half wave capacitive rectifier and explain its operation with wave forms?

2. a)	Sketch typical CE input and output characteristics for an NPN transistor.  Explain in brief.  Derive the relationship between beta and alpha of a transistor.

- Explain how transistor acts as an amplifier.

3. a)	Explain basic concepts in Feed Back.  What are the advantages of negative feed back.

- Explain the operation and frequency response of R-C coupled amplifier.

4. a)	What is Induction heating?  Explain the principle and theory in detail.

- List various methods of generating ultrosonics and describe one method in detail?

5. a)	Draw the neat sketch of CRT and explain the functions of various parts?

- Describe about strain gauges.  Derive expression for gauge factor.

6. a)	What are different types of Resistance welding Processes and explain each processes briefly?

- List the important electric welding controls?  Explain any three.

7. 	Explain the following

  - P & N type semi conductors

  - Wien bridge oscillator

  - LVDT.

8.	Explain the following:

  - SCR

  - Energy storage welding

  - Applications of CRO.

###

Code No. 221802

Set No.

2

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

BASIC ELECTRONICS

(Common to Production Engineering, Mechanical Manufacturing Engineering and Metallurgy & Material Technology)

# Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Explain the current Flow in a p-n Diode when it is

(i) Reverse biased	(ii) Forward biased, with diagrams and graph.

b)	Draw the circuit diagram of a Half wave Rectifier explain the operation, draw the output waveforms.  Derive the expressions for its d.c. output voltage, R.M.S current and voltage.

- Give the advantages and disadvantages of a Full wave Rectifier.

2. a)	Explain the operation of a Transistor as an Amplifier.  Derive the expression for its voltage gain.

b)	Explain Active, cut-off and saturation regions of a CE Transistor with the output characteristic curves.

- Draw the circuit diagram of a Transistor in common collector configuration explain the operation.

3. a)	Explain any Two applications of SCR with circuit diagrams.

b)	Draw a wien bridge oscillator circuit (using transistors) and explain how the requirements of an oscillator is fulfilled in the circuit.

4. a)	Draw the circuit diagram of a IC Electronic Timer and explain its operation and Draw the output waveforms.

- Explain Energy storage welding with circuits and diagrams.

5. a)	Explain the principle of Dielectric Heating.

- Explain the Applications of cathode Ray Oscilloscope.

- Explain the Measurement of Force using a Inductive Transducer with diagrams.

6. a)	Explain the Measurement of Displacement using Linear variable Differential Transformer with a diagram and graph.  Give its Merits and Demerits.

- Explain the measurement of Temperature using a Thermistor in a Bridge circuit.

7. a)	Write brief notes on the following

  - Ultrasonic as Means of Communication.

  - Testing of Materials by Ultrasonic Waves.

- Explain the measurement of Pressure using a capacitive Transducer.

8. a)	Explain the following in a p-n Junction diode.

  - Diode static Resistance

  - Dynamic Resistance

  - Piecewise Linear Approximation for a diode characteristic.

- Explain Electro-Mechanical Timers.

- Write brief notes on Photo-Voltaic cells.

###

Code No. 221802

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

BASIC ELECTRONICS

(Common to Production Engineering, Mechanical Manufacturing Engineering and Metallurgy & Material Technology)

# Time: 3 Hours							Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.  a)	Explain the need for doping in semiconductors.  Discuss how p-type and n-type semiconductors are formed.  Name at least two materials used for doping to make n-type and p-type semiconductors.

- Explain the operation of a P N junction diode with its V-I characteristics.  Describe how the characteristics vary with temperature.

2.  a)	Explain how a P N junction diode can be used as a rectifier.  Draw the circuit diagram of a full wave rectifier and explain its operation with wave forms.  Derive the expressions for its output voltage and ripple factor.

b)	Explain the necessity of filters in rectifier circuit.  Describe with a neat circuit diagram and wave forms how a -section filter reduces the ripple in the above rectifier output voltage.

3.  a)	Sketch the output static characteristics of a CE transistor.  Explain what do you understand by active, cut off and saturation regions.  Mark these regions on the characteristics.  Discuss the effect of temperature on the characteristics.

b)	Explain the need for biasing a transistor.  Draw the circuit diagram of an CE amplifier and explain its operation, with the help of its static characteristics and load line.

4.  a)	Describe the construction features of an SCR.  Sketch its V-I characteristics and justify the same.  Give and explain one typical industrial application of it.

- Describe the role of timer circuit in welding applications.  With a neat sketch explain the operation of the timer circuit.

5.  a)	Enumerate the advantages and disadvantages of negative feedback in amplifiers.

- An amplifier has an open loop voltage gain of 1000.  If a negative feed back of 10% of its output is applied.  Determine the gain of the amplifier with feedback.

- Draw the circuit diagram of a wien bridge oscillator and explain its operation with necessary mathematical equations.  What is its advantage over RC phase shift oscillator?

Contd..2

#### Code No. 221802				.2..				Set No. 3

6.  a)	With a neat sketch describe the constructional details of a cathode ray oscilloscope.  Explain how a CRO can be used for the measurement of phase difference between two sinusoidal wave forms of same frequency.

- Explain dielectric heating briefly.  Discuss the effect of frequency on dielectric heating?

7.  a)	What is ultrasonics?  Describe how ultrasonics are produced.  Explain one application of it with a neat sketch.

- Explain the construction features and principle of operation of a L.V.D.T. (Linear Variable Differential Transformer) for the measurement of small displacements.

8.	Write short notes on any THREE of the following:

- Diode as a switch    b) Energy storage welding  c) Strain gauge and its applications.  d)  light dimmer circuit using a SCR.

@@@

Code No.221802

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

BASIC ELECTRONICS

(Common to Production Engineering, Mechanical and Manufacturing Engineering, Metallurgy and Material Technology)

Time: 3 hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Draw the piece wise linear volt ampere characteristics of a P-n diode and explain.  What is the circuit model for ON and OFF states.

b)	Explain the terms: (i) Intrinsic and extrinsic semi conductors.  (ii) Fermi level.

2.a)	Draw the circuit for a FWR with α-section filter and derive the expression for ripple factor.

b)	Compare HWR, FWR and Bridge rectifier circuits.  What are their applications.

3.a)	Explain the family of CB Input and output characteristics of a Transistor.

b)	Justify how the Transistor can be used as an Amplifier.

4.a)	What are the Berkhausen conditions of oscillations in electronic system?  What are the factors which effects the frequency stability of an oscillator?

b)	Draw the circuit diagram of RC-phase shift oscillator and explain its operation.

5.a)	With a neat circuit diagram explain A.C. Resistance welding.

b)	What is the main disadvantages in single phase welding systems.  How it is eliminated in Energy storage welding systems.  Explain clearly any one energy storage welding.

6.a)	Explain the principle of Dielectric Heating and what are its Applications.

b)	Clearly explain what are the uses of Cathode Ray oscilloscope.

7.a)	Explain the classification of transducers with examples.

b)	Discuss how Inductive Transducer can be used for displacement.

8.	Write short notes on any TWO

i) CE Amplifier

ii) ON and OFF mechanism of SCR

iii) Ultrasonics and its applications.

- - -