Code No.220203 Set No. 1

II-B.Tech. II-Semester Supplementary Examination January - 2003

LINEAR DIGITAL I.C. APPLICATIONS

(Common to Electrical and Electronics Engineering and electronics and Computers Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Define the following electrical parameters of an op-amp input offset current, input bias current. Differential input resistance, input capacitance, offset voltage adjustment range and CMRR.

b)	Determine the output voltage in each of the following cases for the openloop differential amplifier shown in figure.

Case (i): Vin1 = 5v dc		Vin2 = -v dc

Case (ii): Vin1 = 10mvrms		Vin2 = 20 mvrms

The op-amp has the following specification A = 200000, Ri = 2MΩ, Ro = 75Ω, Vcc = +15V, VEE = -15V and output swing = ±14V.

2.a)	Derive closed loop voltage gain, input resistance, output resistance and bandwidth for inverting amplifier with feedback arrangement.

b)	Draw an integrator circuit and explain its operation. Discuss the frequency response for a practical integrator.

3.	Draw a circuit and explain in detail the two operating modes of the 555 timer with timing diagram.

Contd…….2

## Code No.220203			-2-			Set No.1

4.a)	Describe in detail about Binary weighted resistance D/A converter.

b)	Explain the principle of operation of single and dual slope analog to digital converter.

5.a)	Derive the transfer function, gain and phase angle for second order high pass active filter.

b)	Explain how Q, upper cutoff frequency and lower cutoff frequency is determined in Bandpass filter.

6.a)	Draw the circuit diagram of a low-pass sallen and key high pass filter and determine it’s gain.

b)	Draw the block diagram of a band rejection filter and explain it’s operation.

7.a)	Draw the cross section of a IIL inverter and explain its function.

b)	Show the interconnection diagram for a bistable latch in IIL.

8.a)	List in tabular form the fan out, power dissipation, propagation delay and noise margin for the following digital IC logic families. TTL and ECL.

b)	Explain with logic diagram and timing relationship of a master- slave flipflop and edge triggered flipflop.

***     ***     ***

Code No.220203 Set No. 2

II-B.Tech. II-Semester Supplementary Examination January - 2003

LINEAR DIGITAL I.C. APPLICATIONS

(Common to Electrical and Electronics Engineering and electronics and Computers Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Define the following electrical parameters of an op-amp Input offset voltage, Input resistance, CMRR, output voltage swing. Slew  rate, Supply voltage rejection ratio and Gain bandwidth Product.

b)	Determine the output voltage for the Inverting amplifier shown in figure (i) Case 1: Vin = 20mvDc	(ii) Case 2: Vin = -50v peak sine wave

Gain = 200,000 (for both case)

Vcc

+

A	 oVo

-

signal 		2k

source                     -VEE

2.a)	Derive closed loop voltage gain, input resistance, output resistance and Bandwidth for non inverting amplifier with feedback arrangement.

- Derive the voltage gain and input resistance for an Instrumentation amplifier.

3.a)	List the application of IC 565PLL and briefly describe the role of the PLL in any of that application.

- Refering to the circuit shown below determine the free running output, lock range and the capture range.

Contd…….2

Code No.220203			-2-			Set No.2

4.a)	Explain the operation of the successive approximation A/D converter, with a block diagram and timing diagram. Give it’s advantages.

b)	With help of a block diagram and timing diagram explain the operation of ramp type A/D converter.

5.	Derive the transfer function, gain and phase angle for first order and second order low pass active filter.

6.a)	Sketch the circuit of a logarithmic amplifier using one op-amp and explain its operation. State it’s application.

- What is meant by all pass filter. State it’s application.

- Draw a block diagram of a broad-band bandpass filter and explain it’s operation.

7.a)	Draw the block diagram for a ladder network whose resistance has one of two values R or 2R. Explain the operation of the converter.

- Sketch an analog multiplexer system and explain it’s operation.

- What limits the acquisition time in a sample and hold circuit.

8.a)	Define the terms with respect to logic familics fan out, power dissipation, propagation delay and noise margin.

b)	Explain open-collector output, totem pole output and tristate output with respect to TTL logic.

***     ***     ***

Code No.220203 Set No. 3

II-B.Tech. II-Semester Supplementary Examination January - 2003

LINEAR DIGITAL I.C. APPLICATIONS

(Common to Electrical and Electronics Engineering and electronics and Computers Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	With help of a block diagram explain the basic building blocks of an op-amp.

- What does the term ‘balanced output’ mean in an opamp.

- List the parameters that should be considered for AC and DC application.

- What are the three factors that affect the electrical parameters of an opamp.

2.a)	What is the name of the circuit that is used to detect the peak value of the nonsimusoidal input waveforms. Draw it’s circuit and explain it’s operation.

- Draw any one multivibrator circuit using op-amp and explain its operation.

3.a)	Give the Pin configuration of 555 timer and list it’s important features.

b)	Briefly explain the difference between the two operating modes of 555 timer. Explain any one of the modes in detail.

4.a)	What is the difference between A/D and D/A converter. Give an application for A/D and D/A converter.

b)	Define the following terms of D/A converter resolution, settling time and conversion time.

- What is a sample and hold circuit? Why is it needed. Draw a sample and hold circuit and explain it’s operation.

5.a)	Draw a bandpass filter circuit and show how upper and lower cutoff frequency is determined by selecting Q. Derive the transfer function and gain for the band pass filter.

b)	Draw a circuit diagram of a universal biquard filter and show how low pass, band pass and high pass output can be achieved simultaneously.

6.a)	Draw the block diagram for a 2-bit parallel-comparator A/D converter and explain the operation of the system.

b)	Draw a schematic diagram of a ladder network D/A converter. Explain the operation of the converter.

(contd…2)

Code No.220203			-2-				Set No.3

7.a)	Compare the relative merits of TTL and ECL logic families.

b)	Show how to obtain the NOR function A+B and NAND function ABin I I L.

8.a)	Explain the following terms pertaining to the logic families. TPHL, VOL, VIH, DC noise margin, A.C Noise margin.

- Give the block schematic of 556 function generator IC and explain it’s functions.

***     ***     ***

Code No.220203 Set No. 4

II-B.Tech. II-Semester Supplementary Examination January - 2003

LINEAR DIGITAL I.C. APPLICATIONS

(Common to Electrical and Electronics Engineering and electronics and Computers Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Draw the open loop configuration of inverting amplifier and non inverting amplifier. State why open loop configurations are not used in linear application.

- What are the characteristics of an ideal op-amp.

- What is meant by voltage transfer curve in an op-amp?

- Explain with block diagram the different feedback configuration in an op-amp.

2.a)	Draw the circuit of a square wave generator and explain it’s operation.

b)	Draw the circuit of a positive half wave and negative half wave rectifiers and explain it’s operation.

3.	Explain in detail one application each in which the 555 can be used as a monostable multivibrator and astable multivibrator.

4.a)	Draw the block diagram of integration type A/D converter and explain its operation. Give it’s advantage and disadvantages.

- With help of a block diagram explain in detail about parallel type A/D converter.

5.	Draw a band pass filter circuit and show how upper and lower cutoff frequency is determined by selecting Q. Derive the transfer function and gain for the band pass filter.

6.a)	Define by means of a diagram the pass band, stop band, transition band and pass band ripple.

b)	Sketch the ideal frequency-response characteristics of Low pass, highpass and band reject system.

- Draw the block diagram for a counting A/D converter and explain it’s operation.

7.a)	Sketch a 2 input OR ECL gate what parameter determine the noise margin explain why power line spikes are virtually non existent.

- List and discuss at least four advantages and four disadvantages of the ECL gates.

8.a)	Describe in detail about Binary weighted resistance D/A converter.

- Explain the principle of operation of single and dual slope ADC.

***     ***     ***