Code No.220451

OR

II-B.Tech. II-Semester Supplementary Examination January - 2003

SWITCHING THEORY AND LOGIC DESIGN

(Common to Electronics and Communications Engineering, Electronics Instrumentation Engineering ,Electronics and Control Engineering, Electronics & Telematics)

Time:3 hours								Max.Marks:70

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Construct a seven bit error correcting code (Hamming code) to represent the decimal digits by augmenting the 4 bit Gray code and by using odd-1 parity check.

- What is a Binary number system. Illustrate with an example.

2.a)	State the basic laws of Boolean algebra and explain.

- Minimise the following using the map-method.

BCD+ABD+ACD+ABC+ABCD

Optional combinations: ABCD,ABCD;ABCD

c)	Read the map given below:

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| CD 00 | 1 | - |  | 1 |
| 01 | - | 1 | 1 | - |
| 11 |  |  | 1 |  |
| 10 | 1 |  |  | 1 |

AB

3.a)	Describe the truth tables for 3-input AND, OR, NOR and NAND gates.

b)	Draw the ROM connections required for coding of a 7-segment display. How many matrix elements are required?

4.a)	Design a BCD counter with JK flip flops.

b)	Draw the diagram of a 4-bit Binary ripple counter using flip flops that trigger on the positive edge.

5.a)	What do you mean by a state diagram? Explain with illustrations.

- With an example explain State Table.

6.a)	Design a combinational circuit that converts a 4-bit array code numbers to a 4-bit straight Binary number. Implement the circuit with EX-OR gates.

- What are universal gates? What are their advantages?

(contd…2)

### Code No.220451				-2-			OR

7.a)	Design a modulo 6-counter which counts in the way given below. Use T-flip flop.

| Decimal | Gray |
| --- | --- |
| 0 | 000 |
| 1 | 001 |
| 2 | 011 |
| 3 | 010 |
| 4 | 110 |
| 5 | 111 |

- Explain the principle of 4-bit adder.

8.	Write notes on:

- In completely specified sequential machines.

- Decoder

- Capabilities and limitations of finite state machines.

***     ***     ***