Code No:222101

Set No:

1

II.B.Tech II-Semester. Supplementary Examinations. January,2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Distinguish between ideal fluids and real fluids.  Explain the importance of compressibility in fluid flow.

b)	If the stream function for steady flow is given by ψ = x2-y2 determine whether the flow is rotational or irrotational.  Then determine the velocity potential .

2.a)	What is Eulers equation of motion?  How will you obtain Bernoulli’s equation from it.

b)	The water is flowing through a pipe having diameters 20 cm and 15 cm at sections 1 and 2 respectively.  The rate of flow through pipe is 40 Lt/sec.  The section 1 is 6m above datum line and section 2 is 3m above the datum.  If the pressure at section 1 is 29.43 N/cm2.  Find the intensity of pressure at section 2.

3.a)	Define the terms drag and lift.

b)	A Flat plate 2mx2m moves at 40 km/hr in stationary air of density 1.25 kg/m3.  If the coefficient of drag and lift are 0.2 and 0.8 respectively.  Find (i) the lift force (ii) the drag force (iii) The resultant force (iv) The power required to keep the plate in motion.

4.a)	Explain the characteristics of laminar and turbulent boundary layer.

b)	Define boundary layer and  explain the fundamental causes of its existence.  Also discuss the various methods of controlling the boundary layer.

5.a)	Define the following coefficients (i) Coefficient of velocity (ii) Coefficient of contraction (iii) Coefficient of discharge.

b)	A 20x10 cm venture meter is provided in a vertical pipe line carrying oil of specific gravity 0.8 the flow being upwards.  The difference in elevation of the throat section and entrance section of the venture meter is 50 cm.  The differential U tube mercury monometer shows a gauge deflection of 40 cm.  Calculate (i) the discharge of oil (ii) The pressure difference between the entrance section and throat section Take Cd = 0.98 and specific gravity of mercury as 13.6.

6.a)	What is the chief difference between steep and non steep compression waves?  How are they formed.

b)	Air is discharged from a reservoir at Po= 6.91 bar and to = 325oC through a Nozzle to an exit pressure of 0.98 bar.  If the flow rate is 3600 kg/hr.  Determine for isentropic flow. (i) Throat area, pressure and velocity (ii) Exit area and Mach Number.

Contd….2

Code No:222101			-2-				Set No:1

7.a)	Write down Four basic equations which satisfy the state points before and after a normal shock wave.

b)	Describe two practical situations where oblique shock waves are produced.  How are strong and weak waves generated and how they effect the flow.

8.	Write short notes on any four of the following:

a)	Total pressure and centre of pressure

b)	Free vortex and forced vortex flows

c)	Boundary layer separation

d)	Rotameter

e)	Sub sonic and super sonic flows.

*********

Code No:222101

Set No

2    1

II.B.Tech II-Semester. Supplementary Examinations. January,2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the terms total pressure and centre of pressure.

b)	Determine the total pressure and depth of centre of pressure on a plane rectangular surface of 1m wide and 3m deep when its upper edge is horizontal and (a) coincider with water surface (b) 2m below the free water surface.

2.a)	Derive Bernoulli’s equation from Eulers equation of motion.

b)	A smooth wrought iron pipe 0.2m in diameter carries crude oil at a velocity of 2.5m/s.  What is the loss of head per 100 metres.  What is the power required to maintain the flow?  Assume kinematic viscosity for oil 0.4 stokes and specific gravity 0.9.

3.a)	What is kinetic energy correction factor?  Explain.

b)	A pipe through which water is flowing is having diameters 40cm and 20cm at the cross sections 1 and 2  respectively.  The velocity of water at section 1 is given 5m/sec.  Find the velocity head at sections 1 and 2 and also rate of discharge.

4.a)	What do you under stand by (i) Total drag on a body (ii) coefficient of drag (iii) coefficient of lift .

b)	A circular disc 4m in diameter is held normal to 30 m/sec wind of density 1.25 kg/m3.  If the coefficient of drag of disc = 1.1 what force is required to hold the disc at rest.

5.a)	Distinguish between manometers and mechanical gauges?  What are the different types of mechanical gauges.

b)	A U tube differential monometer connects two pressure pipes A and B.  A contains carbon tetra chloride having a specific gravity 1.594 under a pressure of 11.772 N/cm2 and pipe B contains oil of specific gravity 0.8 under a pressure of 11.772 N/cm2 .  The pipe A lies 2.5m above pipe B.  Find the difference of pressure measured by mercury as fluid filling U tube.

6.a)	Describe with the help of sketch the construction, operation and use of pitot static tube.

b)	Find the velocity of an oil through a pipe when the difference of mercury level in a differential U-tube manometer connected to the two tappings of the pitot tube is 15 cm.  Take specific gravity of oil=0.8 and coefficient of pitot tube as 0.98.

Contd…2

Code No:222101			-2-			                   Set No:2

7.a)	Derive the following relations for one dimensional isentropic flow.              dAA=dPPc2   (1-M2).

b)	The pressure, velocity and temperature of air (r=1.4, Cp=1.0 KJ/kg K ) at the entry of a Nozzle are 2 bar, 145 m/sec and 330 k.  The Nozzle exit pressure is 1.5 bar (i) What is the shape of the Nozzle (ii) Determine for isentropic flow the Mach number at entry and exit.

8.	Write short notes on any  FOUR  of the following:

a)	Steady and Unsteady flows.

b)	Laminar and turbulent flows.

c)	Boundary layer parameters

d)	Venturi meters

e)	Shock waves.

********

Code No:222101

Set No

3

II.B.Tech II-Semester. Supplementary Examinations. January,2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	One litre of Crude oil weighs 9.6 N.  Calculate its specific weight, density and specific gravity.

b)	Derive an expression for the force exerted on a submerged vertical plane surface by the static liquid and locate the position of centre of pressure.

2.a)	What are the different forms of energy in a flowing fluid.  Represent schematically the Bernoulli’s equation for flow through a tapering pipe and show the position of total energy line and the datum line.

b)	Oil of specific gravity 0.9 is flowing through a pipe having diameters 30 cm and 20 cm at sections 1 and 2 respectively.  The rate of flow through pipe is 50 Lt/sec.  The section 1 is 5m above datum line and section 2 is 3m above the datum, if the pressure at section 1 is 29.41 N/cm2.  Find the intensity of pressure at section 2.

3.a)	What is meant by Magnus effect?  How does circulation originate around an airfoil.

b)	Find the drag force difference on a flat plate of size 1.5 m x 1.5 m when the plate is moving at a speed of 5m/sec normal to its plate first in water and second in air of density 1.24 kg/m3.  coefficient of drag is given as 1.10.

4.a)	A thin plate is moving instill atmospheric air at a velocity of 4m/sec the length of the plate is 0.5m and width 0.4m.  Calculate the (i) thickness of the boundary layer at the end of the plate and (ii) drag force on an side of the plate.  Take density of air as 1.25 kg/m3 and  kinematic viscosity 0.15 stokes.

b)	What are the different methods of preventing the separation of boundary layers.

5.a)	Differentiate between (i) Absolute and gauge pressure (ii) Piezometer and pressure gauges.

b)	A Horizontal venturi meter with inlet and throat diameters 30 cm and 15 cm respectively is used to measure the flow  of water.  The reading of differential manometers  connected to inlet and throat is 10 cm of mercury.  Determine the rate of flow Take Cd = 0.98.

6.a)	Explain with a neat sketch laser dappler anemometer

b)	An orifice meter with orifice diameter 15 cm is inserted in a pipe of 30 cm diameter.  The pressure gauges fitted upstream and down stream of the orifice meter give readings of 14.715 N/cm2 and 9.81 N/cm2  respectively.  Find the rate  of flow of water through the pipe in Lt/sec.  Take Cd = 0.6.

Contd……2

Code No:222101					-2-			Set No:3

7.a)	Explain how strong compression and expansion waves are formed in a compressible fluid.

b)	Show that for inducing sonic velocity in a gas (r=1.4) the pressure ratio required for a compression wave is 4.77

8.	Write short notes on any FOUR of the following;

a)	Uniform or non uniform flows

b)	Reynolds  Number and Mach Number

c)	Lift and Drag coefficients

d)	Pitot static tube

e)	Static and stagnation Temperatures.

********

Code No:222101

Set No

4

II.B.Tech II-Semester. Supplementary Examinations. January,2003

FLUID MECHANICS AND MACHINERY

(Aeronautical Engineering)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Define the following Fluid properties.

(i) Weight density (ii) Specific volume (iii) Specific gravity of a fluid.

b)	A triangular gate which has a base of 1.5m and an attitude of 2m lies in a vertical plane.  The vertex of the gate is 1m below the surface in a tank which contains oil of specific gravity 0.8.  Find the force exerted by the oil on the gate and the position of the centre of pressure.

2.a)	Define the equation of continuity.  Obtain continuity equation for a 3 dimensional flow.

b)	A pipe line carrying oil of specific gravity 0.8 changes in diameter from 300 m.m at a position A to 500 m.m diameter to a position B which is 5m at a higher level.  If the pressures at A and B are 19.62 N/cm2 and 14.91 N/cm2  respectively and the discharge is 150 Lt/sec.  determine the loss of head and direction of flow.

3.a)	Explain the origin of circulation around an airfoil and discuss the effects of the angle of attack on flow pattern around the airfoil.

b)	A circular disc 3m in diameter is held normal to a 26.4 m/sec wind of density 1.2 kg/m3.  What force is required to hold it at rest?  Assume coefficient of drag of disc as 1.1.

4.a)	Sketch a Hot wire anemometer and explain its working.

b)	The pressure difference measured by the two tappings of a pitot static tube one tapping point upstream and other perpendicular to the flow placed in the centre of a pipe line of diameter 40 cm is 10 cm of water.  The mean velocity in the pipe is 0.75 times the central velocity.  Find the discharge through the pipe.  Take coefficient of pitot tube as 0.98.

5.a)	Write a note on wedge probes.

b)	A 30 cm x 15 cm venture meter is inserted in a vertical pipe carrying water flowing in the up ward direction.  A differential mercury manometer connected to the inlet and throat gives a reading of 30 cm.  Find the discharge Take Cd = 0.98

Contd…2

Code No:222101				-2-				Set No:4

6.a)	Derive the following relation for one dimensional isentropic flow

PP=2r+1+r−1r+1M2rr−1

- Air flows isentropically in a variable duct.  At section 1, V1=64 km/hr P1 = 1.03

bar absolute and T1 = 250C At section 2, P2 = 0.7 bar absolute.  Determine (i) the

velocity and temperature at section 2 and (ii) the area ratio of these two sections.

7.a)	Describe the behavior of flow in convergent divergent Nozzle when it is operated at (i) design pressure ratio (ii) pressure ratio higher than the design value and (iii) pressure ratio lower than the design value.

b)	Explain how strong compression and expansion waves are formed in a compressible fluid.

8.	Write short notes on any FOUR of the following;

a)	Newtonian and Non Newtonian fluids

b)	Laminar and Turbulent flows

c)	Lift and Drag coefficients

d)	Pressure gauges

e)	In compressible and compressible fluid flows.

*********