Code No. 220305

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	List two commonly used materials used for making patterns, their relative merits and specific casting processes where used.

b)	What are the differences in the composition of sands for making moulds and cores?  What are their desirable properties respectively?

2. a)	Describe the shell moulding process.  Name one typical part to be manufactured from this process.

b)	Which type of melting furnace is required for that part named in the above question?  Briefly describe its construction and operation.

3. a)	Discuss the functions of a riser and the need and methods of reducing its size.

b)	What are the requirements of a good gating system?  What is meant by gating ratio?  Discuss its importance.

4. a)	Discuss the relative cases where gas welding are welding and resistance welding are employed.

b)	What is the function of a flux coating?  Discuss the situations where inert gas shield is employed in place of flux.

5. a)	Briefly discuss the characteristic features and applications of different types of oxyacetylene gas welding flames.

b)	Discuss the relative features of arc welding on the

- a.c and d.c welding generators

- Straight and reverse polarity

- Consummable and nonconsummable electrics.

6. a)	Discuss the relative features of butt welding using

- Resistance butt welding

- Friction welding

- Induction welding.

b)	Briefly describe the methods of estimating welding times and costs in arc welding.

7. a)	Briefly describe the injection moulding and vacuum forming of plastics.

b)	Define the following terms in powder metallurgy

- Green and sintered compacts	(ii) Coining 	(iii) Infiltration.

8. 	Write short notes on any THREE of the following.

- Moulding machines 	(ii)	Foundry layout

(iii)	Welding defects	(iv)	Brazing

(v)	Laser beam welding.

###

Code No. 220305

Set No.

2

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

# Time: 3 Hours							 Max. Marks : 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	What are the types of pattern allowances and their need?

b)	Describe the method of testing the permeability of a moulding sand.

2. a)	With the help of a neat sketch describe the construction and operation of Cupola.  Can M.S. be melted in Cupola?  Give reason.

b)	Discuss the features of (i) Investment casting (ii) Die casting with brief description of these processes.

3. a)	What is the function of riser?  Explain the method of (i) calculating its size.

(ii) placement, feeding distance (iii) obtaining directional solidification.

b)	Define the following terms associated with gating system.

(i) Pressurised and unpressurised gating (ii) choke (iii) Sprue well.

4. a)	Briefly describe the resistance spot welding with specifications of voltage, current, time and pressure and effect of each of them.

b)	Discuss the features of fillet and butt welds.  When are the single and double V, U, J type of butt welds employed?

5. a)	Draw a neat sketch of an oxyacetylene gas torch and name the parts.  How is the flame started and different types of flames obtained?

b)	Describe thermit welding process with a listing of two of its applications.

6. a)	What are the different inert gases used for shielding?  Discuss their relative properties and applications.

b)	Briefly describe the explosive welding process and the nature of the weld interface.

7. a)	Explain the concept and need for having destructive and non destructive testing of weld ment.

b)	What is the difference in brazing and soldering?  Discuss the relative applications.

8.	Write short notes on any three of the following:

(a) MIG welding	(b) Methods of preparing metal powders

(c) Transfer Moulding of Plastics	(d) Vacuum forming of plastics

(e) Importance and application of Powder metallurgy products.

###

Code No. 220305

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

# Time: 3 Hours							 Max. Marks : 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	With a schematic diagram show the layout of a typical C.I. foundry.

b)	What are the desirable properties of a core sand and moulding sand?  Discuss their importance?

2. a)	What is composition of a typical cupola charge?  With reasons discuss if cupola can be used for melting aluminium or M.S.

b)	Describe a pressure die casting process.  For which type of parts is it suitable?

3. a)	Discuss the requirements of a good risering systems.

b)	Explain the difference in solidification of pure metals and alloys.  How is this phenomena important in the design of castings?

4. a)	Discuss the relative features of arc and gas welding.

b)	Distinguish the basic mechanisms involved in solid state fused state and resistance welding.

5. a)	Explain the difference and also the procedures of designing fillet welds and butt welds.

b)	With reason give one example in each of the following welding process where that is the only suitable process.

a)	Thermit welding

b)	Resistance spot welding.

- Explosive welding.

6. a)	Describe the friction welding process.  How does it compare with resistance butt welding?

b)	What is the difference in the gas welding and gas cutting torches and the corresponding flames?

7. a)	Describe the importance of plastics as an engineering material to replace metals.

b)	Define the following terms in Powder metallurgy.

- Green and sintered compacts

- Sizing and coining

- Granulation and reduction.

8.	Write short notes on any THREE of the following:

a)	Sand Testing

b)	Brazing and soldering

- Economics of welding

- MIG welding

- NDT common to welding and casting.
                                  # # #

Code No. 220305

Set No.

4

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

PRODUCTION TECHNOLOGY - I

(Common to Mechanical Engineering and Aeronautical Engineering)

# Time: 3 Hours							 Max. Marks : 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	Discuss the relative properties of different types of materials used as patterns.

b)	What is the difference in the sand, binder and desirable properly for cores and moulding?

2. a)	What are the types of melting furnaces commonly used in foundries?  Discuss their relative features.

b)	Explain the importance of a good gating system and risering system in metal casting.

3. a)	Briefly describe the centrifugal casting and die casting processes.

b)	Define the following terms:

- Sprue well, runner and ingates in gating systems

- Riser neck, feeding distance and blind riser in riser design.

4. a)	What are the TIG and MIG welding processes?  Where are they employed?

b)	What is resistance welding?  Discuss its features compared to arc welding?

5. a)	What are the types of gas flames in oxyacetylene gas welding?  How are they obtained and identified?  Give one typical application of each.

b)	Why fillet welds are designed for shear strength though subjected to tensile load?  How are these Joints designed?

6. a)	What is the nature of heat affected zone in arc welding?  Discuss its importance.

b)	What are the commonly found weld defects?  List any three with causes associated and remedial measures.

7. a)	What is the difference in

- Thermoplastics and thermosetting plastics

- Compression moulding and transfer moulding.

- Injection moulding and blow moulding.

b)	What are the relative merits of P.M. parts over conventionally manufactured parts? Give two examples of such P M parts.

8.	Write short notes on any three of the following:

a)	Machine moulding

b)	Economic analysis of welding

- Forge welding and the Joints preperation in it.

- Methods of production of Metal powders.

e)	Laser beam welding.

###