Code No:221051

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

MECHANICS OF FLUIDS AND HYDRAULIC MACHINES

(Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering)

Time: 3 hours.							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

---

1. a)	Explain (i) surface tension and (ii) capillarity .

b)	Derive on expression for capillary rise.

c)	The lower end of a vertical shaft rests in a foot step bearing.  Assuming the end of the shaft and the surface of the bearing are both flat and are separated by an oil film of 0.5 mm thickness.  Find the torque required to rotate the shaft at 1000 revolutions per minute.  The diameter of the shaft is 100 mm and the dynamic viscosity of the oil is 1.5 poise.

2. a)	Differentiate absolute pressure and gauge pressure.

b)	Show that in a micro manometer the accuracy of reading depends on the nearness of the densities of the manometric liquids.

c)	A rectangular plane surface 1.5 m wide and 2.0 m high is immersed in water vertically.  Determine the resultant pressure on one side and the centre of pressure if the top edge of the surface is 10m  below the water surface.

3. a)	Differentiate path line, streak line and stream lined.

b)	State and Prove Bernoulli’s equation.

c)	Water flows in a pipe line.  At a certain Point A, when the pipe diameter is 15 cm, the pressure is 35 kN/m2 and the velocity is 4.4m/s.  At another Point B, 15 m away from A the pipe diameter is 7.5 cm.  If the flow occurs from A to B, find the pressure at B if the pipe line is (i) horizontal and (ii) vertical.  Neglect losses.

4. a)	Derive the expression for the discharge through a triangular notch.

b)	A horizontal venture meter is provided in a pipe line of 30 cm diameter conveying water.  The throat diameter is 10 cm.  If the pressure at the inlet of the meter as 1.4 kg/cm2 and the vacuum pressure at the throat is 37 cm of mercury, find the discharge in the pipe. Assume 4% of the differential head is lost between the inlet and the throat.  What is the coefficient of the venturimeter?

5. a)	Differentiate hydraulic gradient line and total energy line.  Sketch these lines for a pipe line conveying water and crossing(i) a bridge and (ii) a valley.

b)	Two pipes one of 15 cm diameter 300m long and the other of 20 cm diameter, 600m long are connected in parallel.  The total discharge through the pipe system is 60 litres/sec.  Find the discharge through each pipe and the head loss.  Assume friction factor, 4f = 0.0212 for 15cm pipe and 4f = 0.02 for 20 cm pipe.

Contd…..2

Code No:221051			-2-			                             OR

6. a)	Derive the expression for the work done by a jet of water striking a moving curved vane.

b)	A jet of water moves at 14m/s on a vane shaped to deflect the jet through 120o when stationary.  If the vane is moving at 5 m/s, find the angle of the jet so that there is no shock at inlet.  What is the absolute velocity of the jet at the exit in magnitude and direction and the work done per Newton of water?  Assume the vane is smooth

7. a)	Differentiate Impulse turbines and reaction turbines.

b)	A single jet  Pelton wheel is supplied from a reservoir 400m  above the centre of the nozzle.  The supply pipe is 60 cm in diameter and 4.0 km long.  The friction coefficient for the pipe (4f) is 0.01.  The jet has a diameter of 10 cm.  The buckets deflect the jet through 165o and owing to the bucket friction, the relative velocity of water is reduced by 15% in passing over the buckets.  The overall efficiency of the wheel is 75%.  Determine the hydraulic efficiency of the wheel and the overall efficiency of the system.  Take Cv=0.97 and speed ratio is 0.47.

8.a)	With a neat sketch, name the various parts of a centrifugal pump and explain how the pump works.

b)	A centrifugal pump discharges 7500ℓ/min of water against a total head of 25m  when running at 660 r.p.m.  The outer diameter of the impeller is 60 cm and the inner diameter is 30 cm.  The area of  flow through the wheel is 600 sq.cm.  The vanes are set back at an angle of 450.  Water enters the wheel radially without shock. Calculate the manometric efficiency and the vane angle at the inlet.

*********