Code No:220152

OR

II.B.Tech II-Semester. Supplementary Examinations. January,2003

ELECTRICAL SCIENCE

(Common to Civil Engineering, Mechanical Engineering, Production Engineering, Mechatronics, Production Engineering)

Time: 3 hours.							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the following terms (Give neat diagrams)

(i) Self-Inductance (ii) Mutual Inductance (iii) Coefficient of coupling.

b)	Two similar coils have a magnetic coupling coefficient of 0.25.  When they are connected in series cumulatively the total inductance is 80 mH.  Calculate (i) The self inductance of each coil (ii) The total inductance when the coils are connected in series differentially and (iii) The total magnetic energy due to a current of 5A when the coils are connected in series cumulatively.

2.a)	Explain the term Hysteresis in the context of a magnetic circuit.  What are its consequences?

b)	Using Kirchhoff’s laws determine the direction and magnitude of the current in

20 Ω resistor.  Ignore internal resistance of the cells.

+	5Ω

4V

-

-	20Ω

6V	+                                  4Ω

3.a)	Explain clearly what do you understand by average and rms values of an alternating quality and derive rms value of a sinusoidal varying current from fundamentals

b)	When a voltage of 100V at 50 Hz is applied to a coil A, the current taken is 8 A and the power 120 W.  When the same voltage at the same frequency is applied to a coil B, the current is 10A and the power 500 W.  What current and power will be taken when the two coils are put in series across the above supply?

4.a)	Derive from fundamental the emf developed by D.C generator.

b)	A 250 D.C shunt motor draws 5A from the line on no-load and runs at 1000 rpm.  The armature and shunt field resistances are 0.2Ω and 250Ω respectively.  What will be the speed of the motor, when it is loaded and takes a current of 50 A?  Armature reaction weakens the field by 3%.

Contd…….2

Code No:220152				-2-			OR

5.a)	With neat circuit diagrams explain how can you determine the efficiency and regulation of a single phase transformer by performing O.C and S.C tests.

b)	A 230/460V transformer has a primary resistance of 0.2Ω and a reactance of 0.5Ω and the corresponding values for the secondary are 0.75Ω and 1.8Ω respectively.  Find the secondary terminal voltage when supplying (i) 10A at 0.8 P.F log (ii) 10A at 0.8 P.F leading.

6.a)	Draw torque slip characteristics of a 3-phase induction motor for three different rotor resistances and explain them.

b)	What do you understand synchronous impedance and synchronous reactance of an alternator.  How can you determine them.

7.a)	With the help of neat diagram, explain the principle of operation of a moving coil ammeter.  Mention its advantages and disadvantages.

b)	Explain with neat sketches the principle of operation,. construction and working of electrodynamics wattmeter.

8.	Write short notes on any THREE of the following:

a)	Effects of Hysteresis and eddy currents.

b)	Resonance in single phase circuit.

c)	Three point starter for D.C.motor.

d)	Measurement of three phase power.

*********