Code No.220304 Set No. 1

II-B.Tech. II-Semester Supplementary Examination January - 2003

KENEMATICS OF MACHINERY/

MECHANICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering, Mechatronics and Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Differentiate between a Crank and a Lever. State how their use gives different practical applications of a quadric cycle chain.

- Describe Grasshopper mechanism with a neat sketch.

2.	In a whitworth Quick returen motion, a Crank AB rotates about the fixed center A. The end B operates a slider reciprocating in a slotted link, rotating about a fixed center D, 5cm vertically above A. The crank AB which is 10cm long, rotates in a clockwise direction at a speed of 100rpm. Find the angular acceleration of a slotted link for the configuration in which AB has turned through an angle of 45o past its lowest position.

3.	A cam rotating with uniform angular velocity of 150rpm operates a reciprocating follower. The roller radins is 2.5cm and the distance between line of stroke of the follower and the cam center is 2.5cm. The least radins of the cam is 5cm and the stroke of the follower is 5cm. Ascent and descent both take place with uniform acceleration and deceleration. Ascent takes place during 75o of cam rotation and descent during 90o of cam rotation. Dwell between ascent and descent corresponds to 60o of cam rotation. Draw cam profile to full size.

4.	Two equal involute gear wheels of 20o pressure angle have 20 teeth each. Calculate the length of arc of contact, if the addendum was standard and was equal to one module. Pitch of teeth is 6mm of diameter per tooth.

What should be the addendum, if the arc of contact is to be maximum possible? What is then the length of arc of contact?

5.	A reduction gear between two shafts having the same axis consists of an epicyclic train as follows:

Wheel A is keyed on the driving shaft. Wheel B gears with A and also with a fixed annular Wheel C. Wheels B and D are fixed to the common spindle which is carried by an arm which can rotate about the axis of the wheel A and the wheel D gears with an annular wheel E which is keyed to the driven shaft. If A has 20, B has 24 and D 16 teeth and all teeth have the same pitch find the velocity ratio of the two shafts.

(contd…2)

Code No.220304			-2-			Set No.1

6.	Derive an expression for the minimum number of teeth on the pinion  and gear having involute teeth in terms of pressure angle, gear ratio and standard addendum.

7.a)	Sketch Davis steering gear and show that it satisfies the conditions for correct steering.

b)	A car using Ackermann type steering gear has a wheel base of 2.7m and a track of 1.4m. The track rod is 1.15m and each track arm is 14cm long. The distance between the pivots of  front stub axles is 1.23m. If the car is turning to the right find the radins of curvature of the path followed by the inner wheel for the correct steering.

8.	Explain any TWO of the following:

- Peaucellier mechanism

- Differential gear of an automobile

- Instantaneous center of rotation and Kennedy’s three center in line theorem.

***     ***     ***

Code No.220304 Set No. 2

II-B.Tech. II-Semester Supplementary Examination January - 2003

KENEMATICS OF MACHINERY/

MECHANICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering, Mechatronics and Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Answer any FIVE questions

All questions carry equal Marks

---

1.	The link lengths of a quadric cycle chain are as follows:

l1 = 10cm, l2 = 20cm, l3 = 30cm and l4 = 40cm. Find all the inversions of this quadric cycle chain and classify them.

2.	A single cylinder engine machanism has a crank of 8cm length and connecting rod 36cm length. The engine speed is 2000rpm clockwise. Determine the acceleration of the piston. Also determine the angular acceleration of the connecting rod and the total acceleration of its midpoint.

3.	Construct the profile of a disk cam with translating roller follower using the following data:

- Base circle diameter-15cm

- Roller diameter – 1.5cm

- Lift-5cm in 180o and return in 135o and then dwell.

Both lift and return motions are simple harmonic. The cam rotates in a clockwise direction.

4.	Establish a relation for the number of teeth in contact for a pair of gear wheels with involute teeth in terms of pressure angle, the number of teeth on each of the wheels and the addendum.

The number of teeth in the wheels are 20 and 31 with a pressure angle of 20o. Module pitch is 2.5mm and standard addendum of one module. Calculate the length of path of contact.

5.	In a reverted train, an arm A carries two concentric wheels B and C and a compound wheel DE. B gears with E and C with D. B has 75 teeth, C has 30 and D has 90. Find the speed and direction of C when B is fixed and the arm A makes 100 rpm clockwise. Sketch the gear train.

6.	Sketch and describe Peancellier mechanism.

(contd…2)

Code No.220304			-2-			Set No.2

7.	In a Davis steering gear mechanism, derive an expression for the conditions for correct steering. Find the inclimation of track arms to the longitudinal axis of the car when the car is moving on a straight path. Given the distance between the pivots of the front axle is 1.0675m and the wheel base is 2.5925m.

8.	Explain the following:

- Robert straight line motion mechanism

- Methods of eliminating interference in gears

- Types of kinematic pairs.

***     ***     ***

Code No.220304 Set No. 3

II-B.Tech. II-Semester Supplementary Examination January – 2003

KENEMATICS OF MACHINERY/

MECHANICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering, Mechatronics and Aeronautical Engineering)

Time:3 hours								Max.Marks:80

# Note:- 1.   Answer any FIVE questions.

- All questions carry equal Marks.

- Assume any data missing suitably.

-  - -

1.a)	Write a note on Kinematic pairs.

- What is an inversion of a mechanism? Explain the working of any two inversions   of single slider crank chain mechanism.

2.a)	Explain the working of Peancellier mechanism and show that it generates exact straight line.

- Make a note on Carioli’s component of acceleration.

- For the mechanism shown in figure determine (a) The velocity of slider D and

(b) Angular velocity of links BD and AB.

4.a)	Show that Davis steering gear satisfies the condition of exact steering.

- A Hooke’s joint connects two shafts whose axes intersect at 150o. The driving shaft rotates uniformly at 120rpm. The driven shaft operates against a steady torque of 150N-m and carries a fly wheel whose mass is 45kg and radius of gyration 150mm. Find the maximum torque which will be exerted by the driving shaft.

(contd…2)

Code No.220304			-2-			Set No.3

5.	Construct the profile of a cam to suit the following specifications:

Camshaft diameter = 40mm

Least radius of cam = 25mm

Dia of roller = 25mm

Angle of lift = 120o, dwell at lift = 60o

Angle of fall = 150o, dwell at fall = 30o

Lift of follower = 40mm

During the lift the motion is SHM. During the fall the motion is uniform acceleration and deceleration. The line of stroke of the follower is offset by 12.5mm from the center of the cam.

6.a)	Derive an expression for arc of contact for involute gear pair in standard form.

b)	Explain the working of a differential gear unit used in automobiles.

7.a)	Write a note on gear trains.

b)	For the epicyclic gear train shown in figure determine the speed of gear C, when gear B is fixed.

8.	Write short notes on the following:

a) Pantograph

b) Ackerman steering gear

c) Automobile gear box.

***     ***     ***

Code No.220304 Set No. 4

II-B.Tech. II-Semester Supplementary Examination January - 2003

KENEMATICS OF MACHINERY/

MECHANICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering, Mechatronics and Aeronautical Engineering)

## Time:3 hours								Max.Marks:80

# Note:- 1.  Answer any FIVE questions.

2. All questions carry equal Marks.

3. Assume any data missing suitably.

---

1.a)	What is a Kinematic pair? Explain the various Kinematic pairs that you come across.

b)	Show that Hart’s mechanism generates exact straight line.

2.a)	Derive an expression for Coriolis acceleration in standard form.

b)	Explain the working of a differential unit used in automobiles.

3.a)	Show that Davis steering gear satisfies the condition of exact steering.

b)	The driving shaft of a Hooke’s joint runs at a uniform speed of 240rpm and the angle between the shafts is 20o. The driving shaft with attached masses has a mass of 55kg at a radius of gyration of 150mm. If a steady torque of 200N-m resists rotation of the driven shaft. Find the torque required at the driving shaft, when  = 45o.

4.	For the mechanism shown in figure.1, find out (a) The velocity of slider, E and

(b) The angular velocity of link AB.

(contd…2)

### Code No.220304	-2-			Set No.4

5.a)	What are the different types of motion with which a follower can move?

- Derive expressions for acceleration for a tangent cam operating on a radial-translating roller follower, when the contact is on straight flank.

6.a)	List out the merits and demerits of cycloidal and involute profiles for gear teeth.

b)	Derive an expression for arc of contact for two involute teeth gear wheels in action, in standard form.

7.a)	Make a note on gear trains.

b)	For the epicyclic gear train shown in figure.2 find the speed of shaft C stating the direction of rotation to that of B.

8.	Write short notes on the following:

  - Chebicheff mechanism

  - Kleins construction

  - Pantograph.

***     ***     ***