Code No.221001

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Draw the circuit of a series type ohmmeter and explain the working of it with necessary equations.

b)	The coil of a moving coil galvanometer has 300 turns and is suspended in a uniform magnetic field of 0.1 wb/m2.  The control constant is 0.2 x 10-6 N-m/radian.  The coil is 2 cm wide and 2.5 cm high, with a moment of intertia of  0.15 kg-m2.  If the galvanometer resistance is 200 ohms, calculate the value of resistance which when connected in the galvanometer terminals, will give critical damping.  Assume the damping to be entirely electromagnetic.

2.a)	Describe the construction of suspension type galvanometer.  Derive its torque equation.

b)	Draw the block diagram of a Q-meter and explain its operation.

3.a)	Give the construction details of power factor meter and explain the principle and operation of it.

b)	What is the importance of FET in the input side of a voltmeter.

4.a)	Explain the importance of Delay line and describe a method of delay line.

b)	What are special purpose CROs.  Explain the working of a special purpose CRO.

5.a)	What are Lissajous figures?  Describe how frequency and phase are determined by using Lissajous figures.

b)	What is wave Analyzer?  Describe the working of any one type of wave analyzer.

6.a)	Explain the principle of operation of  a Sine wave generator.  Derive the necessary equation.

b)	What is the dynamic range of a spectrum analyzer with a third-order intercept point of +25 dBm and a noise level of -85 dBm.

7.a)	Using a suitable circuit, explain the measurement of frequency ratio.

b)	Explain the various errors in the measurement of time and frequency using an electronic counter.

8.a)	What are the effects on accuracy and resolution, when a fixed modulus prescalar is added to a frequency counter.

b)	Explain the principle of operation of a spectrum analyzer for higher frequencies.

- - -

Code No.221001

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Describe the construction and principle of operation of dynamometer type instruments.

b)	Describe with a circuit, the working of Rectifier type a.c voltmeter.

c)	An a.c. voltmeter of 0-300v range employing full wave rectification and a 20 mA PMMC movement.  What would be the ohms/volt sensitivity of this voltmeter.

2.a)	Explain the measurement of power at Radio frequencies with suitable set up.

b)	Explain the operation and working of a R.F millivoltmeter.

3.a)	Explain the principle of operation of Thermo-electric Instruments.

b)	Explain the operation of a vom meter.

4.a)	Derive an expression for deflection sensitivity of a CRT.

b)	The X deflection plates in a CRT are 20 mm long and 5 mm apart.  The centre of the plates is 0.25 m from the screen.  The accelerating voltage is 300v.  Determine the deflection sensitivity and deflection factor of the CRT.

5.a)	Explain the operation of a vector impedance meter with the help of a block diagram.

b)	What are the materials used for screen in CROs.  What are their characteristic features.

6.a)	Explain the working of a harmonic mixing spectrum analyzer with relevant circuit.

b)	Discuss about the applications of a spectrum Analyser.

7.a)	Explain the extension of the frequency range of frequency counter by automatic heterodyning unit.

b)	Explain the period measurement using a counter.

8.a)	Explain the working of an intermodulation distortion meter.  Give the block diagram and the necessary waveforms.

b)	Why period measurement is preferred of low frequencies.

- - -

Code No.221001

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Give the constructional details of permanent moving coil instruments and explain the principle of operation of it.  Discuss the advantages and disadvantages of it.

b)	A moving coil instrument has a resistance of 5Ω between terminals and full scale deflection is obtained with a current of 15 mA.  The instrument is to be used with a manganis shunt to measure 100 A at full scale.  Calculate the error caused by a 10oC rise in temperature.

Temperature coefficient of copper = 0.004/oC.

Temperature coefficient of manganis = 0.00015/oC.

2.a)	Explain the principle of operation of True RMS responding voltmeter.  What is the difference between ordinary voltmeter and true RMS voltmeter.

b)	Explain the construction and operation of a single phase Electrodynamometer power factor meter.

3.a)	Draw the Basic block diagram of CRO and explain function of each block.

b)	What is a sampling oscilloscope.  Explain its principle with relevant timing diagram.

4.a)	Draw the block diagram of a 	Q-meter and explain its operation.

b)	Why it is necessary to use delay line in the vertical section of CRO.

5.a)	What is a wave analyzer?  Explain the working of Heterodyne wave Analyzer with block diagram.

b)	Explain the operation of a function generator with a neat block diagram.

6.a)	Explain the operation of phase locked loop frequency synthesizer.

b)	How do you measure frequency, phase and time period of a waveform using CRO.

7.a)	Explain the measurement of frequency with the help of a basic frequency counter.

b)	Discuss about the measurement errors of an electronic counter.

8.a)	Give the applications of wave analysers.

b)	Discuss about the various distortions of a sinusoidal waveform.

c)	Discuss about a standard time base generator.

- - -

Code No.221001

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL AND ELECTRONICS MEASUREMENTS

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	With a neat sketch explain the working of an induction type two element energy meter for 3 phase 3-wire system.

b)	What are the errors occurring in a single phase energy meter.  How are they compensated.

2.a)	Explain how high impedance components can be measured using Q-meter.

b)	Explain how the distributed capacitance can be measured using Q-meter.  Calculate the distributed capacitance for C1=460 pf and C2=100pf at 2 MHz and 4 MHz respectively in the first and second measurement.

3.a)	What are instrument transformers.  Give the circuits of current and potential transformers.  What are the advantages of instrument transformers.

b)	Explain the operation of differential amplifier type of electronic voltmeter using FETs.

4.a)	Explain about Dual beam and Dual trace in CRT.

b)	A Lissajous pattern on an oscillioscope is stationary and has 5 horizontal tangencies and 2 vertical tangencies.  The frequency of horizontal input is 1000 Hz.  Determine the frequency of vertical input.

5.	Give brief notes on the following:

(i) Wave Analyzer  (ii) Spectrum Analyzer  (iii) Lissajous figures.

6.a)	Explain the operation of sine wave signal generator with frequency counter and automatic level controller.

b)	Explain about signal generator modulation.

7.a)	Explain the working of fundamental suppression Harmonic distortion Analyzer.

b)	Explain the measurement of time interval using a universal counter.

8.a)	Give the block diagram of a universal counter and explain its operation.

b)	Give the importance of time base in a digital frequency counter.

- - -