Code No.220356

OR

II-B.Tech. II-Semester Supplementary Examination January - 2003

KENEMATICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering and  Mechatronics)

Time:3 hours								Max.Marks:70

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Differentiate between a Crank and a Lever. State how their use gives different practical applications of a quadric cycle chain.

- Describe Grasshopper mechanism with a neat sketch.

2.	In a whitworth Quick returen motion, a Crank AB rotates about the fixed center A. The end B operates a slider reciprocating in a slotted link, rotating about a fixed center D, 5cm vertically above A. The crank AB which is 10cm long, rotates in a clockwise direction at a speed of 100rpm. Find the angular acceleration of a slotted link for the configuration in which AB has turned through an angle of 45o past its lowest position.

3.	A cam rotating with uniform angular velocity of 150rpm operates a reciprocating follower. The roller radins is 2.5cm and the distance between line of stroke of the follower and the cam center is 2.5cm. The least radins of the cam is 5cm and the stroke of the follower is 5cm. Ascent and descent both take place with uniform acceleration and deceleration. Ascent takes place during 75o of cam rotation and descent during 90o of cam rotation. Dwell between ascent and descent corresponds to 60o of cam rotation. Draw cam profile to full size.

4.	Two equal involute gear wheels of 20o pressure angle have 20 teeth each. Calculate the length of arc of contact, if the addendum was standard and was equal to one module. Pitch of teeth is 6mm of diameter per tooth.

What should be the addendum, if the arc of contact is to be maximum possible? What is then the length of arc of contact?

5.	A reduction gear between two shafts having the same axis consists of an epicyclic train as follows:

Wheel A is keyed on the driving shaft. Wheel B gears with A and also with a fixed annular Wheel C. Wheels B and D are fixed to the common spindle which is carried by an arm which can rotate about the axis of the wheel A and the wheel D gears with an annular wheel E which is keyed to the driven shaft. If A has 20, B has 24 and D 16 teeth and all teeth have the same pitch find the velocity ratio of the two shafts.

(contd…2)

Code No.220356			-2-			 	OR

6.	Derive an expression for the minimum number of teeth on the pinion  and gear having involute teeth in terms of pressure angle, gear ratio and standard addendum.

7.a)	Sketch Davis steering gear and show that it satisfies the conditions for correct steering.

b)	A car using Ackermann type steering gear has a wheel base of 2.7m and a track of 1.4m. The track rod is 1.15m and each track arm is 14cm long. The distance between the pivots of  front stub axles is 1.23m. If the car is turning to the right find the radins of curvature of the path followed by the inner wheel for the correct steering.

8.	Explain any TWO of the following:

- Peaucellier mechanism

- Differential gear of an automobile

- Instantaneous center of rotation and Kennedy’s three center in line theorem.

***     ***     ***