Code No.220803

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

FLUID MECHANICS

(Chemical Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	The temperature of the earth’s atmosphere drops about 5oC for every 1000 m of elevation above the earth’s surface.  If the air temperature and pressure at ground level are 27oC and 750 mm Hg respectively, what is the pressure at an elevation of 9000m?  Assume that air behaves as an ideal gas.

2.	Prove that the flow of a liquid in laminar flow between infinite parallel flat plates is given by;

Pa-Pb= 12μVLa2gc

Where L = length of plate in direction of flow

a= distance between plates.

All other symbols have their usual significance.  Neglect end effects.

3.	A 20 cm diameter pipe divides into a 15 cm branch and a 10 cm branch.  If the total flow is 0.5 m3/sec and if the same average velocity occurs in each branch, what is the discharge rate in each branch?

4.	A particle of specific gravity 2.6 is falling by gravity in water (viscosity=1 CP) at a Reynolds number of 200.  What is the size of the particle in microns and its terminal settling velocity? (density of water = 990 kg/m3).

5.	A venturimeter having inlet and throat diameters of 15 and 7.5 cms respectively is fitted in a vertical pipe line carrying water upwards.  A manometer with Mercury as the manometric fluid is fitted between the throat and the inlet.  Calculate the flow rate through the meter, if the manometer reading is 6 cms.  Assume CV=0.95.

6.a)	What is superficial velocity and how is it related to the average velocity in the packed bed?

b)	Draw a neat sketch of a multiple-discharge pneumatic conveying system.

c)	What are the reasons for the decrease of the actual head developed by a centrifugal pump from that calculated from ideal pump correlations?

d)	Why does a centrifugal pump need priming before starting?  Explain.

e)	Explain, why the drap force is independent of flow rate for a rotameter.

Contd………2

Code No.220803					-2-			         Set No.1

7.	A long pipe line carries water from a point ‘A’ where the pressure is 5.15 x 106 Pa gauge to a point ‘B’, 3.6m above ‘A’, where the pressure is 4.7 x 106 Pa gauge.  If the velocity of the fluid is 1 m/sec, at point ‘A’ where the pipe is of 50 mm diameter and the pipe is of 25 mm diameter at point ‘B’, find the power expended in over coming the losses.

8.	Write short notes on:

a) non – Newtonian fluids

b) Positive displacement pumps

c) Flow in boundary layers.

- - -

Code No.220803

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

FLUID MECHANICS

(Chemical Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the term “Rheological behaviour” of a fluid.  Describe the Rheological behaviour of Pseudo plastics and dialatants and indicate the differences.

b)	In the figure shown below fluid A is water and fluid B is mercury(Sp.gr=13.6).  If the pressure difference between M and N is 3.5 x 104 N/m2 and a=1m and b=30cm.  What is the manometer reading.

2.a)	A pipe of diameter 20 cm. and length 10m. is used to transport oil of specific gravity 0.9 and viscosity 1.5 poise.  The oil is pumped at a rate of 20 lit./s.  Find the average velocity; the maximum velocity:-shear rate at the wall and pressure drop due to friction.

b)	Write notes on “Reynolds Stress”.

3.	Water at 20oC is pumped at a constant rate of 9 m3/h from a large reservoir resting on the floor to the open top of an experimental absorption tower.  The point of discharge is 5m above the floor, and frictional losses in the 50 mm pipe from the reservoir to the tower amount to 2.5 j/kg.  At what height in the reservoir must the water level be kept if the pump can develop only 0.1 kw.

4.a)	 Write notes laminar flow in pipes of Newtonian fluids.

b)	Explain the concept of friction losses from sudden expansion and sudden contraction of cross sections.

Contd……2

Code No. 220803					-2-			     Set No.2

5.	Natural gas consisting of methane is to be transported through a 20-in-ID pipeline over flat terrain.  Each pumping station increases the pressure to 100 lbs/in2 abs. and the pressure drops to 25 lbf/in2 abs. at the inlet to the next pumping station 50 mi. away.  What is the gas flow rate in cubic feet per hour measured at 60oF and 30 in Hg pressure?

6.a)	Explain the terms wall drag, form drag and drag coefficients.

b)	Write about particulate and agrigative fluidization.

c)	Write about pneumatic conveying.

7.a)	Explain in detail the theory of Centrifugal pump theory.

b)	Write notes on positive displacement compressors.

8.	A horizontal venture meter having a throat dia of 20 mm is set in a 75 mm.  ID pipe line.  Water at 15oC is flowing through the line.  A monometer containing mercury under water measures the pressure differential over the instrument.  When the manometer reading is 500 mm, what is the flow rate in gallons per minute?  If 12 percent of the differential is permanently lost, what is the power consumption of the meter?

- - -

Code No.220803

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

FLUID MECHANICS

(Chemical Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	A centrifuge bowl 25.4 cm i.d is turning at 4000 rpm.  It contains a layer of chlorobengene 5cm. thick.  If the density of chlorobenzene is 1000 kg/m3 and the pressure at liquid surface is atmospheric, what gauge pressure is exerted on the wall of centrifuge.

b)	Show that the pressure at a point in a static fluid is same in all directions.

2.a)	Explain the term “fluctuating components of velocity” and ‘steady flow’ with respect to turbulent flow.

b)	Distinguish between Newtonian and Non-Newtonian fluids.  Explain the different curser of Non-Newtonian fluids.

3.a)	Discuss in detail about the flow in Boundary layers.

b)	Write about boundary layer separation and wake formation.

4.a)	A water tank is 30 bt in diameter and the normal depth is 25 bt.  The outlet is a 4 in horizontal pipe at the bottom.  If this pipe is sheared off close to the tank, what is the initial flow rate of water from the tank?(Neglect friction loss in the short stub of pipe).

b)	How long will it take for the tank to be empty?

c)	Calculate the average flow rate and compare it with the initial flow rate.

5.a)	Discuss in detail about flow of liquids in this layers.

b)	A Newtonian fluid is in laminar flow in a rectangular channel with a large aspect ratio.  Derive the relationship between local and maximum velocity and determine the ratio Umax/V.

6.a)	Write in detail about Adiabatic frictional flow and Isothermal frictional flow through straight condricts.

b)	Explain the process of compressible flow.

7.a)	Explain the process of fluidization and develop an equation for pressure drop across fluidized bed.

b)	Write about drag coefficient and explain the criterion for setting regime.

8.	Write short notes on:

(a) NPSH and Cavitation   (b) Positive displacement pumps  (c) Blowers

(d) Areameter.

- - -

Code No.220803

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

FLUID MECHANICS

(Chemical Engineering.)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Derive an expression for the pressure distribution across a liquid in a centrifuge bowl.

b)	A U-tube manometer is used to measure the pressure drop across an orifice.  The manometer fluid is mercury (density 13,590 kg/m3) and the fluid flowing through the orifice and filling the manometer leads is brine (density 1290 kg/m3).  The gauge pressure at the upstream tap is 0.16 bar and the pressure at the downstream tap is 200 mm Hg below the atmospheric pressure.  What is the manometric reading in mm?  What is the increase in manometer if the pressure is increased by 10% and the difference in the densities of the fluids is decreased by 10%?

2.a)	Write the Bernoulli equation for fluid friction, explaining the terms.  Write also the Bernoulli equation without fluid friction, assuming the kinetic energy correction factors αa=1 and αb=1.

b)	A liquid is drained from the bottom of a large open tank through a pipe located at a point 7 m below the surface of the liquid in the tank.  Considering a stream line starting at the liquid surface and passing through the centre of the drain line to the point of discharge and assuming negligible friction losses, calculate the velocity of flow along the stream line at the point of discharge from the pipe.  Take g=9.80665 m/s2.  If  the friction losses amount to 7 m2/s2, what is the velocity of flow?

3.a)	In the case of a Newtonian fluid flowing through a pipe, derive an expression for the ratio of the local velocity to the maximum velocity in terms of the radii.

b)	Calculate the pressure drop for the laminar flow of a fluid, flowing at an average velocity of 10ms in a pipe of length of 100 m and diameter 8 cm.  The density of fluid is 930 kg/m3 and its viscosity is 4 CP.

4.a)	Derive an equation for the pressure ratio for an isentropic flow of a fluid, in terms of Mach Number and r.

b)	Air enters a convergent-divergent nozzle at 560 K and a pressure of 25 atm.  Assuming the Mach number in the throat as 0.8, calculate: the pressure, linear velocity, density and mass velocity at the throat.  For air, r=1.4, Mol.wt=29.  Take R=82.056 x 10-3atm−m3Kgmol.K and 1 atm=1.01325 x 105Kg−ms2

Contd…….2

Code No.220803					-2-			       Set No.4

5.a)	Discuss how the friction is determined in channels of non circular cross section by means of the mean hydraulic radius.  How is the friction factor estimated for laminar flow through an annulus?

b)	Derive the relationship between the expansion-loss coefficient Ke and the ratio of the cross section of the small pipe (Sa) to be cross section of the larger pipe (Sb).

6.a)	What is terminal velocity?  Derive an equation for terminal velocity of spherical particles settling in a fluid by gravity, in the Stoke’s Law region.

b)	Estimate the terminal velocity of solid particles of mean diameter of 0.1 mm, falling in a liquid of  density of 1000 Kgm3 .The viscosity of the liquid is 1 cp and the density of  the solids is 2000 Kgm3. Use the criterion, K=DPgρρp−ρμ213<2.6, for the Stoke’s Law to apply.

7.a)	Derive an expression for the power required by an adiabatic compressor.

b)	A three-stage reciprocating compressor is to compress 300 m3/h of methane from 1 to 64 atm.  The inlet temperature is 27oC.  Taking the CP of the gas as 39JgmoloC  and r=1.31, calculate the brake power in KW, given the mechanical efficiency as 80%.  Also, estimate the discharge temperature from the first stage.

8.	Write short notes on the following:

a) Newtonian and non-Newtonian fluid behaviour

b) Advantages and disadvantages of fluidization

c) Vacuum pumps.

- - -