Code No.220154

OR

II-B.Tech. II-Semester Supplementary Examination January - 2003

MECHANICS OF FLUIDS

(Civil Engineering)

Time:3 hours								Max.Marks:70

# Answer any FIVE questions

All questions carry equal Marks

---

1.a)	Define the following:

i)   Specific weight	(ii) Specific mass	(iii) Specific gravity	and

iv) Specific volume.

- Enumerate the classification of fluids.

- Derive an expression for the pressure inside a soap bubble of diameter ‘d’.

2.a)	State and prove Pascals law.

b)	A cylindrical gate 3m in diameter rests on the crest of a dam. Find the vertical and horizontal components of water pressure acting per meter length of gate. Assume water level up to the top of gate.

3.a)	Distinguish between the following:

- Stream line and path line.

- Steady flow and unsteady flow.

- Uniform and Non-uniform flow

- Laminar and turbulent flow.

- Rotational and Irrotational flow.

b)	Examine whether the following flow is possible, and if so, rotational or irrotational.

u = x+y+z; v = 2x+2y+2z  and  w = 3x-4y-5z.

4.a)	With a neat sketch explain a flownet, explain its uses and limitations.

- Define potential function and show that 2/x2+2/y2 = 0 for continuity of flow.

5.a)	Show that the discharge flowing through a venturimeter is independent of orientation of the venturimeter.

b)	A pipeline carrying oil of specific gravity 0.87. Changes in diameter from 200mm at position ‘A’ to 500m.m at position ‘B’ which is 4 m at a higher level than ‘A’. If the pressures at ‘A’ and ‘B’ are 10N/cm2 and 5N/cm2 respectively and the discharge is 200 lit/sec. Determine the loss of head and direction of flow.

(contd…2)

## Code No.220154				-2-			OR

6.a)	Derive an expression for the velocity distribution for steady, laminar flow through

parallel plates with a gap of ‘t’.

b)	Two parallel plates kept 0.1m apart have laminar flow of oil between them with a max velocity of 1.5m/s. Calculate the discharge per metre width, the shear stress at the plates, the difference in pressure in Pascals between two points 20m apart. Take viscosity of oil to be 2.453 N-S/m2.

7.a)	Explain the characteristics of laminar and turbulent boundary layer.

- Differentiate between skin friction drag and pressure drag.

- Water at 30oC and atmosphere pressure flows through a small pipe 5cm internal diameter. The flow is fully developed and is at rate of 2 litres per second. Calculate  (i) Friction factor	(ii) Pressure drop over a length of 5m and

iii) Thickness of laminar sub-layer at 30oC . Kinematic viscosity of water at 30oC is 0.796 x 10-6 m2/s.

8.	Write short notes on:

- Free vortex motion

- Manometers

- Pipes connected in parallel

- Notches and weys.

***     ***     ***