Code No.220854

OR

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

FLUID MECHANICS

(Chemical Engineering.)

Time: 3 hours							   Max. Marks:70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	A centrifuge bowl 25.4 cm i.d is turning at 4000 rpm.  It contains a layer of chlorobengene 5cm. thick.  If the density of chlorobenzene is 1000 kg/m3 and the pressure at liquid surface is atmospheric, what gauge pressure is exerted on the wall of centrifuge.

b)	Show that the pressure at a point in a static fluid is same in all directions.

2.a)	Explain the term “fluctuating components of velocity” and ‘steady flow’ with respect to turbulent flow.

b)	Distinguish between Newtonian and Non-Newtonian fluids.  Explain the different curser of Non-Newtonian fluids.

3.a)	Discuss in detail about the flow in Boundary layers.

b)	Write about boundary layer separation and wake formation.

4.a)	A water tank is 30 bt in diameter and the normal depth is 25 bt.  The outlet is a 4 in horizontal pipe at the bottom.  If this pipe is sheared off close to the tank, what is the initial flow rate of water from the tank?(Neglect friction loss in the short stub of pipe).

b)	How long will it take for the tank to be empty?

c)	Calculate the average flow rate and compare it with the initial flow rate.

5.a)	Discuss in detail about flow of liquids in this layers.

b)	A Newtonian fluid is in laminar flow in a rectangular channel with a large aspect ratio.  Derive the relationship between local and maximum velocity and determine the ratio Umax/V.

6.a)	Write in detail about Adiabatic frictional flow and Isothermal frictional flow through straight condricts.

b)	Explain the process of compressible flow.

7.a)	Explain the process of fluidization and develop an equation for pressure drop across fluidized bed.

b)	Write about drag coefficient and explain the criterion for setting regime.

8.	Write short notes on:

(a) NPSH and Cavitation   (b) Positive displacement pumps  (c) Blowers

(d) Areameter.

- - -