Code No. 220303

Set No.

1

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	With a neat sketch, explain the working of a 4-stroke CI engine:

- List the merits and demerits of 4 stroke engines compared with 2 stroke engines?

2. a)	Discuss the various factors that are to be considered in the design of SI engine combustion chambers.

b)	With neat sketches, explain the various types of combustion chambers used in SI engines?

3. 	An eight-cylinder automobile petrol engine of 100 mm bore and 90 mm stroke has a compression ratio of 7.  The engine develops 136 kW at 4000 rpm.  The engine operates at 20% rich in fuel and the atmospheric conditions are 27oc and 760 mm of Hg barometer.  The bsfc is 0.34 kg/k Wh. Estimate the power output and bsfc when (i) barometer is 740 mm of Hg and temperature is 47oC.  (ii) barometer is 775 mm of Hg and temperature is 7o C.  fmep at 4000 rpm is 2.10 bar.

4. a)	Define volumetric efficiency and Isothermal efficiency of a reciprocating air compressor.

b)	The L.P cylinder of a two-stage double-acting air compressor running at 120 rpm has 50 cm diameter and 75 cm stroke.  It draws in air at a pressure of 1 bar and 20oC and compresses it adiabatically at 3 bar.  The air is then delivered to an intercooler where it is cooled at constant pressure to 35oC, and is then further compressed to 10 bar in H.P cylinder.  Determine the required power of an electric motor to drive the compressor.  Assume the mechanical efficiency of the compressor 90 percent and of the motor 86 percent.

5. a)	Discuss the important properties of refrigerants.

- Explain briefly ammonia-absorption refrigeration cycle.

6. a)	Discuss the stages of combustion in SI engines.

b)	A single cylinder oil engine has a compression ratio of 10 to 1.  The specific fuel consumption is 0.6 kg/kwh.  The calorific value of the fuel oil is 44,000 KJ/kg.

Calculate	a)	the Thermal efficiency and

- the relative efficiency, assuming

the engine operates on the constant volume cycle.  Take r = 1.4 for air.

7. a)	How compressors are classified?

- Describe the working of a roots blower with the help of a p-v diagram.

Contd…….2

Code No.220303				-2-				        Set No.1

8.	A simple R-12 plant is to develop 4 tonnes of refrigeration.  The condenser and evaporator temperatures are to be 35o c and – 15o c respectively.  Determine

- The refrigerant flow rate in kg/s,

- The volume flow rate handled by the compressor in m3/s,

- The compressor discharge temperature,

- The pressure ratio,

- The heat rejected to the condensor in kW,

- The flash gas percentage after throttling,

- The COP and

- The power required to drive the compressor.

How does this COP compare with that of a cornot refrigerator operating between 35oC and –15oC?

###

Code No. 220303

Set No.

2

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	How IC engines are classified?

b)	Consider an air-standard Diesel cycle having a compression ratio of 17.  At the beginning of compression, the temperature at the end of constant pressure process is 1500o C. Find (a) The heat supplied per kg of air (b) the  cycle efficiency and (c) the m.e.p.

2	Explain briefly the normal and abnormal combustion in SI engines.  Also discuss the effects of detonation.

3.	A full load test was conducted on a 2-stroke engine and the following results were obtained: Speed: 500 rpm, brake load: 50 N, imep: 3 bar, oil consumption: 5 kg/hr, jacket water temperature rise: 35oC, jacket water flow rate: 7 kg/min, air fuel ratio by mass: 30, exhaust gas temperature: 350oC,  Room temperature 25oC, Atmospheric pressure: 1 bar.

Cylinder diameter: 22 cm, stroke : 28 cm, brake diameter: 1.6 m, fuel C.V: 42,000KJ/kg, proportion of hydrogen by mass in the fuel : 15, specific heat of dry exhaust gas : 10 KJ/kg – K, specific heat of dry steam:  20 KJ/kg-K.  Determine

- The indicated thermal efficiency.

- The specific fuel consumption.

- The volumetric efficiency based on atmospheric conditions.

Draw up a heat balance sheet for the test.

4. a)	Distinguish between rotary and rotodynamic compressors.

- With a neat sketch, explain the working of a centrifugal compressor.

5. a)	What is air-conditioning?  How does it differ from refrigeration?

b)	Consider a vapor compression refrigeration system using R-12 as a refrigerant.  The maximum and minimum pressure of the cycle are 8 bar and 1.2 bar respectively.  At the compressor inlet, the vapor temperature is –12oc  and the temperature of liquid at the condenser outlet is 30oC.  The required refrigerating load is 2.2 kW.  The Compressor runs at 600 rpm and has a volumetric efficiency of 75%.  Find the COP and swept volume of the compressor.

6. a)	What are the air-standard assumptions.

- Discuss briefly the pre-combustion chamber and air-cell chamber of CI engines.

Contd…2

#### Code No: 220303				.2.				Set No. 2

7. a)	Explain the following.

- valve timing diagram of a 4. stroke CI engine.

- Port timing diagram of a 2 stroke SI engine.

b)	Draw a detailed Sankey diagram for SI engine and explain.

8. a)	Discuss the phenomenon of surging and choking in rotary compressors.

- Define the following:

(i) Relative humidity		(ii) Wet bulb temperature

(iii) Psychrometer		(iv) Degree of saturation.

###

Code No. 220303

Set No.

3

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1. a)	What are the air-standard assumptions?

b)	Draw the p.v and T.s diagrams of ideal Diesel cycle and obtain the expression for thermal efficiency.

2.	Explain briefly the different stages of combustion in a CI engine.

3. a)	Explain the principle of prony brake and rope brake in measuring engine power.

b)	Define mean effective pressure.  A single cylinder 4-stroke CI engine has a bore and stroke of 75 mm and 100 mm respectively.  Find the bmep if the torque is 25 N-m.

4. a)	Derive an expression for the work done in a single stage reciprocating air compressor when,

- Clearance is neglected.

- Clearance is considered.

b)	A double acting air compressor of 50 k W in which air is drawn at 1 bar, 25oC and compressed according to the law pv1.2 = constant to 6 bar.  The compressor runs at 150 rpm with an average piston speed of 175 m/min.  Neglecting clearance, find the size of the cylinder.

5. a)	Explain a cooling and dehumidification process.

b)	Air enters a window air-conditioner at 1 atm, 30o C and 80% relative humidity at a rate of 10 m3/min, and it leaves as saturated air at 14oC.  Part of the moisture in the air which condenses during the process is also removed at 14oC.  Determine the rates of heat and moisture removal from the air.

6. 	Consider an ideal Diesel cycle.  At the beginning of compression process, the cylinder volume is 1500 cm3 and the end of heat addition process, it is 150 cm3. The compression ratio is 15.  Air is at 20oC and 101 k Pa at the beginning of the compression process.

Calculate:

- The pressure at the beginning of the heat rejection process.

- The network per cycle, in kJ and

- The mean effective pressure.

7. a)	Explain briefly the various factors that influence the flame speed.

b)	Discuss briefly the gravimetric fuel flow measurement and air flow measurement in IC engines.

8. a)	Define ton of refrigeration.

- What are the desirable properties of  an ideal refrigerant.

- Explain briefly the gas cycle refrigeration.  Why is it preferred air crafts.

###

Code No. 220303

Set No.

4

## II B.Tech II- Semester-Supplementary- Examinations January – 2003

THERMAL ENGINEERING-I

(Mechanical Engineering)

# Time: 3 Hours							 Max. Marks: 80

## Answer any Five questions

All questions carry equal marks

---

1.	With neat sketches, explain a wet sump lubrication system.  How does it differ from dry sump lubrication system.

2. a)	How SI and CI engine fuels are rated?  Explain.

- Discuss briefly the important qualities of SI engine fuels.

3. a)	Define the following parameters for an IC engine.

i.	Volumetric efficiency,	ii.	Mean effective pressure.

iii.	Brake Thermal efficiency	iv.	Mean piston speed.

b)	A 4-stroke gasoline engine develops a brake power of 410 kw.  The engine consumes 120 kg of fuel in one hour and the air consumption is  40 kg/min.  If the mechanical efficiency is 87% and the fuel heating valve is 43,000 KJ/kg, determine the following:

- air-fuel ratio.

- Indicated and brake thermal efficiency.

4. a)	Draw an ideal velocity diagram at exit of a centrifugal compressor having

- Backward curved vanes.

- Forward curved vanes.

b)	A Centrifugal Compressor compresses air from 1 bar, 15oC to 2.5 bar, 95oC.  The mass of air delivered is 2.2 kg/s and no heat is added to the air from external sources during compression.  Find the efficiency of the compressor relative to ideal adiabatic compression and estimate the power absorbed.  Also find the change in entropy of air during the compression.

5. a)	Explain the effect of super heating and sub cooling on the vapour compression cycle.

b)	Air enters the compressor of an aircraft cooling system, at 101 kPa, 10oC.  The air is now compressed to 2.5 bar with an isentropic efficiency of 72%.  After being cooled to 47oC at constant pressure.  In a heat exchanger, the air then expands in a turbine to 1 bar with an isentropic efficiency of 75%.  The cooling load of the system is 3 tonnes of refrigeration.  After absorbing heat at constant pressure, the air re-enters the compressor which is driven by the turbine.  Find the COP of the refrigerator, the driving power required and the air mass flow rate.

Contd…2

###### Code No. 220303				.2.				    Set No. 4

6. a)	Explain the principle of thermosyphon system.

- What are the functions of a lubricant?  Discuss briefly the various desired properties of a lubricant and explain.

7. a)	What  are the important qualities of CI engine fuels.

- A 2. stroke Diesel engine develops a brake power of 420 KW.  The engine consumes 195 kg/h of fuel and the air fuel ratio is 22:1.  The calorific value of the fuel is 42,000 KJ/kg.  If 76 KW of power is required to overcome the frictional losses, calculate

- Mechanical efficiency

- Air consumption and

- Brake thermal efficiency.

8. a)	How compressors are classified?  Explain.

- With a neat sketch, explain a vane type blower.

- Explain the process of heating and humidification in air-conditioning.

###