Code No.220402

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL TECHNOLOGY

( common to Computer Science and Engineering, Computer Science and Instrumentation Technology, Electronics and Communication Engineering, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	“Though the output voltage of a d.c. generator is a d.c. voltage, yet the e.m.f. induced in the armature conductors is an a.c. voltage” – Explain.

b)	A 6-pole, wave wound d.c. generator, running at a speed of 300 r.p.m. generates an induced e.m.f. of 535 volts.  Calculate the flux/pole, if it has 650 conductors.  Derive the formula used.

2.a)	Explain what you mean by the “Back e.m.f.” of a d.c. motor and why it should be less than the applied voltage.

b)	The armature of a 6-pole, lap-wound d.c. shunt motor takes a current of 350A at a speed of 400r.p.m.  The flux/pole is 75 mWb and the number of armature conductors is 1200.  Calculate the brake horsepower if 3% of the torque is lost in friction, windage and iron losses.  Derive the formula used.

3.a)	“The core of a transformer is made of silicon steel laminations” – Discuss.

b)	A 25 KVA transformer has 500 turns on the primary and 50 turns on the secondary winding.  The primary is connected to 3,000 V, 50 – Hz supply.  Find the maximum flux in the core.  Derive the formula used.

4.a)	Draw and explain the phasor diagram of a single phase transformer supplying a lagging p.f. load.

b)	The O.C. and S.C. tests on a 4 KVA, 200/400-V, single phase transformer gave the following results:

O.C. test :	200V, 	0.8A;	80W on the l.v. side.

S.C. test: 	20V; 	10A;	100W on the h.v. side.

Calculate the efficiency of the transformer when delivering 34full-load at 0.8 p.f. lagging.

5.a)	Explain the production of a rotating magnetic field in a 3-φ induction motor.

b)	From fundamentals, establish that the torque developed by a 3-φ Induction motor varies linearly with slip at low values of slip and that it follows a rectangular hyperbolic variation at high values of slip.

Contd……….2

Code No.220402				      -2-				      Set No.1

6.a)	Derive an expression for the distribution factor of an a.c. armature winding, from fundamentals.

b)	A 3-phase, star-connected alternator is rated at 1,600 KVA, 13,500V.  The synchronous impedance is (1.5+j30) ohms/phase.  Calculate the percentage voltage regulation for a load of 1,200 KW at 0.75 leading p.f.  Derive the formula used.

7.a)	“Single phase induction motors are not self-starting”-  Discuss.

b)	Explain the principle of operation of an a.c. servo motor and mention some of its applications.

8.a)	Discuss the star-delta method of starting a 3-phase induction motor, with the help of a sketch.

b)	“3-phase synchronous motor can operate only at synchronous speed but at no other speed” – Discuss.

- - -

Code No.220402

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL TECHNOLOGY

( common to Computer Science and Engineering, Computer Science and Instrumentation Technology, Electronics and Communication Engineering, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	“The voltages induced in the armature winding of a d.c. generator are inherently alternating in nature” – Explain.

b)	The armature of a 6-pole d.c. generator having 650 conductors generates an induced e.m.f. of 536.25 volts when running at a speed of 300 r.p.m., the flux per pole being 55 mWb.  What is the type of the simplex winding used?  Derive the formula used.

2.a)	Discuss the current-speed and current-torque characteristics of a d.c. series motor.

b)	The armature of a 6-pole, lap-wound d.c. shunt motor takes 400A at a speed of 300 r.p.m.  The flux/pole is 0.008 Wb and the number of turns is 600.  Calculate the brake horse power, if 2.5% of the torque is lost in stray losses.  Derive the formula used.

3.a)	Discuss the factors that favour the use of silicon steel for the core of a transformer.  Also, explain why it should be laminated.

b)	A 6,600/600-V, 50-Hz single phase transformer has a maximum flux  density of 1.2 T in its core.  The net area of cross-section of the core is 200 cm2.  Find the number of turns in the primary and secondary windings.  Derive the formula used.

4.a)	Explain the step-by-step development of the equivalent circuit of a single phase transformer.

b)	The o.c. and s.c. tests on a 4 KVA, 200/400-V, single phase transformer gave the following results.

O.C. test :	200V, 0.8A; 80W on l.v. side.

S.C. test:	20V; 10A; 100W on h.v. side.

Calculate the KVA load for maximum efficiency and the maximum efficiency at 0.8 p.f. lagging.

5.a)	“A 3-phase induction motor may be thought of as a Generalised Transformer” – Justify.

b)	From fundamentals, establish that the maximum torque developed by an induction motor is independent of the rotor resistance.

Contd………2

Code No.220402				-2-				      Set No.2

6.a)	Explain what you mean by the winding factor of an a.c. armature winding.  What is its influence on the generated e.m.f.?

b)	A 3-phase, star-connected alternator is rated at 1,500 KVA, 13,500V.  The synchronous impedance is (1.2+j24) ohms/phase.  Calculate the percentage voltage regulation for a load of 1,200 KVA at 0.8 p.f. lagging.  Derive the formula used.

7.a)	“A single-phase induction motor is to be converted into a two-phase induction motor at the time of starting”.  Why and how?

b)	With a neat sketch, explain the principle of operation of a shaded-pole motor.

8.a)	Compare and contrast the stator-resistance and stator-reactance methods of starting 3-phase induction motors.

b)	“A 3-phase synchronous motor can develop a unidirectional torque at the synchronous speed only” – Justify.

- - -

Code No.220402

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL TECHNOLOGY

( common to Computer Science and Engineering, Computer Science and Instrumentation Technology, Electronics and Communication Engineering, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	“Though the voltages induced in the armature winding of a d.c. generator are alternating in nature, yet the output voltage is a d.c. voltage”-Explain.

b)	A certain wave-wound d.c. generator, running at a speed of 300 r.p.m. is to generate an induced e.m.f. of about 535 volts, the flux /pole being 0.055 Wb.  Determine the number poles if the number of conductors is 650.  Derive the formula used.

2.a)	Discuss the current-speed and current-torque characteristics of a d.c. shunt motor.

b)	The armature of a 6-pole, lap-wound d.c. shunt motor takes 380A at a speed of 350 r.p.m.  The flux/pole is 0.075 Wb and the number of armature conductors is 1,200.  Calculate the brake horse power, if 3% of the torque is lost in the stray losses.  Derive the formula used.

3.a)	Discuss in detail why the core of a transformer should be made of a magnetic material and the necessity of laminating the core.

b)	A 200 KVA; 3300/240-V; 50-Hz single phase transformer has 80 turns on the secondary winding.  Assuming an ideal transformer, calculate the primary and secondary currents on full-load.  Derive the formula used.

4.a)	Draw and explain the phasor diagram of a single phase transformer supplying a leading p.f. load.

b)	The O.C. and S.C. tests on a 4 KVA, 200/400-V, single phase transformer gave the following results:

O.C. test :	200V;	0.8A;	80W on the l.v. side.

S.C. test:	20V;	10A;	100W on the h.v. side.

5.a)	Explain the principle of operation of a 3-φ Induction motor.

b)	Discuss the effect of the rotor resistance on (i) the maximum torque and             (ii) the slip-torque characteristics of a 3-phase induction motor.

Contd…….2

Code No.220402					-2-			        Set No.3

6.a)	What is meant by the coil-span factor of an a.c. armature winding?  What are the advantages of using short-pitched windings?

b)	A 3-phase, star-connected alternator is rated at 1,500 KVA, 13,500V.  The synchronous impedance is (1.2+j24) ohms/phase.  Calculate the percentage voltage regulation for a load of 1,200 KVA at 0.8 p.f. leading.  Derive the formula used.

7.a)	Explain in detail why a single phase induction motor is to be provided with an auxiliary starting winding.

b)	Explain the principle of operation of a capacitor-start induction motor and mention some of its applications.

8.a)	Discuss the Rotor resistance starting of a 3-phase induction motor and its merits.

b)	Explain the role of the Damper windings at the time of starting a 3-phase synchronous motor.

- - -

Code No.220402

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

ELECTRICAL TECHNOLOGY

( common to Computer Science and Engineering, Computer Science and Instrumentation Technology, Electronics and Communication Engineering, Electronics and Control Engineering, Computer Science and Systems Engineering, Electronics and Telematics)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.  a)	Explain how the a.c. voltages induced in the armature winding of a d.c. generator and converted into a d.c. voltage at the output terminals.

b)	A 6-pole, wave-wound d.c. generator has 650 conductors.  The flux per pole is 0.05 Wb. Calculate the speed at which it is to be driven to generate an e.m.f. of 550 volts.  Derive the formula used.

2.  a)	From fundamentals, show that the e.m.f. induced in the armature of a d.c. motor opposes the applied voltage and that it should be less than the applied voltage.

b)	The armature of a 6-pole, lap wound d.c. shunt motor takes 400 A at a speed of 350 r.p.m.  The flux/pole is 80 m Wb and the number of turns is 600.  Calculate the brake horse power if 3% of the torque is lost in mechanical and magnetic losses.  Drive the formula used.

3.  a)	Explain the salient features of the construction of the core of a core-type transformer.

b)	A single phase transformer has a core with a cross-sectional area of 150 cm2.  It operates at a maximum flux density of 1.1 Tesla from a 50-Hz supply.  Determine the output KVA when connected to a load of 4 ohms impedance.  The secondary winding has 66 turns.

4.  a)	Discuss the various losses that occur in a transformer and obtain the condition for the efficiency to be a maximum.

b)	The O.C. and S.C. tests on a 4 KVA, 200/400-V, Single phase transformer gave the following results.

O.C. Test	:	200V, 	0.8A, 	80W on l.v. side.

S.C. Test	:	20V, 	10A,	100 W on h.v. side.

5.  a)	“A 3-phase induction motor can never operate with  a slip equal to zero” – Discuss.

b)	From fundamentals, establish that the torque developed by a 3- Induction motor is inversely proportional to its standstill reactance/phase.

Contd…….2

Code No.220402				    -2-			            Set no.4

6.  a)	Explain why a.c. armature windings are normally distributed and short-pitched.

- A 3-phase, star-connected alternator is rated at 1,600 KVA; 13,500V.  The synchronous impedance is (1.5 + j30) ohms/phase calculate the percentage voltage regulation for a load of 1,280 KW at 0.8 p.f. lagging.  Derive the formula used.

7.  a)	Explain why a single phase induction motor has no starting torque.

- Explain the principle of operation of a stepper motor and mention same of its applications.

8. 	Write short notes on:

(a) Any two methods of starting 3-phase induction motors, together with their relative merits and demerits.

(b) Principle of operation of a 3-phase synchronous motor.

- - -