Code No.221101

Set No.

1

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

PROBABILITY AND RANDOM VARIABLES

( Bio-Medical Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the significance of the probability density function of a continuous random variable and state the properties of the probability density function.

b)	Consider the probability density function fX(x)=ae-b|x|, where X is a random variable whose allowable values range from x=- to x=+.  Find the cumulative distribution function F(x) and the relationship between a and b.  What is the probability that the outcome X lies between 1 and 2?

2.a)	Define the mean value and the variance of a continuous random variable.  Calculate these results for a random variable X, whose probability density function is given below:

fX(x)=1/(b-a)		a≤x≤b

=0		elsewhere

- The independent random variables X and Y have the probability density functions fX=1a[4(x)-4(x-a)] and fy(y)= 1b[4(y)-4(y-b)]  with o<a<b.  Find and plot the probability density function of the variable Z=X+Y.

3.a)	Prove the random process X(t)=cos (ct+) is wide-sense stationary if it is assumed that c is a constant and  is a uniformly distributed variable in the interval [O, 2].

b)	What are the properties of the power spectral density of a random process?

4.a)	A random variable x has the probability density function

fX(x)=332 (-x2+8x-12)		2≤x≤6

=0					elsewhere

Find the mean and variance of the random variable X.

- The autocorrelation function of a stationary random process X(t) is given by RXX()=(252+36)/(6.25 2+4).  Find the mean and variance of the random process.

5.a)	Explain the following:

(i) Noise sources,  (ii) noise temperature  (iii) Available noise bandwidth.

b)	Derive the noise figure of a two stage cascaded amplifier whose available power gains of the first stage and the second stage are GA and GB, respectively.  The individual noise figures of the two stages are FA and FB.

Contd……….2

Code No.221101				-2-				Set No.1

6.	Consider five messages given by the probabilities 12,14,18,116,116.

(a) Calculate the average information of the messages.

(b) Use the Shannon-Fano algorithm to develop an efficient code and for that code, calculate the average number of bits/message.

7.a)	Consider a binary symmetric channel characterized by the transmission probability of 0.6.  Calculate the mutual information of the channel when the a-priori probability of symbol 1 at the channel input is 0.5.

b)	State and prove the Shannon-Hartely law of information theory.

8.	Write short notes on any THREE of the following:

a) Rayleigh probability density function.

b) Central limit theorem

c) Huffmen source encoding

d) Properties of auto correlation of random process.

- - -

Code No.221101

Set No.

2

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

PROBABILITY AND RANDOM VARIABLES

( Bio-Medical Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define the probability density function of a continuous random variable.  State and prove the properties of the probability density functions.

b)	Suppose a certain random variable X has the cumulative distribution function (CDF)

FX(x) 	= 0		x≤0

=Kx2		0<x≤10

=100K		x>10

Evaluate K, find the values of probability P(X≤5) and P(5<X≤7) and plot the corresponding probability density function (PDF).

2.a)	Find out the characteristic function of a random variable X having a probability density function given below.  Then, obtain the mean value of the random variable X form the derived characteristic function.

FX(x) = 1b exp [-(x-a)/b]	x>a

=0			x<a

- State the central limit theorem.  Prove the theorem in the case of a set of statistically independent Gaussian random variables.

3.a)	The autocorrelation function of a wide-sense stationary random process is given by

RXX()=4τ2+100τ2+4  Find the mean and variance of this process.

- A random signal X(t), characterized by the autocorrelation function

RXX()=exp[-2||], where  is a constant, is applied to the low-pass RC filter shown in Fig.1.  Determine the power spectral density and autocorrelation function of the output.

R

Input	 C	   Output

Fig.1

Contd…….2

Code No.221101				-2-			              Set No.2

4.a)	Consider a random process X(t) defined by X(t)=Sin (2f t) in which the frequency f is a random variable uniformly distributed over the interval [O, W].  Show that X(t) is nonstationary.

b)	Prove the following two properties of the auto correlation function RXX() of a random process X(t):  (i) If X(t) contains a DC component equal to A, then RXX() will contain a constant component equal to A2.  (ii) If X(t) contains a sinusoidal component, then RXX() will also contain a sinusoidal component of the same frequency.

5.a)	Explain the following: (i) Noise figure  (ii) Noise temperature  (iii) Quadrature representation of narrow band noise.

b)	Find the rms noise voltage across terminals aa' for the network shown in Fig.2

a

1 H

1 F	1 F

1Ω	           1Ω

a1

Fig.2

6.a)	A binary source is emitting an independent sequence of 0’s and 1’s with probabilities p and 1-p, respectively.  Calculate and plot the entropy of this source as a function of p(0<p<1).  Show that the source entropy is maximum when the symbols occur with equal probabilities.

b)	A source is transmitting messages A, B, C, D, E, and F with probabilities

P(A) = 0.4; P(B)=0.03; P(C)=0.02; P(D) = 0.2; P(E)=0.2; P(F)=0.15.  Find the variable length source code words using Shannon-Fano encoding scheme.

7.a)	State and prove Shannon-Hartely law of information theory.

b)	In a facsimile transmission of picture, there are about 2.25 x 106 picture elements per frame.  For a good reproduction, 12 brightness levels are necessary.  Assume all these levels equally likely to occur.  Calculate the channel bandwidth required to transmit 1 picture every 3 minutes.  Assume the signal to noise power ratio over the channel to be 30 dB.

8.	Write short notes on any THREE of the following :

a) Rayleigh probability density function.

b) Mutual information

c) Noise sources

d) Available noise bandwidth.

- - -

Code No.221101

Set No.

3

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

PROBABILITY AND RANDOM VARIABLES

( Bio-Medical Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the relation among the probability, probability cumulative distribution function and probability density function of a continuous random variable.

b)	Let fX(x) =xe-xu(x).  Find cumulative distribution function FX(x) and evaluate the probabilities P(X≤1), P(1<X≤2) and P(X>2).  Note fX(x) is the probability density function of X.

2.a)	If X is a normal random variable with probability density function

fX(x) =1σ2Π exp[-(x-m)2/(22)]

where m and  are parameters.  Prove that the mean and variance of the normal random variable are m and 2 respectively.

- State the central limit theorem.  Prove the theorem in the case of a set of statistically independent Gaussian random variables.

3.a)	Consider the sinusoidal process X(t)=A Cos (2fct)  where the frequency fc is constant and the amplitude A is uniformly distributed:

fA(a) =1,	0≤a≤1

=0,	elsewhere

Determine whether or not this process is strictly stationary.

- What is narrow band noise?  Explain its representation in quadrature components.

4.a)	Find the mean and variance of a stationary random process whose autocorrelation function is given by

RXX()=18+26+τ2

- A white Gaussian noise process of zero mean and power spectral density No2 is applied to the input of the high-pass RL filter shown in Fig.1.  Find the autocorrelation function and power spectral density of the random process at the output of the filter.

R

Input	          L	Output

Fig.1					Contd….2

Code No.221101				-2-			               Set No.3

5.a)	Explain the following: (i) Noise sources  (ii)Noise Figure  (iii) Noise temperature.

b)	A receiver connected to an antenna whose resistance is 50Ω has an equivalent noise resistance of 30Ω.  Calculate the receiver’s noise figure in decibels and its equivalent noise temperature.  The standard temperature is assumed as 300oK.

6.	A source is transmitting messages A, B, C, D, E and F with probabilities P(A)=12; P(B)= 132; P(C)= 116; P(D)= 14; P(E)= 132; P(F)= 18

(a) Calculate the entropy of the source  (b) Find the source code words using Shannon-Fano encoding scheme.  Also find the average length of the thus obtained code words.

7.a)	Calculate the capacity of a Gaussian channel with a bandwidth of 1 MHz and S/N ratio of 30 dB.  How long will it take to transmit one million ASCII characters over this channel? (In ASCII code, each character is coded as an 8-bit binary word.)

b)	A non symmetric binary channel is shown in Fig.2.  Calculate the mutual information of the channel when P(X=0)=1/4; P(X=1)=3/4.

0.2

0  	          0

X			    Y

0.3		        0.8

1			     1

0.7

8.	Write short notes on any THREE of the following:

a) Rayleigh probability density function

b) Huffman source encoding scheme

c) Trade-off between bandwidth and SNR

d) Autocorrelation of random process.

- - -

Code No.221101

Set No.

4

II-B.Tech. II-Semester –Supplementary-Examinations January-2003.

PROBABILITY AND RANDOM VARIABLES

( Bio-Medical Engineering)

Time: 3 hours							   Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Determine which of the following are valid cumulative distribution functions:

(i) FX(x) = 1-eXp[-x/2], x0

= 0		,  x<0

(ii) FX(x)=0		, x<0

=0.5+0.5sin[(x-1)/2], 0≤x<2

=1			   ,  x2

(iii) FX(x) = xa[4(x-a)-4(x-2a)]

- Let the probability density function of a random variable X be                         fX(x) = 0.5 exp[-|x|].  Find the cumulative distribution function FX(x) and use it to evaluate probabilities P(X≤0), P(0<X≤1) and P(X>1).

2.a)	Define the conditional probability density function of a continuous random variable.  What are the properties of the conditional probability density function?

b)	The joint probability density of the random variables X and Y is

fX,Y(x,y)=0.25 eXp[-|X|-|Y|]	-<x<

-<y<

(i) Are X and Y statistically independent random variables?  (ii) Calculate the probability that X≤1 and Y≤0.

3.a)	What are the properties of the autocorrelation function of a random process?

b)	The autocorrelation function of a stationary random process X(t) is given by

RXX()=25+41+6τ2  Find the mean and variance of the process.

4.a)	Consider a random process X(t) defined by X(t) = Cos (t+) where  is a constant and  is a random variable with a probability density function

fX()=12Π		,	0≤≤2

=0			,	elsewhere

Determine the power spectral density of X(t).

Contd…….2

Code No.221101				-2-				      Set No.4

- For the RC network shown in Fig.1, derive the expression for the root mean square value of the noise voltage across the capacitor terminals.  The temperature of the resistor R is ToK.

R                                C

Fig.1

5.a)	Explain the following:  (i) Thermal noise  (ii) Effective input noise temperature  (iii) Narrow band noise.

b)	A parallel-tuned circuit, having a Q of 20, is resonated to 200 MHz with a 10 pF capacitor.  If this circuit is maintained at 17oC, what noise voltage will a wideband voltmeter measure when placed across it?

6.	A source emits independent sequence of symbols from a source alphabet containing five symbols with probabilities 0.4, 0.2, 0.2, 0.1 and 0.1

(a) Compute the entropy of the source  (b) find the variable length source code words using the Huffman code scheme.

7.a)	Prove that the mutual information of a channel is always symmetric and nonnegative.

b)	A black and white TV picture consists of 625 lines of picture information.  Assume that each line consists of 500 picture elements and that each element can have 256 brightness levels.  Pictures are repeated at the rate of 25/sec.  Calculate the bandwidth required when a signal to noise ratio of 1000 is specified for a satisfactory reproduction of the picture.

8.	Write short notes on any three of the following:

a) Central limit Theorem

b) Shannon-Fano source encoding scheme

c) Trade off between bandwidth and SNR

d) Noise Figure of cascaded amplifier.

- - -