Code No: 221151

### OR

# II B.Tech. II Semester Supplementary Examinations January,  2003

# BIO-MEDICAL INSTRUMENTATION

(Bio-Medical Engineering)

Time: 3 Hours							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.  a)	Explain about “Micro Electrodes” based on micro electronic technology.

b)	Briefly Explain about (i) Insulated needle electrode  (ii)  Co-axial needle electrode  (iii) Bi-polar co axial electrode

2.  a)	Explain about care and maintenance in using electrodes.

- Explain about different Body-Surface recording electrodes.

3.  a)	Explain about Piezo-Electric Sensors with its equivalent circuit.

b)	i)  A Piezo - Electric Sensor has C = 500 PF.   The Sensor leakage resistance is 10 G and the amplifier input impedance is 5M.  What is the low corner frequency.  (ii) Briefly discuss about V-I response of a Piezo-electric sensor to a step displacement “x”.

4.	Write a short notes on the following:

- Carbon micro phones  (b) Photo transistors  (c) LDR

5.  a)	The output of a biopotential pre-amplifier that measures the electro-oculogram is an undesired dc voltage of  5V due to electrode half-cell potentials, with a desired signal of IV super imposed.  Design a circuit that will balance the dc voltage to zero and provide a gain of –10 for the desired signal without saturating the OP-amp.

b)	Explain the operation of an Instrumentation amplifier with the help of a neat circuit diagram.  Also explain how its gain is calculated

6.  a)	What are the General considerations for Bio-electric recorder amplifiers.

b)	With the help of a neat block diagram explain the principle of operation of the carrier amplifier.

7.  a)	Write about Direct reading spectrophotometer

b)	Explain about Microprocessor based spectrophotometers.

8.  a)	Briefly write about Ultrasonic Theraphy.

- Explain the principle of operation of a haemodialyzer.

@@@@@