Code No: 320303

Set No

1

III-B.Tech. II Semester Examination April, 2003

AUTOMOBILE ENGINEERING

(common to Mechanical Engineering, Production Engineering, Mechatronics, and Mechanical Manufacturing Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Compare the merits of front wheel drive vehicles with rear engine wheel drive 	vehicles.

b)	What is frameless construction? What are its advantages?

2.a)	What are the limitations of simple carburetor?

b)	How can the starting difficulty with simple carburetor  be over come?

3.	Explain the various types of engine cooling systems and compare them.

### 4.a)	Explain the following:

i)	 Firing order

ii)	 Ignition advance.

b)	What are the requirements of a good sparkplug?

5.a)	What is meant by recharging battery? When and how is the battery  recharged?

b)	Explain the Bendix drive starting mechanism with neat sketch.

6.	Write notes on:

i)	Automatic power transmission

ii)	Clutch for mopeds.

iii)	Wet clutches

iv)	Gearbox lubrication.

7.a)	Explain the salient features of torque tube drive with a neat sketch.

b)	Explain the construction and working of fully floating rear axle.

8.a)	 Explain clearly how the King-Pin inclination produces directional stability?

b)	Explain why do the front wheels have to toe-out in turns?

c)	Explain what is meant by center point steering.

$$$

Code No: 320303

Set No

2

III-B.Tech. II Semester Examination April, 2003

AUTOMOBILE ENGINEERING

(common to Mechanical Engineering, Production Engineering, Mechatronics and  Mechanical Manufacturing Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Give classification of Internal Combustion engines.

b)	 How does a two-stroke engine differ from a four-stroke engine?

2.a)	Describe the distributor type fuel injection pump.

b)	Explain the various types of injector nozzles.

3.a)	Sketch and explain the working of water pump.

b)	Describe troubles of cooling systems and their remedies.

4.a)	What are the important requirements of high voltage ignition source for the spark ignition process.

b)	Sketch and explain the working of capacitor in the ignition system.

5.a)	What is the importance of temperature indicator?

b)	Explain the working of a temperature indicator used in automobile engines.

6.a)	Explain the construction and use of Overdrive? List out its advantages and limitations.

b)	Explain the working of a dog clutch.

7.a)	Sketch the sectional view of the tyre and explain its various parts.

b)	Discuss the constructional details of a cross ply tyre.

c)	What are the functions of tyres?

8.a)	Arrange the following in correct sequence of the path of force transmission from the steering wheel to the front wheels in a steering system:

Steering knuckle, arm drag link, steering wheel, pitman arm, steering gear, tie rod, steering column, steering arm, front wheels and stub axle.

b)	Explain the various steering troubles and suggest their remedies.

$$$

Code No: 320303

Set No

3

III-B.Tech. II Semester Examination April, 2003

AUTOMOBILE ENGINEERING

(common to Mechanical Engineering, Production Engineering, Mechatronics and Mechanical Manufacturing Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	What kinds of oil pumps are used in various lubrication systems?

b)	Sketch a gear type oil pump and explain its construction and working.

2.a)	How is the diesel fuel pump  tested for its performance?

b)	What is the purpose of helical groove in the diesel pump? Sketch its position for	 	 different operating conditions of the engine.

3.a)	What are the requirements of a cooling system.

b)	Explain the working of forced circulation cooling system.

# 4.a)	Explain the constructional features of any one type of auto transformer

# b)	Sketch  and name the parts of a spark plug.

5.a)	What is the necessity for dipper provided in headlights and how is it 	accomplished?

b)	What are the various types of head lamps? Give details.

6.a)	What are the requirements of a clutch?

b)	With the help of a neat sketch, explain the construction and operation of a sliding mesh gearbox.

7.a)	What are the types of suspension system?

b)	Sketch and explain the construction and working of wishbone type independent

front suspension used on any Indian vehicle.

8.	Write notes for the following:

i)	Hill-holder.

ii)	Leading and trailing shoes.

iii)	Requirements of brake fluid.

iv)	Brake shoe material.

$$$

Code No: 320303

Set No

4

III-B.Tech. II Semester Examination April, 2003

AUTOMOBILE ENGINEERING

(common to Mechanical Engineering, Production Engineering, Mechatronics and Mechanical Manufacturing Engineering.)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Describe with neat sketches the construction and functions of rotor and vane type 	pump.

b)	What are the different types of oil filters in automobile use?  Explain the 	construction and functions of Cartridge type oil filter.

2.a)	Draw the layout of fuel feed system in a petrol engine.  Name and give the 		  functions of each component.

b)	Explain the mechanism provided in a A C mechanical pump to stop the fuel		 supply when the carburetor float chamber is full.

3.a)	Explain the working of evaporative cooling system.

b)	What are the merits of evaporative cooling system over conventional water-	cooling system?

# 4.	Write notes on:

i)	Distributor order

ii)	Ignition Harness

iii)	Condenser

iv)	Contact break points.

5.	Draw and explain the wiring circuit for a passenger car and give the function of each component circuit.

6.a)	Explain the working of a multi-plate clutch with   a suitable sketch. List out the 
advantages of it over single plate clutches.

b)	Explain the principle of clutch.

7.a)	Discuss the merits of independent front suspension system.

b)	Sketch a typical front suspension of a car and name its components.

8.a)	Why is a parking brake provided on four wheelers?

b)	Explain the working of a vacuum brake with a neat sketch.

c)	How is Vacuum brake different from an air brake?

$$$