Code No.321501

Set No.

1

III-B.Tech.  II  Semester Examinations April, 2003

OPERATIONS RESEARCH

(Computer Science and Systems Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a) 	What is Operations Research?

b) 	Explain the concept, scope and tools of Operations Research as applicable to 	business and industry.

2.a)	Explain the use of artificial variable and its use in linear programming.

b) 	An animal feed company must produce 200 Kg of a mixture containing of 	ingredients A and B respectively. Ingredient A Costs Rs. 3/- per Kg. And B costs 	Rs. 8/- per Kg. No more than 80 Kg of A can be used and at least 60 Kg of B 	must be used. Find how much of each ingredients should be used if the company 	wants to minimize cost.

3.	Find sequence of production as to minimize set up cost per cycle of  the matrix

To item

A	B	C	D	E

A		5	8	4	5

From	B	5		7	4	5

item	C	8	7		8	6

D	4	4	8		8

E	5	5	6	8	

4.	The mean rate of arrival of planes at an airport during the peak period is 20 hour, 	but the actual number of arrivals in any hour follows a Poisson distribution with 	the respective averages. When there is congestion, the planes are forced to fly 	over the field in the stack awaiting the landing of other planes that arrived earlier.

  - How many planes would be flying over the field in the stack on an average in good weather and in bad weather?

  - How long a plane would be in the stack and in the process of landing in good and in bad weather?

  - How much stack and landing time to allow so that priority to land out of order would have to be requested only one time in twenty?

5.	Machine A costs Rs.9000/-. Annual operating costs are Rs200/- for the first year 	and then increased by Rs2000/- every year. Determine the best age at which to 	replace the machine with the optimum replacement is followed:

Machine B costs Rs.10000/- annual operating costs are Rs.400/- for the first year 	and increases by Rs800/- every year. You have now a machine of type A, which 	is one year old. Should you replace it with B, and if so, when? Assume that both 	the machines have no resale value when replaced and that future costs are not 	discounted.

Contd…..2

Code No:321501				-2-				Set No:1

6.a)	Explain the following:

(i). Two-person zero-sum game    (ii) Saddle point.

b) 	Solve the following game graphically:

Player A

| A1 Player B B1 B2 | A2 | A3 | A4 |
| --- | --- | --- | --- |
| -6 | 0 | 6 | -1.5 |
| 7 | -3 | -8 | 2 |

7.a)	A manufacturer has to supply 12000 units of a product / year to his customer. The 	demand is fixed and known and the shortage cost is assumed to be infinite. The  	inventory holding cost is Rs. .2 /unit / month and the set up cost / run is Rs. 350/- 	Determine.

i) The optimum run size.

ii) Optimum scheduling period.

iii) Minimum total variable yearly cost.

b) 	What are probabilistic?  Explain.

8.a)	Explain the concept of dynamic programming.

b)	Solve the following problem by using dynamic programming:

Max. 12 x13+27x23+147x33 in non negative xi subject to i−1nxi=1.

- - -

Code No.321501

Set No.

2

III-B.Tech.  II  Semester Examinations April, 2003

OPERATIONS RESEARCH

(Computer Science and Systems Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain the meaning and nature of Operations Research.

b) 	Write a note on Scope of Operations Research.

2.	An aviation fuel manufacturer sells two types of fuel i.e. Fuel A and Fuel; B. 	Type A fuel is 25% grade 1 gasoline, 25% grade 2 gasoline and 50% of grade 3 	gasoline. Type B fuel is 50% grade 2 gasoline and 50% grade 3 gasoline. 	Available for production are 500 liters per hour of gosling grade 1 and 200 liters 	per hour of grade 2 and 3 each. Costs are 60 paise per liter for grade 1, 120 paise 	per liter for grade 2 and 100 paise per liter for grade 3. Type A can be sold at

Rs. 7.50 per liter while type B sells at Rs.9/- per liter.

a) How much of each fuel should be made for maximizing the profit?			b) Write the dual of the above problem and solve it and interpret the values.

3.a)	Give a brief note of Johusan & Bellman rule.

b)	There are seven jobs as under each of which has to go through machines A and B in the order BA.  The processing time is in hours.

Job		1	2	3	4	5	6	7

Machine A	12	10	10	6	8	1	3

Machine B	10	12	15	6	3	11	9

Determine the sequence of jobs that will minimize the total time for completion of all the jobs.

4.	A telephone booth with Poisson arrivals spaced 10 minutes apart on the average, and exponential call lengths averaging 3 minutes.

a) What is the probability that an arrival will have to wait more than 10 	minutes before the phone is free?

b) What is the probability that it will take him more than 10 minutes before 	altogether to wait for phone and complete his call?

c) Estimate the fraction of a day that the pone will be in use.

d) Find the average number of units in the system.

Contd……2

Code No:321501				-2-				Set No:2

5.a)	Discuss replacement policy.

b)	 The maintenance and resale value per year of a machine whose purchase price is 	Rs.7000/- is given below:

| Year | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Maintenance cost in Rs. | 900 | 1200 | 1600 | 2100 | 2800 | 3700 | 4200 |
| Resale value in Rs. | 4000 | 2000 | 1200 | 600 | 500 | 400 | 400 |

When should the machine be replaced?

6.a)	Explain the following :

(i). Pay -off  (ii) Saddle point.

b)	Solve the following game what are the optimal shortages for from I and II:

Firm II

|  | No advertising | Medium advertising | Large advertising |
| --- | --- | --- | --- |
| No advertising Firm I | 60 | 50 | 40 |
| Medium advertising | 70 | 70 | 50 |
| Large advertising | 80 | 60 | 75 |

7.a)	A company producing 3 items has a limited storage space of averagely 750 items 	of all types. Determine the optimal production quantities for each item separately 	when the following information is given:

| Product | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Holding Cost ( Rs. ) | 0.05 | 0.02 | 0.04 |
| Set up Cost ( Rs. ) | 50 | 40 | 60 |
| Demand rate ( per unit ) | 100 | 120 | 75 |

b) 	What is quantity discounts? Explain briefly.

8.a)	Discuss the relation between linear and dynamic programming.

b)	Solve the following problem by using dynamic programming:

Max. Z= 2x1 + 9 x2

subject to 2x1 + x2 ≤ 25

x2 ≤ 11

x1, x2 ≥ 0.

- - -

Code No.321501

Set No.

3

III-B.Tech.  II  Semester Examinations April, 2003

OPERATIONS RESEARCH

(Computer Science and Systems Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define Operations Research and discuss its scope.

b) 	Discuss the Scientific Method in Operations Research.

2.a)	Solve the problem by using Simplex Method:

Maximize 10 x + 6y + 4 z subject to

1x + 1y + 1 z            100

10 x + 4 y + 5 z        600

2 x + 2 y + 6 z          300    and x, y, and z all     0.

b) 	What are slack variables? Give physical interpretation of the same.

3.	State the conditions under which the problem of processing of jobs through three machines has been solved.  Describe the corresponding algorithm.

Find the sequence that minimizes the total time required to complete the following tasks:

| Task | A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Machine I | 9 | 8 | 7 | 4 | 3 | 8 | 7 |
| Machine II | 1 | 3 | 2 | 5 | 4 | 4 | 3 |
| Machine III | 5 | 7 | 5 | 11 | 6 | 6 | 12 |

4.	A company distributes its product by trucks loaded at its only loading station. Both company’s trucks and contractor’s trucks are used for this purpose. It was found out that on an average every five minutes on truck arrived and the average loading time was three minutes. 50% of the trucks belong to the contractor. Find out:

a) The probability that a truck has to wait.

b) The waiting time of truck that waits and

c) The excepted waiting time of contractor’s trucks per day assuming a 24 hours shift.

Contd…..2

Code No:321501				-2-				Set No:3

5.	A manufacturer finds from his records that the cost per year associated with the machine whose purchase price is Rs 5000/- are as follows:

| Year | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Running cost in Rs. | 1500 | 1600 | 1800 | 2100 | 2500 | 2900 | 3400 | 4000 |
| Resale value in Rs. | 3500 | 2500 | 1700 | 1200 | 800 | 500 | 500 | 500 |

Determine age at which the machine should be replaced.

6.a)	Explain the following:

(i) competitive games     (ii) Two-person zero -sum game.

b)	Solve the following game using dominance principle.

Player B

|  | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| I | 19 | 6 | 7 | 5 |
| II | 7 | 3 | 14 | 6 |
| III | 12 | 8 | 18 | 4 |
| IV | 8 | 7 | 13 | -1 |

Player A

7.		A Company uses annually 24000 units of raw material which costs 1.25 / units

placing each order costs Rs. 22.5 and carrying cost is 5.4 % per year of the 	average inventory . Find E.O.Q. and the total inventory cost should the company 	accept 	the offer made by its supplier of a discount of 5 % on the cost price on a 	single 	order of  24000 units. Suppose the company works for 300 days a year if 	the procurement time is 12 days safety stock is 400 units find the reorder point the 	minimum, maximum and average inventory.

8.a)	Enumerate Ballman’s principle of optimality.

b)	Consider loading a vessel with stock of three types of items.  Each unit of item i 	has a weight wi and a value ri (r=1, 2, 3).

i		wi		ri

1		2		30

2		6		80

3		4		65

The maximum cargo weight W is 10.  It is required to determine the most 	valuable cargo load without exceeding the maximum weight of the vessel.  Use 	Dynamic Programming technique.

- - -

Code No.321501

Set No.

4

III-B.Tech.  II  Semester Examinations April, 2003

OPERATIONS RESEARCH

(Computer Science and Systems Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.	Write an essay on Scope, Methodology and Phases of Operations Research.

2.a)	Explain the meaning of unbounded solution.

b) 	Minimize Z = 3 w+ 2 x+ 1 y+ 4 z  subject to

2 w + 4 x + 5 y + 1 z        10

3 w – 1 x + 7 y – 2 z          2

5 w + 2 x + 1 y + 6 z          15    and w, x, y and z all        0.

3.	Solve the following traveling salesman problem:

From/To	1	2	3	4	5	6

1		8	5	9	7	6

2	8		9	6	4	5

3	5	9		10	3	12

4	9	6	10		8	7

5	7	4	3	8		4

6	6	5	12	7	4	

4.	A repairman is to be hired to repair machines, which break down at an average 	rate of 6 per hour. The break down follows Poisson distribution. The productive 	time of a machine is considered to cost Rs.20 per hour. Two repairmen, Mr.X and 	Mr. Y have been interviewed for this purpose. Mr. X charges Rs. 10 per hour and 	he services breakdown machines at the rate of 8 per hour. Mr. Y demands Rs.14 	per hour and he services at an average rate of 12 per hour. Which repairman 	should be hired? (Assume 8 hour shift per day).

5.a)	Explain the major approaches for replacement decisions.

b) 	A machine costs Rs.500/-. The operation and maintenance costs are zero for the 	first year and increased by Rs.100/- every year. If money is worth 5% every year, 	determine the best age at which the machine should be replaced. The resale value 	of the machine is negligibly small. What is the weighted average cost of owning 	and operating the machine?

Contd…..2

Code No:321501				-2-				Set No:4

6.a)	What is two- person zero –sum game? What are its properties?

b)	Solve the following game whose pay-off matrix is given by:

B

1            7            2

A            6           2            7

5           1            6

7.	Given the following data for an item of uniform demand, instantaneous delivery time and back order facility.

Annual Demand = 800 units

Cost of  an Item = Rs. 40/-

Ordering Cost    = Rs. 800/-

Inventory Carrying Cost = 40 %

Back order cost  = Rs. 10/-		Find

a) Minimum cost order quantity.

b) Maximum Inventory level.

c) Maximum number of back orders.

d) Time between orders and

e) Total annual cost.

8.a)	Briefly discuss the applications of dynamic programming.

b)	Solve the following problem by using dynamic programming:

Max. x1 x2 x3

subject to x1+x2+x3 = 8

x1, x2, x3 ≥ 0.

- - -