Code No.320357

OR

III-B.Tech. II-Semester Examination April, 2003

FINITE ELEMENT METHODS

(Common to Mechanical Engineering, Production Engineering, Mechatronics and Mechanical Manufacturing Engineering)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What do you mean by Finite Element method? Write the strain-displacement relationship for plane stress problem.

b)	Use the Rayleigh-Ritz method to find displacement of the midpoint of rod shown in figure1. Body force per unit volume, g = 1.

2.	Describe the following:

a)	Properties of stiffness matrix

b)	Finite element modeling of a plate with hole

c)	Bandwidth and skyline assembly procedure.

3.	For the truss shown in figure.2. Determine the displacements, support reactions and stresses in elements (2) and (3).

(Contd…2)

Code No: 320357			.. 2..					OR

4.	Find the deflection at the load and the slopes at the ends for the steel shaft shown in figure.3. Consider the shaft to be simply supported at the bearings A and B.

5.a)	The nodal coordinates of the triangular element are shown in figure.4. At the point p, the x-coordinate is 3.3 and N1 = 0.3. Determine N2, N3 and the y-coordinate at  point p.

b)	What do you mean by plane stress condition? Estime the strains Ex, Ey and  γxy if

U = -3x2 + 5xy - 6y2

V = 6x2 – 16xy + 20y2

at a point p(-1,-1).

(Contd…3)

Code No: 320357			.. 3..					OR

6.	A long bar of rectangular cross section, having thermal conductivity of 1.5W/moC is subjected to the boundary conditions shown in figure.5. Two opposite sides are maintained at a uniform temperature of 180oC. Determine the temperature distribution in the bar.

7.	Determine the eigen values and eigen vectors with sketches for the shaft shown in figure.6. E = 2E11N/m2 and density  = 7850kg/m3.

8.	Write short notes on any THREE of the following:

- Formulation axisymmetric element.

- Numerical integration and Gaussian points.

- Formulation of analysis of shaft subjected to Torsion.

- Formulation of transient heat transfer, analysis of a fin.

***     ***     ***