Code No.: 321401

Set No.

1

III-B.Tech. II-Semester Examination April, 2003

PRINCIPLES OF MACHINE DESIGN

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Describe the designation of carbon steels as per B.I.S according to chemical 	composition and mechanical properties.  Illustrate with adequate examples.

b)	Compare the maximum principle stress theory with maximum shear stress theory, 	by briefly describing each theory first.

2. a)	 Describe the design procedure of a Split Muff coupling?

b)	Determine the value of ZN/P, the coefficient of friction and heat generated per 	minute for a bearing with: load 3.5kN; speed = 1200 RPM;  Diameter of journal   	= 50mm; Length of journal= 50mm; R/c = 1000 and viscosity of oil = 130 	centipoise at 55oC.

3. a)	What are the factors which reduce the value of endurance limit?

b)	Determine the shaft diameter subjected to varying torque of 200kNm clockwise to 	100kNm anticlock-wise at a point where the bending moment variations of 	400kNm to 200kNm.  Find the shaft diameter if fut = 360 N/mm2 and                     	fyt = 	280N/mm2.  	Assume factor of safety = 2.5

- Design a trunk type cast iron piston for a single cylinder 4-stroke diesel engine running at 450RPM with bore = 0.24m; stroke= 0.3m; Brake mean pressure = 0.8N/mm2.  Max gas pressure of 5 N/mm2; Fuel consumption of 0.18kg/kW/hour.  HCV of fuel = 1100 k Cal/kg.

5.	A pair of spear gears is to transmit 24kW when pinion rotates at 303 RPM.  The velocity ratio is 1:3.  The allowable static stress for the pinion and gear materials are 120N/mm2 and 100N/mm2.  The pinion has 15 teeth and face width is 14 times module.  The form factor can be taken as 0.154-0.912/N; (N= Number of teeth).  Velocity factor = 3/(3+v); V = velocity in m/s.  Find a) face width;          b) Pitch circle diameter of both gear and pinion from the strength aspect with dynamic loading only.

6.	A belt transmits 12kW from a pulley 0.9m diameter running at 500RPM.  The 	density of belt is 0.004 kg/cm3 and maximum stress in belt is 50kg/cm2.  The 	angle of lap is 165o and  = 0.3.  The belt is 4.5mm thick.  Calculate the width of 	the belt.

Contd…2

Code No. 321401				.2.				Set No:1

7.	Design a compression coupling for a shaft of 75mm diameter.  The torque to be 	transmitted is 2.4kNm.  The number of bolts used are six and the allowable stress 	in the material of bolt is 70 N/mm2.  The friction coefficient between shaft and 	muff material is 0.3.  The safe stress in shaft and key= 40N/mm2.

8.	Write short notes on

a)	Shaft design on rigidity basis.

b)	Soderbergs equation

c)	Types of Journal bearings

d)	Stress strain diagram for ductile and brittle materials.

###

Code No.: 321401

Set No.

2

III-B.Tech.  II-Semester Examination April, 2003

PRINCIPLES OF MACHINE DESIGN

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Compare maximum strain Energy theory and distortion energy theory.

b)	What are the factors which influence safety factor of design?  What are the values 	of safety factors which are normally taken for various loading situations?

2. a)	Describe the design procedure of roller bearings.

b)	Design a split Muff coupling to connect two shafts for transmitting 35kW at 	100R.P.M.  The permissible shear stress in the shaft is 40 N/sq.mm.  The number 	of bolts connecting two halves is 6.  The permissible tensile stress in bolts is 	70N/sq.mm.  The coefficient of friction = 0.3

3. a)	What are the design rules to avoid fatigue failures and stress concentration.

b)	A steel rod of ultimate strength 600N/sq.mm. and yield strength of 400N/sq.mm. is subjected to cyclic torque varying from 320kNm to 120kNm.  Calculate the diameter of rod assuming factor of safety = 2; surface factor = 0.88;                   size factor =0.80 and 	stress concentration factor of 1.4.

4.	Design a trunk type of piston of C.I. for 4-stroke diesel engine running at 	400RPM with bore=30 mm and stroke = 40 mm; B.m.e.p=0.75 N/mm2.  Fuel 	consumption = 0.2kg/kW/hour HCV of fuel = 1100 k cal/kg.

5.	A shaft running at 300 RPM is driven by 12kW, 960RPM motor through a pair of 	14.5o involute spur gears.  The centre distance is 400mm.  The gear is to be made 	of cast steel and pinion of forged steel.  Design the gears assuming safe static 	stress for gear and pinion are 140 and 200 N/sq.mm respectively.  Service 	factor=0.65. Check the gear for wear of cast steel with BHN165.

6.	The diameters of driving and driven pulleys are 1m and 1.3m respectively in an 	open belt drive.  The output of the driven shaft is 120kW and centre distance is 	3m.  Assuming friction coefficient as 0.3 and belt speed 20m/s, slip as 1.6% at 	each pulley and friction loss at each shaft as 4% calculate (a) RPM of each shaft 	(b) belt tensions (c) size of belt.

7. a)	A journal bearing 80mm diameter and 120mm long supports a load of 6kN at 	1000RPM.  If the radial clearance is 0.004mm and frictional power is 1kW.  	Calculate the viscosity of lubricating oil.

b)	Compare antifriction bearings with other bearings.

Contd…2

Code No. : 321401				.2.				Set No:2

8.	Write short notes on:

a)	Preferred Numbers

b)	Shaft design for fluctuating loads

c)	V-belt merits over flat belts

d)	Different types of keys.

###

Code No.: 321401

Set No.

3

III-B.Tech. II-Semester Examination April, 2003

PRINCIPLES OF MACHINE DESIGN

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	What are the general considerations to be taken into account while designing a 	machine element?

b)	What are the factors which are essential to be considered while selecting a 	material for design?

2. a)	Describe the design of keys with special reference to rectangular cross section.

b)	Design a Hydrodynamic bearing for the following

Diameter of journal	=	62mm

Speed of shaft		=	174 RPM

Load on shaft		=	7kN steady

Viscosity of oil	=	Say bolt of 270 sec at 38oc.

Specific gravity	=	0.905 at 15.5oC

3. a)	What are the design rules to minimize stress concentration?

b)	Calculate the diameter of a shaft subjected to a maximum load of 34kN and a minimum of 12 kN in tension.  The yield stress is 420MPa and endurance limit of 320MPa.  Factor of safety = 1.5.  What will be the change in diameter of shaft if, stress factor 	of 1.8 is to be used.

4. a)	Determine the size of piston rod subjected to a total load having a cyclic 	fluctuation from 150kN in compression to 250kN in tension.  The endurance limit 	is 360 N/mm2 and yield strength of 400N/mm2.  The impact factor = 1.25; factor 	of safety=1.5;  surface factor is 0.88;  The stress factor is 2.25.

b)	What are the functions of (i) ribs (ii) rings (iii) pin of an I.C Engine piston.

5.	Design a spur gear to transmit 15kW at 150 RPM of Cast Iron with pitch circle diameter of gear = 700mm; maximum load on teeth = 65N/mm of face width. Bending moment on 	shaft = 0.6 times twisting moment.  Safe shear stress in shaft and key = 40N/mm2.  	Assume all other data.

6.	The diameter of a pulley in an open belt drive on driving shaft is 0.5m and 	rotating at 150RPM and counter shaft is driven at 300RPM.  If the centre distance 	is 2.6m and  = 0.3, determine the 	width of the belt so that 4kW is transmitted.  	The permissible stress is 16N/mm width of belt.

Contd…2

Code No. : 321401					.2.			Set No:3

7.	Design a cast Iron flange coupling for a steel shaft transmitting 15kW at 	200RPM; having an allowable shear stress of 40N/mm2 and stress in bolts should 	not exceed 30N/mm2.  Assume same material for shaft, key and crushing stress is 	= 2 x shear stress and maximum torque is equal to 25% more than full torque.  	The shear stress for CI= 4N/mm2.

8.	Write short notes on

a)	Goodman’s design criteria.

b)	Rankines theory and Guest’s theory.

c)	Roller bearing types

d)	Rigid coupling versus Flexible coupling.

###

Code No.: 321401

Set No.

4

III-B.Tech. II-Semester Examination April, 2003

PRINCIPLES OF MACHINE DESIGN

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Describe maximum shear stress theory and distortion energy theory and compare 	their effectiveness.  Which theory yields optimum result?  Why?

b)	Describe the designation of various Cast Irons as per B.I.S?

2. a)	Describe the strength design of shafts under the combined load of torsion and 	bending.

b)	Design a muff coupling to transmit 50kW at 1750RPM; With a breakdown torque 	of 225% the rated torque.  Diameter of bore is 60mm.  Assume the suitable values 	of the stresses.

3. a)	What are the methods of estimating endurance strength?

b)	A shaft is subjected to a torsional load that varies from -100Nm to 440Nm.  Determine the diameter of shaft using factor of safety of 1.6.                           Take fut = 500N/mm2; fyt = 32 N/mm2.

4.	Design a connecting rod for a petrol engine from the following data:

diameter of piston			=	110mm.

Weight of reciprocating parts		=	2kg

Length of connecting between centres = 	325mm

Stroke = 150mm, RPM = 1500 with a possible over speed of 2250.  Compression 	ratio=4:1.  Maximum explosion pressure = 2.5 N/sq.mm.

5.	Design a pair of spur gears for transmitting 40kW at 900RPM to another at 	150RPM.  Static stress for pinion and gear 210 and 80 N/sq.mm.  Velocity factor 	is 3/(3+); form factor is 0.124 - 0.684/N (N= Number of teeth).  Stress fatigue 	factor= 144.  The gears are with 14.5o full depth involute teeth.

6.	A flat belt 0.12m wide and 6mm thick has a belt speed of 600mpm.  The ratio of 	tensions is 2.5.  If the  permissible stress in the belt material is 2.4N/mm2 and 	density of belt material is 1000kg/m3, determine the power transmitted.  Also 	determine the speed at which maximum power is transmitted and maximum 	power transmitted.

Contd…2

Code No. 3210401				.2.				Set No:4

7.	Design a journal bearing for a centrifugal pump running at 1200RPM.  The 	journal is of 8cm and load on each journal is 18kN.  The ZN/P for bearing is 	2600.  The l/d is 1.2.  The correction factor is 0.002.  The bearings and 	atmospheric temperatures are 80oC and 30oC respectively.

8.	Write short notes on

a)	Material selection factors

b)	Preferred Numbers

c)	Advantages of flexible coupling over others

d)	Stress concentration reduction methods.

###