Code No.: 321303

Set No.

1

III-B.Tech. II-Semester Examination April, 2003

ROBOTICS AND AUTOMATION

(Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer and five questions

All questions carry equal marks

---

1. a)	What are the basic components of a robot system?  State the main functions of 	each of the components.

b)	Describe the configuration of a Articulated manipulator.

2. a)	What do you understand by degrees of freedom?  How may Degrees of  Freedom are required to position end effector in 3D space.

b)	Describe the various drives used in robotics.

3. a)	Distinguish between tactile and non tactile sensors?  Explain examples of each 	type.

b)	Discuss the machine vision system

4. a)	What do you mean by Homogeneous co-ordinates?  Explain the D-H notation for 	robot manipulator.

b)	What are the design considerations for robot grippers?  Explain

5. 	Establish the lagrange Eular equation of motion for the 2-line Manipulator as 	shown in Fig.1 (Assume d1 = d2 = 0, a1 = a2 = l, 1 = 2 = 0), m1, m2 are mass of 	the links, 1 and 2 are joint angles.

Fig.

6.	a) What do you  mean by multiple solution?

b) Explain the Jacobian work envelope.

c) Application of robot in spot welding.

Contd…..2

Code No:321303				-2-				Set No:1

7.	a) How do you select a robot?  Explain.

b) Discuss the robot cell design?

c) Compare the differences between the representation of angular velocity and 	kinetic Energy of lagrange-Euler and Newton Euler equation of motion.

8.	Write short notes on any three of the following.

a) Repeatability, accuracy and resolution,  (b) Manipulator joint velocities and 	torques, (c) Hill climbing techniques,  (d) laser, acoustic sensors.

###

Code No.: 321303

Set No.

2

III-B.Tech. II-Semester Examination April, 2003

ROBOTICS AND AUTOMATION

(Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer and five questions

All questions carry equal marks

---

1.	Sketch and explain arm and wrist motions of following configuration.

a) Polar	(b) Cartesian and 	(c) Spherical.

2. a)	Differentiate various types of drivers used in robots with a neat sketch.

b)	Enumerate the advantage, working principle and application of Machine vision 	system.

3. 	Discuss the applications and working principle of the following sensors.

a)	Range sensors

b)	Magnetic and Acoustic sensors

c)	Tactile sensors.

4.	For two link R-R, planar manipulator, obtain joint velocities, given the end point 	velocity as [Vx   Vy]T.  Explain the singular configuration.

5.	Explain D-H notation for establishing link co-ordinate system for cartesian 	manipulator and obtain OA3 Matrix.

6.	a) Sketch the various manipulator control circuits?

b) What is end effector?  Describe the design considerations pf grippers.

c) Sketch various types of grippers.

7.	a) Explain the lagrangian analysis to find the torques required by the attenuator 	for joints.

b) Derive dynamic equations using Newton-Euler formulation for the 2-link I 	manipulator.

8.	Write short notes on any 3 of the following:

a) Machine interface

b) Medical application of robots

c) Robot programming languages

d) Jacobian work envelope.

###

Code No.: 321303

Set No.

3

III-B.Tech. II-Semester Examination April, 2003

ROBOTICS AND AUTOMATION

(Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer and five questions

All questions carry equal marks

---

1. a)	What do you mean by Automation?  Sketch the robot anatomy.

b)	Discuss Asimov’s laws of robotics?  Explain dynamic stabilization of robots.

2. a)	Differentiate between the hydraulic, pneumontic and electric drives used in 	robots.

b)	What do you understand by Degrees of Freedom?  How many Degrees of Freedom are required to position end effector in 3D space.

3. a)	Explain different levels of vision and processes involved.

b)	What is meant by range and proximity sensor describe them with a heat sketch?

4.	Establish D-H representation of establishing link co-ordinates system for 	Cartesian manipulator and obtain OA3 Matrix.

5. a)	Write the Euler angle representation for cylindrical coordinates for positioning the 	end effector.

b)	Sketch various types of grippers.

6.	A 2dof manipulator is shown in Fig.1 The length of each lime is 1m, establish its 	link coordinates frames and OA and 1A2.  Find the inverse kinematics solution 	for this manipulator.

Fig

Contd…2

Code No:321303				-2-				Set No:3

7. a)	Compare the differences between the representation of angular velocity and 	kinetic energy of lagrange Euler and Newton Euler equation of motion.

b)	Explain machine interface?

8.	Write short notes on any three of the following

a) Jacobian work envelope.	b) Robot programming languages

c) Robot applications		d) Fiber optic sensors

###

Code No.: 321303

Set No.

4

III-B.Tech. II-Semester Examination April, 2003

ROBOTICS AND AUTOMATION

(Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 80

Answer and five questions

All questions carry equal marks

---

1. a)	Sketch the two configurations for the jointed arm robot and explain the work 	volume.

b)	What do you mean by Degrees of Freedom?  Define the following terms (i) work envelope (ii) Repeatability (iii) Accuracy.

2. a)	One of the axes of robot is a telescoping arm with a total range of 0.5m.  The 	robots control memory has an 8-bit storage capacity for this axis.  Determine the 	control resolution.

b)	What is the resolution (in degrees) and output value of an encoder with 12 tracks.  	Assume the shaft angle a 10 radians.

3. a)	 Discuss the working principle of Hydraulic drives with reference to the accuracy 	and positioning the end effector with a neat sketch?

b)	What do you mean by robot path determination?  Explain.

4. a)	What are the various elements in a robot machine vision?  Explain.

b)	Discuss various sensors used in robotics.

5. a)	Explain different types of Grippers?

b)	Draw a heat sketch of electronics and pneumatic manipulator control circuits and 	explain them.

6.	Explain D-H represent of establishing link co-ordinate system for cylindrical 	manipulator and obtain OA3 matrix.

7.	Describe Euler angle representation of Orientation and obtain Euler angle using 	inverse solution.

8.	Write short notes on any 3 of the following:

a) Robot work cell design		b) Applications of robots

c) Robot programming 		d) Hill climb technique.

###