Code No.: 322103

Set No.

1

III-B.Tech. II-Semester Examination April, 2003

AEROSPACE PROPULSION – II

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Draw the schematic diagram of a simple gas turbine cycle with intercooler and 	heat exchanger and explain briefly the working principle.  Draw also the P-V and 	T-S diagrams of the cycle.

b)	Derive the expression for specific work output and efficiency variation as the 	function of pressure ratio for the above cycle.

2. a)	What is meant by thrust augmentation and explain how is it affected.

b)	Explain the principle of After burner in thrust augmentation.

3. a)	What are the basic construction differences between a turboprop and turbojet 	engine.

b)	A turboprop aircraft is flying at 800km/hr at an altitude where the ambient 	conditions are 0.567 bar and -20oC.  Compressor pressure ratio 8:1.  Maximum 	gas temperature 1100K.  The intake duct efficiency is 0.95 and total head 	isoentropic efficiency of compressor and turbine is 0.92 and 0.95 respectively.  	Calculate the specific power output in KJ/kg, thermal efficiency of the unit taking 	mechanical efficiency of transmission as 0.96 and neglecting the losses other than 	specified.  Assume that exhaust gases leave the aircraft at 800km/h relative to the 	aircraft.

4. 	Air expands under reversible adiabatic conditions in a nozzle from 4 bar and 	725oC to a final pressure of 1bar.  Determine

a)	Type of nozzle required

b)	Critical velocity

c)	Air flow rate if the minimum nozzle diameter is 1.25cm.

5. a)	What is the purpose of a diffuser in the Inlet?  Explain.

b)	Briefly explain subsonic Inlets?

Contd…2

Code No.: 322103					.2.			Set No:1

6.	A ramjet engine operates at M=1.5 at an altitude of 6500m.  The diameter of the 	inlet diffuser at entry is 50cm and the stagnation temperature at the nozzle entry is 	1600K.  The calorific value of the fuel used is 40MJ/kg.  The properties of the 	combustion gases are same as those of air (r=1.4, R=287 J/kgk). The velocity of 	air at the diffuser exit is negligible calculate.

a)	The efficiency of the Ideal cycle.

b)	Flight speed 				c) 	Air flow rate

d)	Diffuser pressure ratio			e)	fuel-air ratio

f)	nozzle Pressure Ratio			g)	Nozzle Jet Mach number

h)	Propulsive efficiency and		i)	thrust

Assume the following values

D = 0.98,  B  = 0.98,  j = 0.96, stagnation pressure loss in the combustion 	chamber is 0.02 PO2 .

7. a)	Describe with the aid of illustrative diagrams two arrangements of solid 	propellant grains employed for restricted and Unresticated-burning Indicate the 	directions of burning and flow of gases.

b)	What are the advantages and Disadvantages of solid propellant rockets.

8.	A rocket nozzle has a throat area of 18cm2 and combustion chamber pressure of 	25 bars.  If the specific impulse is 127.42 seconds and weight flow rate is 44.145 	N/s.

Determine

- The thrust coefficient

- Propellant weight flow coefficient

- Specific propellant consumption and

- Characteristic velocity

###

Code No.: 322103

Set No.

2

III-B.Tech. II-Semester Examination April, 2003

AEROSPACE PROPULSION – II

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Draw the schematic diagram of a simple gas turbine cycle with inter cooler and 	Reheater and explain briefly the working principle.  Draw also P-V and T-S 	diagrams of the cycle.

b)	Derive the expression for specific work output and efficiency variation as the 	function of pressure ratio for the above cycle.

2. a)	With the aid of the schematic diagram and thermodynamic cycle, explain the 	Turbojet engine.

b)	A turbojet engine flying at a speed of 800 km/h consumes air at the rate of 45kg/s.  	Calculate

i) Jet exit velocity, the enthalpy change for the nozzle is 180kJ/kg and velocity 	coefficient is 0.95.

ii) Fuel flow in kg/h and thrust specific fuel consumption assuming the air fuel 	ratio is 80.

iii) Thermal efficiency of plant given h combustion 95% and calorific value of 	fuel used is 42000 kJ/kg.

iv) Propulsive power and Thrust power.

3. a)	What is meant by thrust augmentation and explain how is it affected.

b)	How does water-alcohol mixture Injection help thrust augmentation?  Explain.

4. a)	Explain the terms “over expanding” and  “under expanding” as applied to a fluid 	flow through a nozzle.

b)	2.8m3 of air at 0.14 bar and 20oC is contained in a rigid vessel.  Air from 	atmosphere (1 bar and 20oC) flows into the vessel through a small hole which is 	made in the vessel.  Considering the hole to be a nozzle with effective throat area 	of 0.0625 cm2, calculate the time taken for the pressure inside the vessel to 	increase to 0.28 bar.

5.	Describe the working of a ramjet engine.  Depict the various thermodynamic 	processes occurring in it on h-s diagram.  What is the effect of flight Mach 	number on its efficiency.

Contd…2

Code No. : 322103				.2.				Set No:2

6.	A Jet propelled plane consuming air at the rate of 18.2 kg/s is to fly at Mach number 0.6 at an altitude of 4500m (Pa = 0.55 bar, Ta = 255k).  The diffuser which has a pressure coefficient of 0.9 decreases the flow to a negligible velocity.  The compressor pressure ratio is 5 and maximum temperature in the combustion chamber is 1273K.  After expanding in the turbine, the gases continue to expand in the nozzle to a pressure of 0.69 bar.  The isentropic efficiencies of compressor, turbine and nozzle are 0.81, 0.85 and 0.915 respectively.  The heating value of the fuel is 45870 kJ/kg. Assuming Cp = 1.005 kJ/kgk Cpg = 1.147 kJ/kgk,  rair = 1.48, rgas = 1.33

Calculate

- Power input of the compressor

- Power output of the turbine

- The fuel air ratio

- The thrust provided by the engine and

- The thrust power developed.

7. a)	What is the basic difference between Jet propulsion and Rocket propulsion?  Can 	rocket work in vacuum.

b)	The effective jet exit velocity of a rocket is 8000m/s, the forward flight velocity is 	4000 m/s and the propellant consumption is 161 kg/s calculate.

i) Thrust 	ii) Thrust Horse Power and 	iii) Propulsive efficiency,

8.	Draw a typical layout of a solid propellant rocket and explain its working.  Give 	its applications.

###

Code No.: 322103

Set No.

3

III-B.Tech. II-Semester Examination April, 2003

AEROSPACE PROPULSION – II

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	In what way the gas turbine cycles can be grouped and what are the important 	distinctions between them

b)	The ratio of network to turbine work of an Ideal gas turbine plant is 0.563.  Take 	the inlet temperature to the compressor as 300k.  Calculate the temperature drop 	across the turbine.  If the thermal efficiency of the unit is 35%.  Assume the mass 	flow rate of 10kg/s, cp = 1kJ/kg K and r = 1.4.

2. a)	With the aid of schematic diagram and thermodynamic cycle, explain the turbo 	prop engine.

b)	Mention various advantages and disadvantages of a turbo prop engine and also 	bring out the applications.

3. a)	Explain what do you mean by critical pressure ratio.  When the nozzle is 	subjected to this pressure ratio, outline the steps in a derivation of the maximum 	mass flow through the nozzle.

b)	Air at a pressure and temperature of 60 bar and 27oC expands isentropically in a 	nozzle to a back pressure of 40 bar.  If the area at the throat of the nozzle is 	20mm2, determine the outflow velocity and the rate of flow of air. Neglect the 	entrance velocity.

4. a)	What are the various functions of an exhaust nozzle?

b)	Write short notes on	(i) Exhaust nozzle area ratio	(ii) Thrust Reversing.

5. a)	Explain the principle of Jet propulsion and mention how jet propulsion engines 	are classified.

b)	The following data apply to a turbojet unit of Boeing aircraft flying at altitude of 	6.1 km where the ambient conditions are 0.458 bar and 248K.

Speed of air craft			=	805 km/h

Pressure ratio for compressor		=	4:1

Turbine Inlet temperature		=	0.21 bar

Combustion chamber pressure loss	=	1100K

Intake duct efficiency			=	95%

Isentropic efficiency of compressor	=	0.85

Isentropic efficiency of turbine	=	0.90

Mechanical efficiency of transmission =	99%

Nozzle	efficiency			=	95%

Nozzle outlet area			=	0.0935 m2

L.C.V of fuel				=	43 MJ/kg

Contd…2

Code No. : 322103					.2.			Set No:3

Find the thrust and specific fuel consumption in kg/Nh of thrust.  Assume 	convergent nozzle.  Take cpa = 1.005 kJ/kg k and r= 1.4 cpg 1.147 kJ/kg k and      	r = 1.33.

6. a)	With the aid of a neat diagram, explain the working principle of ramjet engines.

b)	Draw the thermodynamic cycle of the ramjet engine and derive the equation for 	thrust.

7. a)	What is meant by mono propellant and Bipropellant fuels for rocket engines? Give important examples of each and their use.

b)	A rocket flies at 10,080 kmph with an effective exhaust jet velocity of 1400m/s 	and propellant flow rate of 5.0 kg/s.  If the heat of reaction of the propellants is 	6500 kJ/kg of the propellant mixture.

Determine

- Propulsion efficiency and Propulsive Power

- Engine output and thermal efficiency and

- Overall efficiency

8. a)	Draw a schematic diagram of a liquid propellant rocket engine using a pump feed.  	Explain its working.

b)	State advantages and disadvantages of liquid propellant rockets.

###

Code No.: 322103

Set No.

4

III-B.Tech. II-Semester Examination April, 2003

AEROSPACE PROPULSION – II

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	List the various methods of Improving efficiency of a gas turbine power plant and 	discuss them with the help of suitable sketches.

b)	An oil gas turbine installation consists of a compressor, a combustion chamber 	and turbine.  The air taken in at a pressure of 1 bar and temperature of 30oC is 	compressed to 6 bar, with an Isentropic efficiency of 87%.  Heat is added by the 	combustion of fuel in combustion chamber to raise the temperature to 700oC.  The 	efficiency of turbine is 85%.  The calorific value of the oil used is 43.1 MJ/kg.  	Calculate for an air flow of 80 kg/min.

i)	The air/fuel ratio of the turbine gases

ii)	The final temperature of exhaust gases

iii)	The net power of installation and

iv)	The overall thermal efficiency of the installation.

2. 	Define Polytropic efficiency.  Derive suitable expressions for polytropic 	efficiency and bring out the relation between the polytropic efficiency and 	isentropic efficiency.

3. a)	What is meant by thrust?  Derive the thrust equation for a general propulsion 	system.

b)	Explain clearly the various factors affecting the performance of a propulsive 	device.

4. 	Derive the expression relating the critical pressure ratio to index of expansion ‘n’, 	for expansion in a nozzle.

The discharge of gas from a relatively large combustion chamber takes place 	through a convergent divergent nozzle.  The pressure and temperature values in 	the combustion chamber are 3.8 bar and 815oC.  The pressure at the exit from the 	nozzle is the atmospheric pressure 1 bar.  For isentropic expansion through the 	nozzle.

Determine:

- Pressure and temperature of the gas at throat and exit

- Velocities at throat and exit and

- Throat and exit areas if the flow rate of gas is 5.5kg/min.

Index of expansion = 1.35 and gas constant R= 287 J/kgK.

Contd…2

Code No.: 322103					.2.			Set No:4

5. a)	What is the purpose of an aircraft gas turbine inlet and nozzle?

b)	Briefly explain subsonic Inlets?

6.	Describe the working of a scramjet engine.  Write down its main advantages over 	the ramjet.

7. a)	Explain with suitable graphs the performance of a Ramjet engine.

b)	Air enters a turbojet engine at a rate of 12 x 104 kg/h at 15oc and 1.03 bar and is 	compressed adiabatically to 182oC and four times the pressure.  Products of 	combustion enter the turbine at 815oC and leave it at 650oC to enter the nozzle.  	Calculate the isentropic efficiency of the compressor, the exit speed of gases and 	thrust developed when flying at 800 km/h.  Assume the isentropic efficiency of 	turbine is same as that of the compressor and the nozzle efficiency 90%.

8. a)	Discuss the relative applications of the solid and liquid propellant Rocket 	Engines.

b)	A rocket engine has the following data

Propellant flow rate		=	5.0kg/s

Nozzle exit diameter		=	10 cm

Nozzle exit pressure		=	1.02 bar

Ambient pressure		=	1.013 bar

Thrust chamber pressure	=	20 bar

Thrust 				=	7kN

Determine the effective Jet velocity, actual Jet velocity, specific Impulse and 	specific Propellant consumption.

###