Code No: 320304

Set No

1

III-B.Tech. II Semester Examination April, 2003

PRODUCTION TECHNOLOGY – II

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	What are the popular tool designation systems in common use? Explain them 	briefly.

2.a)	Define taper. State the methods of setup used for taper turning. Explain 		any two methods in detail.

b)	How threads are formed on the shaft? Explain the half nut mechanism for thread

cutting.

3.a)	What is the function of a cam in automatic lathe operations?

b)	What are the varieties and types of cams used in automats?

4.a)	Differentiate between Counter boring, Counter sinking and Spot facing?

b)	Explain the procedure for making contour shape work on a planer?

5.	What is jig-boring machine? Sketch and explain the constructional features

and applications of jig boring machine.

6.a)	Define what is cutting speed, feed and depth of cut in milling.

b)	A slot 25 mm deep is to be cut through a workpiece 250 mm long with the help of

H.S.S side and face cutter whose diameter is 150 mm and that has 10 teeth. The

cutting speed is 50m/min and feed is 0.25 mm per tooth. Determine (i) Table feed in

mm/min (ii) Total cutter travel  (iii) Time required to machine the slot.

7.	Explain with simple illustrations.

a)	Reciprocating table type horizontal spindle surface grinding machine.

b)	Rotary table type horizontal spindle surface grinding machine.

8.	Explain in detail:

a) Horizontal broaching machine.

b) Vertical broaching machine.

c) Continuous surface broaching machine.

$$$

Code No: 320304

Set No

2

III-B.Tech. II Semester Examination April, 2003

PRODUCTION TECHNOLOGY – II

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.	Explain with the help of sketches the different systems of specifying tool 	geometry.

2.a)      State and explain in detail machining operations carried out on Lathe machine  	Tool.

b)      How are multiple threads  formed on the job using lathe machine?

3.a)	Describe design of cams for sliding head in automatic lathe machine.

b)	List out the various cutting tool used on capstan and turret lathe machine and

explain important tools in detail.

4.a)	What are the salient features of simple vertical drilling machine?

b)	How is torque and thrust defined in drilling.  What are the factors that influence 	torque 	and thrust in drilling?

5.a)	Explain the importance of jig boring.

b)	Explain the term locating, cutting and checking in jig boring machine.

6.a)	What do you understand by cutting direction of milling cutters and how is it

determined ? What are its effects?

b)	Determine the cutting time required in face milling a workpiece 400 mm long and 	250mm wide with a face mill of 100 mm diameter having 8 teeth. Depth of cut is  	3.0 mm. Cutting speed is 75 m/min and feed is 0.07 mm/tooth.

7.	Explain vertical spindle surface grinding machines.

a)   Double Disc type

b)   Swivel head type

c)   Orbital wheel type.

8.a)	Explain the quick return mechanism for a planner table.

b)	What are the general trouble shooting guides for milling operations?

$$$

Code No: 320304

Set No

3

III-B.Tech. II Semester Examination April, 2003

PRODUCTION TECHNOLOGY – II

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	How is the chip formed in metal cutting? Explain the terms shear plane and 	shear zone.

b)	With the help of suitable diagram, derive the expressions for finding out 	the 	shear velocity and chip flow velocity when cutting velocity, shear 	angle and rake 	angle are given.

2.a)	What are the different tool holding devices? Explain any three in detail.

b)	What is thread chaser? How it is useful in making threads?

3.a)	With a  neat sketch explain the working principle of a collet.

b)	How does bar feeding mechanism work?

4.a)	Name and sketch any four work holding devices used in shaper.

b)	Differentiate between shaping and slotting.

5.a)	Describe design of cams for sliding head in automatic lathe machine.

b)      List out the various cutting tools used on capstan and turret lathe machine and

explain important tools in detail.

6.	A certain component can be milled with either a slab mill of 90 mm diameter, or

a 150 mm diameter face mill. Each cutter has 16 teeth and the cutting speed is 20

m/min. If the feed for the slab mill is 0.15 mm per tooth and for the face mill 0.25 	 	mm per tooth, what is the rate of feed for each cutter?

7.	 Explain in detail:

a) Tool and Cutter grinders

b) Any one type of special purpose grinding machine.

8.a)	Differentiate between Shaping ,Planning and Slotting as regards relative tool   and  	work motion.

b)	State the safety guidelines for working on a lathe.

c)	Explain the follower rest and steady rest used on the lathe.

$$$

Code No: 320304

Set No

4

III-B.Tech. II Semester Examination April, 2003

PRODUCTION TECHNOLOGY – II

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.	What are the common methods of chip breaking and what are the means used for 	the same?

2.a)	What are all the important drives used in a machine tool? Explain.

b)	With a suitable diagram explain the constructional features of a feed gearbox of a 	typical machine tool.

3.a)	Can you mount a part-off tool on the turret? Why not?

b)	What is swivelling tool head? Illustrate an application of this.

4.a)	How is a slotter  specified?

b)	With a neat sketch, explain internal cutting in a slotter.

5.a)	Explain the term cutting speed, feed, and depth of cut with reference to the

drilling operation.

b)	How are the parallel shank tools held on a drilling machine? Explain the

construction and working of a three jaw self centring drill chuck.

6.a)	What are the advantages of helical teeth over straight teeth on cutter for slab milling.

b)	A 250 mm by 40 mm workpiece will be face milled on a 75 mm diameter cutter with 	5 cemented carbide inserts. The operations will take place at a spindle speed of 100 	rpm a feed of 0.2 mm/tooth and depth of cut 2.5 mm. Find the machining time and 	metal removal rate. Also determine the feed per tooth and metal removal rate if the 	desired machining time is 1 min.

7.a)	Explain the term speed feed and depth of cut as applicable to grinding work.

b)	Explain any two methods of grinding thread forms.

8.a)	Explain the quick return mechanism of Ram in a shaper.

b)	Distinguish between shaper and planner.

$$$