Code No.: 320354

OR

# III-B.Tech. II-Semester Examinations April 2003

# operations research

(common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering, Mechatronics, Chemical Engineering.)

## Time: 3 Hours  							Max. Marks: 70

# Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Define operations Research and discuss its characteristics and limitations.

b)	A small scale manufactures provides two types of products A and B which gives a profit margin of Rs 4-00 and Rs 3-00 respectively. There are two plants I and II each having a capacity of 72 and 48 hrs per day. Cycle times of A and B are 2 hrs and 1 hour in plant no. I respectively. Similar figures for plant No. II are 1 and 2 hours. Formulate the L.P problem and solve it.

2.a)	By duality, solve the L.P problem maximize, Z = 30x1 + 23x2 + 29x3

subject to,	6x1 + 5x2 + 3x3  26

4x1 + 2x2 + 5x3  7

x1  0 ; x2  0 ; x3  0.

- Find the initial feasible solution for the following transportation problem.

| Source | Destinations D1 D2 D3 D4 | Total |
| --- | --- | --- |
| O1 O2 O3 Total | 9 7 11 10 10 13 14 8 15 10 17 16 45 20 30 30 | 35 50 40 125 |

3.a)	Give a comparison between assignment problem and routing problem.

b)	Find optimum assignment for the following cost matrix.

| Areas Salesmen | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
| S1 | 11 | 17 | 8 | 16 |
| S2 | 9 | 7 | 12 | 10 |
| S3 | 13 | 16 | 15 | 12 |
| S4 | 14 | 10 | 12 | 11 |

Contd…2

Code No.: 320354				-2-					OR

4.a)	Four jobs are to be processed on two machines with machining times in minutes as given in table.

| Machines | Jobs J1 J2 J3 J4 |
| --- | --- |
| M1 | 25 15 6 17 |
| M2 | 15 20 22 13 |

Prepare a schedule for machining the jobs.

b)	A factory has large number of bulbs, all of which must be in working conditions. Mortality rate is given below.

| Week | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Proposition Which failed | 0.10 | 0.15 | 0.25 | 0.35 | 0.12 | 0.03 |

If a bulb fails in service it costs Rs 3.50 to replace; if all bulbs are replaced at a time it costs Rs 1.20 each. Find optimum replacement policy.

5.a)	What is a game in a game theory? What are the properties of a game?

b)	Solve the following game graphically

A

-6	-1	1	4	4	3

B

7	 -2	6	3         -5	7

6.a)	Show how simulation technique can be used in solving waiting line problem.

b)	Customers arrive at box office window being manned by a single person according to Poisson input process with mean rate of 30 per hour. The time required to serve a customer has an exponential distribution with mean of 90 seconds. Find the average waiting time of the customer.

7.a)	Explain ABC analysis.

b)	A company uses rivets at the rate of 5000 Kg/Year, rivets costing Rs 2.00 per/Kg. It costs Rs. 20.00 to place an order and carrying cost of inventory is 10% per year. How frequently should order for rivets be placed and how much?

8.	Write short notes on any three of the following:

a)	The calculus method of solution for dynamic programming.

b)	Linear programming as a case of dynamic programming.

c)	Group replacement.

d)	Stochastic models.

^^^