Code No.321853

OR

III-B.Tech. II-Semester Examination April, 2003

FOUNDRY TECHNOLOGY

(Metallurgy and Material Technology)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Distinguish between a jobbing foundry and a mass production foundry.

b)	What is the scope of a foundry.

c)	Explain development of Foundry and list down the various types of foundries.

2.	Describe briefly the various types and functions of patterns. Also discuss the factors that have to be considered in the design of a pattern.

3.a)	Give the advantages and limitations of CO2 process of making moulds and cores.

b)	Discuss the above process with respect the following

i)	Theory of hardening

ii)	Gassing procedure

- Time of gassing vs properties developed by the mould

- Temperature vs properties of the hardened mould.

4.a)	Define the term ‘Gating system’.

b)	Briefly explain the various types of gating systems and functions of pouring cups and basins.

c)	Explain under what conditions should the pressurized and unpressurised gating systems be used.

5.a)	What is critical radius of nucleus.

b)	How are you evaluate the above from the thermodynamic back ground.

c)	What is the effect of ‘under cooling’ on critical radius?

d)	Can you expect the aforesaid in the actual practice? Justify.

6.	Distinguish clearly between

a)	Cold chamber and Hot chamber Die casting.

b)	Full mould process and Lost wax process.

c)	Hot boy and cold boy moulds.

7.a)	What do you understand by coke bed? And what is an ideal coke bed height in cupola?

b)	Discuss the effect of coke bed height on cupola operation and explain how the bed height can be ascertained during operation.

8.	Write notes on any four of the following:

a)	Inoculation	(b)  Mealleablizing	(c)  Continuous casting

d)	V-forming full mould process		(e)  Cold and self setting processes.

***     ***     ***