Code No: 320104

Set No

1

III-B.Tech. II Semester Examination April, 2003

ESTIMATING AND CONTRACTS

(Civil Engineering)

Time: 3 Hours							Max. Marks: 80

1)	Answer all questions

2)	Assume suitable data where ever necessary

3)	Data sheet may be used.

---

1.	Prepare a detailed estimate for any three of the following items of work for the 	building plan  given (attached figure) 				(45 Marks)

a) Earth work excavation for foundations

b) R.R. Masonry in CM(1:6) for foundations

c) Sand filling for Basement

d) Plastering in CM(1:5) for walls (inside and out side)

e) Brick Masonry for super structure in CM(1:6)

2.	Calculate the rates of any two of the following items of work for appropriate 	unit.									(20 Marks)

a) Cement concrete (1:5:10) using 40 cu.m broken stone.

b) Coursed Mubble stone Masonry in CM(1:6)

c) RCC roof slab in CC (1:2:4)

Rates of labour and Material at site

Sand 				Rs. 220 per cu.m

Cut stone 			Rs. 150 per cu.m

Cement			Rs. 120 per bag of 50 Kg.

40 mm metal			Rs. 310 per cu.m

20 mm metal			Rs. 420 per cu.m

Steel 				Rs.   18 per Kg.

Mason 	or brick layer 		Rs. 120 per day

Male or Female mazdoor 	Rs. 80 per day

Mixing charges 		Rs. 40 per cu m

Bar bending 			Rs.1.40 per Kg.

Centering and Shultering 	Rs. 220 per cu.m

3.	Answer any three of the following questions				(15 Marks)

a) Write detailed specification for coursed rubble stone masonry in basement with 	     CM (1:5)

b) List the different conditions mentioned in the contract agreement

c) Explain lead and lift charges.

d) Write short notes on (i) Plinth area estimate for building (ii) Revised estimate.

Contd….2.

Code No.320104				:: 2 ::				Set No.1

@@@@@

Code No: 320104

Set No

2

III-B.Tech. II Semester Examination April, 2003

ESTIMATING AND CONTRACTS

(Civil Engineering)

Time: 3 Hours							Max. Marks: 80

1)	Answer ALL questions

2)	Assume suitable data where ever necessary

3)	Data sheet may be used.

---

1.	Prepare a detailed estimate for any three of the following items of work for the 	building plan is given (attached figure) 				(45 Marks)

a) Earth work excavation in foundations

b) C.R. stone Masonry in CM(1:6) for basement and part in foundations

c) Sand filling in Basement

d) Brick Masonry in CM(1:6) for super structure

e) (i) Wood work for doors and windows, (ii) Painting for Doors and Windows.

2.	Calculate the rates of any two of the following items of work for appropriate 	unit.									(20 Marks)

a) Cement concrete (1:4:8) using 40 cum broken stone.

b) Random Rubble stone Masonry in CM(1:6)

c) Plastering with cement mortar in CM (1:5) 20 mm thick.

3.	Answer any three of the following questions				(15 Marks)

a) Write detailed specification for plastering in CM (1:5) 12 mm thick.

b) Distinguish between Running bill and Final bill.

c) Explain briefly about lumpsum contract.

d) Explain over head charges and contingent charges.

Rates of labour and Material at site

Cement			Rs. 130 per bag of 50 Kg.

Sand 				Rs. 200 per cu.m

40 mm metal			Rs. 280 per cu.m

Rough Stones			Rs. 120 per cu.m

Mason 	or brick layer 		Rs. 130 per day

Male or Female mazdoor 	Rs. 80 per day

Mixing charges 		Rs. 40 per cu m

Contd….2.

Code No.320104				:: 2 ::				Set No.2

@@@@@

Code No: 320104

Set No

3

III-B.Tech. II Semester Examination April, 2003

ESTIMATING AND CONTRACTS

(Civil Engineering)

Time: 3 Hours							Max. Marks: 80

1)	Answer ALL questions

2)	Assume suitable data where ever necessary

3)	Data sheet may be used.

---

1.	Prepare a detailed estimate for any three of the following items of work for the 	building plan is given (attached figure) 				(45 Marks)

a) P.C.C. (1:5:10) for foundations

b) C.R. Masonry in CM (1:6) for Basement

c) Plastering in CM(1:5) for walls (inside and out side)

d) R.C.C. (1:2:4) for roof slab and lintels.

e) R.R. Masonry in CM(1:6) for foundations

2.	Calculate the rates of any two of the following items of work for appropriate 	unit.									(20 Marks)

a) Cement concrete (1:3:6) using 40 mm broken stone.

b) Brick Masonry in cement mortar (1:6) using Ist class Bricks.

c) Plastering in CM (1:5) – 12 mm thick.

Rates of labour and Material at site

Cement:			Rs.  120 per bag of 50 Kg.

Sand: 				Rs.  210 per cu.m

40 mm metal:			Rs.  320 per cu.m

Bricks: 			Rs.1000 per 1000 No.

Mason 	or brick layer: 		Rs.  130 per day

Male or Female Mazdoor:	Rs.    80 per day

Mixing charges: 		Rs.    40 per cu.m

3.	Answer any three of the following questions				(15 Marks)

a) Write detailed specification for Brick masonry in CM (1:6)

b) Write down the different types of estimates.  Explain the supplementary 	   	     estimate.

c) Explain briefly Tender document

d) Briefly explain item rate contract.

Contd….2.

Code No.320104				:: 2 ::				Set No.3

@@@@@

Code No: 320104

Set No

4

III-B.Tech. II Semester Examination April, 2003

ESTIMATING AND CONTRACTS

(Civil Engineering)

Time: 3 Hours							Max. Marks: 80

1)	Answer ALL questions

2)	Assume suitable data where ever necessary

3)	Data sheet may be used.

---

1.	Prepare a detailed estimate for any three of the following items of work for the 	building plan  given (attached figure) 				(45 Marks)

a) Earth work excavation for foundations

b) P.C.C. bed (1:5:10) for foundations.

c) R.R. Masonry in CM(1:6) for foundations

d) Brick Masonry in CM(1:6) for super structure

e) White washing inside and out side for walls and parapet wall.

2.	Calculate the rates of any two of the following items of work for appropriate 	unit.									(20 Marks)

a) Cement concrete (1:6:12) using 40 mm broken stone.

b) Plastering in CM (1:5) 20 mm thick.

c) Brick Masonry in Cement Mortor (1:8) using Ist class bricks.

Rates of labour and Material at site

Cement			Rs. 140 per bag of 50 Kg.

Sand 				Rs. 180 per cu.m

40 mm metal			Rs. 290 per cu.m

Bricks 				Rs.1200 per 1000 Nos.

Mason 	or brick layer 		Rs. 120 per day

Male or Female Mazdoor 	Rs. 70 per day

Mixing charges 		Rs. 30 per cu.m

3.	Answer any three of the following questions				(15 Marks)

a) Write detailed specification for plastering in CM (1:6) 20 mm thick

b) Write short notes on (i) Measurement Book  (ii) Standard Schedule of rates

c) List out the types of contracts in the contract system.

d) List the approximate estimates and explain briefly any one of the approximate 	  	    estimate.

Contd….2.

Code No.320104				:: 2 ::				Set No.4

@@@@@