Code No.: 320251

OR

III-B.Tech II-Semester Examination April, 2003

POWER SYSTEMS – II

(Electrical and Electronics Engineering)

Time: 3 hours							Max. Marks: 70

Answer any five questions

All questions carry equal marks

---

1.a)	Derive the formula for inductance of a 3Q 3wire unsymmetrical transmission line 	assuring transposition.

b)	A single phase double circuit line is shown in figure conductors AA'form one

path and BB' form the return path which are connected in parallel.  Calculate the 	inductance of each per km assuring the current to be equally divided.  Diagram of

each conductor is 8mm.

2.a)	What do you understand by surfe impedance of a line?  What is its physical 	significance.

b)	Derive A,B,C,D constants of a medium line using nominal ‘Π  and nominal ‘Т’ 	approximations.

3.a)	Derive the expression for capacitance of a 3-phase line with equilateral spacing.

b)	A 3phase 220kV, 50 Hz transmission line consists of 1.5cm radius conductors

spaced 2mts apart in equilateral triangular formation.  If the temperature is 40o c

and the atmospheric pressure is 76cm, calculate corona power loss per km of the

line.  Take mo=0.85.

Contd…2

Code No.: 320251         			-2-				OR

4.a)	A string of 6 insulator unit has a self capacitance equal to 10 times the pin-to-	earth capacitance.  Find the string efficiency?

- Explain stringing charts and sag templates.

5.a)	Derive expression for electric stress in a single core cable.

b)	A singe core lead sheathed cable is graded by using 2 dielectrics of relative 	permittivity 3.6(inner) and 2.5 (outer).  The thickness of each being 1cm and core

dia.1cm, determine the maximum stress in the 2 dielectrics. System operating 	voltage 66kv.

6.	An overhead line at river crossing is supported from two towers 20m and 14m

above water level. The distance between  the towers being 250m, the conductors

used are 7 / 0.345 cu. each of dia. 1.035cm;  Find the minimum clearance 	between the line conductors and the water surface.  Also find the sag in the above 	line if the supports are at the same level.

Weight of the conductor = 590 kg/km

Breaking load		    =   2662 kg.

Factor of safety	    =          2

7.a)	What are the methods employed to control voltage in a power system and explain 	any one of the methods in detail.

b)	Determine the symmetrical components of the three currents Ia=10   0o,

Ib=  10    230o  and Ic=10  130o   amps.

8.	Write short notes on any TWO of the following :

a)  Positive, negative and zero sequence impedances.

b)  Tap changing transformers.

c)  Skin effect.

d)   GMD and GMR concepts.

-----