Code No.310101

Set No.

1

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Differentiate steady uniform flow and unsteady non uniform flow.

b)	State Chezy’s equation and Kutter’s (G.K.) equation for uniform flow and give the ranges of the Chezy’s constant and Kutter’s n for different conditions.

c)	A trapezoidal channel with a bed width of 4m and side slopes of 1½ :1 is laid on a slope of 1 in 2000.  Determine the discharge when the depth of flow is 1.5m and Manning’s n is 0.015 for the material of the channel section.

2.a)	Sketch the specific energy diagram for a constant discharge in an open channel and indicate the salient  points on it.

b)	Obtain the relationship between critical depth and the alternate depths in open channel flow.

c)	A trapezoidal channel having a bed width of 1.0m and side slopes of 1½ hor:1 ver is laid on a slope of 0.0016.  If it carries a uniform flow at a rate of 10 m3/s, compute the normal depth and velocity given the Manning’s ‘n’ to be 0.025.

3.a)	What is a hydraulic jump?  Obtain the expression for the ratio of the jump heights after the jump and before the jump on a horizontal rectangular channel.

b)	A rectangular channel 4m wide with a bed slope of 0.0009 carries a discharge of 16m3/s at a depth of 2m at a section.  What is the type of water surface profile.  At what distance upstream or down stream will the depth be 2.2m.  Assume Manning’s n to be 0.01.

4.a)	Explain the hydraulic similitude with reference to viscous flow problems and give the scale ratios for discharge, torque and power between the model and the proto type.

b)	Show that the velocity V through an orifice can be expressed in the form

V=2gh.φdh,μρVh,σρV2h Where h is the head,  is the mass density d is the diameter of the orifice, is the viscosity g is the acceleration due to gravity,  is the surface tension and  represents a function.

Contd….2.

Code No.310101				:: 2 ::				Set No.1

5.a)	Show that the maximum efficiency of a fire jet striking normally a series of flat plates mounted on the periphery of a wheel is 50%.

b)	A jet of water 5 cm, in diameter moving with a velocity of 20 m/s strikes a flat vane, which is normal to the axis of the jet stream.  Find the force exerted by the jet (i) if the vane moves with a velocity of 10 m/s (ii) if instead of a single vane, a series of vanes so arranged that each vane appears successively before the jet in the same position and moving with a velocity of 10m/s.

6.a)	Give the classification of hydraulic turbines and mention the factors affecting the selection of these turbines.

b)	A Francis turbine with an overall efficiency of 80% is required to develop 500 kW of power.  It is working under a head of 250 m.  The speed ratio and flow ratio may be taken as 0.3 and 0.96 respectively. The wheel runs at 300 rpm.  The hydraulic losses may be taken as 20%.  Assuming radial discharge, determine      (i) the guide blade angle (ii) the vane angle at the inlet (iii) the diameter of the wheel and (iv) the width of the wheel at the inlet.

7.a)	With a neat sketch, explain the governing of pelton wheel.

b)	A model turbine built to a scale of ¼ is tested under a head of 10m.  The prototype turbine is required to work under a head of 30m and run at 350 rpm.  At what speed must the model be run, if it develops 50 kW and uses a discharge of 0.96 m3/s.  What Power can be expected from the prototype.  Also mention the type of the turbine.

8.a)	Give a neat sketch of a centrifugal pump and indicate the parts.

b)	A centrifugal pump is installed 4 m above the water level in the sump to deliver water against a static delivery head of 40m.  The suction and delivery pipes are both 7.5 cm in diameter.  The equivalent length of the suction pipe is 6.0m while that of the delivery is 50m.  The friction coefficient for the pipe, ‘4f’ = 0.012.  The impeller is 30cm in diameter and 3 cm wide at the exit. It runs at 1400 rpm. The blade angle at the exit is 450. Monometric efficiency of the pump is 75% and overall efficiency is 60%.  Calculate the discharge through the pump and the power required to drive it.

@@@@@

Code No.310101

Set No.

2

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Derive the condition for the most economical section of a trapezoidal channel.  Obtain the expression for the hydraulic radius for such a channel.

b)	A trapezoidal channel carrying a discharge of 20 m3/s is laid in non erodible bed on a slope of 1 in 4000.  If the Manning’s n is 0.017, obtain the dimensions of the channel section taking the side slope to be ½ :1.

2.a)	Differentiate normal depth, alternate depth and critical depth in open channel flow.

b)	A rectangular channel 3m wide carries a uniform flow of 6m3/s. Plot the specific energy curve and determine (i) the minimum specific energy and critical depth.    (ii) the alternate depths at a specific energy of 2.50 N-m/N and (iii) the bed slope required to maintain a uniform flow at a depth of 0.6m given Manning’s n to be 0.02.

3.a)	Derive the dynamic equation for gradually varied flow in an open channel.

b)	A river 50m wide 3 m deep has stable bed and vertical banks with the water surface slope of 1 in 3000.  Estimate the length of back water curve produced by an afflux of 1.5m given the Manning’s n to be 0.030.  Use two steps.

4.a)	State Buckingham  theorem.

b)	Write the dimension of (i) Angular velocity (ii) Momentum and (iii) Power.

c)	Assuming that the discharge, Q of a centrifugal pump is dependent on the mass density, P., pump speed in rpm, N; the diameter of the impeller, d; the pressure, p; and the viscosity,  of the fluid, show that the discharge can be expressed by Q=Nd3.φgHN2d2,υNd2  Where H is the head and  is the Kinematic viscosity of the fluid.

Contd….2.

Code No.310101				:: 2 ::				Set No.2

5.a)	Obtain the expression for the torque exerted on a wheel with radial curved vanes and the efficiency of the vanes.

b)	A jet of water having a velocity of 40m/s impinges without shock a series of vanes moving at 15 m/s, the direction of motion of the vanes being inclined at 15o to that of the jet.  The relative velocity at the outlet is 0.9 times that at inlet and the absolute velocity of the water at the exit is to be normal to the motion of the vanes.  Find (i) vane angles at the inlet and exit, (ii) the work done on the vanes per unit weight of the water supplied by the jet and (iii) the hydraulic efficiency.

6.a)	Give a neat sketch of a Francis turbine and indicate the parts.

b)	A Pelton wheel has to be designed for the following data.  Power to be developed is 10 M.W., Net head available is 300m, speed = 500 rpm, Ratio of jet diameter to wheel diameter is 1/10 and overall efficiency is 85%.  Find the number of jets, diameter of the jet, diameter of the wheel and the rate of flow required.  Assume Cd = 0.98 and speed ratio = 0.46.

7.a)	Define unit speed, unit power, unit discharge and specific speed.

b)	Derive the expression for specific speed and state the assumptions made.  A model having a scale of 1/10 is constructed to determine the best design of a Kaplan turbine to develop 12 K.W. of power under a net head of 9m at a speed of 300 rpm.  If the head available at the laboratory is 6m and the model efficiency is 85%.  Find (i) the running speed of the model, (ii) the flow required in model and the power developed by the model.  Also find the specific speed.

8.a)	Obtain an expression for the minimum starting speed of a centrifugal pump given the ratio of the diameters at the inlet to the exit of the vanes to be 0.5.

b)	A 3 stage centrifugal pump has an impeller 40 cm in diameter and 2 cm wide.  The blade angle at the outlet is 45o and the area covered by the vane thickness may be assumed to be 8% of the outlet area.  If the pump delivers 60 liters/s of water when running at 920 rpm, calculate (i) the power of the pump (ii) the manometric head and (iii) the specific speed.  Assume the manometric efficiency to be 78% and mechanical efficiency to be 90%.

@@@@@

Code No.310101

Set No.

3

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	What are the different types of flows in open channels?  Explain in detail.

b)	A trapezoidal channel is 10.0m wide and has a side slope of 1.5 H and 1V.  The bed slope is 0.0003.  The channel is lined with smooth concrete of n = 0.012.  Compute the mean velocity and discharge for a depth of flow of 3.0m.

2.a)	How do you classify channel bottom slopes?

b)	A rectangular channel 15m wide carries a discharge of 30m3/Sec.  The slope of channel is 0.0002.  If the depth of flow is 1.7m, how far from this section (up stream or down stream) will the depth be 2.3m.  Assume Manning’s n = 0.013.

3.a)	Explain i) Geometric similarity, ii) Kinematic similarity, iii) Dynamic similarity.

b)	List out the variables that influence the pressure drop over a distance in a pipe line.  Using Buckingham theorem, find the dimensionless expression for pressure drop.  From this obtain Darcy Weisbach equation.

4.a)	What is a velocity triangle?  Explain the different sides and angles of a velocity triangle.  Draw the velocity triangle at inlet of a pelton wheel turbine.

b)	A 7.5 cm diameter jet having a velocity of 30m/sec strikes a flat plate, the normal of which is inclined at 45o to the axis of the jet.  Find the normal force on the plate (i) when the plate is stationary (ii) when the plate is moving with a velocity of 15m/sec in the direction of the jet and away from the jet.

5.a)	How are the turbines classified?

b)	A plelton wheel has a diameter of 0.8m and runs at 1000 rpm.  The net head is 400m.  If the side clearance angle is 15o and discharge through the nozzle is 150 litres/sec, determine. (i) The power available at nozzle (ii) Hydraulic efficiency.

6.a)	Explain (i) Unit speed (ii) Unit power (iii) Unit discharge.  Derive the equation for unit speed.

b)	A Francis turbine develops 100 hp working under a head of 5m and at a speed of 200 rpm.  The discharge is 1.8m3/sec.  The runner diameter is 1 m.  If the head on this turbine is increased to 16m, determine its new speed, power and discharge.

Contd….2.

Code No.310101				:: 2 ::				Set No.3

7.a)	What is priming of a centrifugal pump?  Explain.

b)	The diameters of an impellor of a centrifugal pump at inlet and outlet are 20cm and 40cm respectively.  Determine the minimum starting speed if it works against a head of 25m.

8.	Write short notes on the following:

a) Cavitation in Centrifugal pump, b) Draft tube, c) Characteristic curves             d) Hydro Power Installation.

@@@@@

Code No.310101

Set No.

4

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

HYDRAULICS AND HYDRAULIC MACHINERY

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain Energy and Momentum correction factors.

b)	Determine the most efficient section of a trapezoidal channel with side slope 1V and 2H.  The channel carries a discharge of 11.25m3/sec with a velocity of 0.75 m/sec.  What is the bed slope of the channel.  Take Manning’s n = 0.025.

2.a)	Explain i) Sub critical flow and  ii) Super critical flow in detail.

b)	A rectangular channel 10m wide carries a discharge of 30m3/sec.  It is laid at a slope of 0.0001.  If at a section in this channel the depth is 1.6m, how far (upstream or down stream) from this section will the depth be 2.0m?  Take Manning’s n as 0.015.

3.a)	Explain Buckingham’s  Theorem.

b)	The pressure drop “∆P” in a pipe of diameter D and length l depends on mass density  and viscosity  of the following fluid, mean velocity of flow V and average height ‘K’ of roughness projections on the pipe surface.  Obtain a dimensionless expression for ∆P.  Hence show that hf=fLV22gD.

4.a)	Explain Momentum Equation.

b)	A plane surface made with metal of unit weight 7700 kg/m3, is hinged at the top.  It has a thickness of 1cm and side area of 20cm x 20cm.  A horizontal jet of water of 2cm diameter impinges on the plate 5cm below the hinge.  The velocity of jet is 9.81m/sec.  Find the angle of swing of the plate.

5.a)	What is the use of a draft tube?  Bring out the differences between reaction turbines and impulse turbines.

b)	A turbine is to operate under a head of 25m at 200 rpm.  The discharge is 9.0m3/sec.  If the efficiency is 90% determine.

(i) Specific speed (ii) Power generated (iii) Type of turbine.

6.a)	What is “specific speed” of a turbine?  Derive the equation.

b)	A hydraulic turbine has an output of 9000 hp when it works under a head of 25m and at a speed of 100 rpm.  What would be its speed and power when it works under a head of 16m.

Contd….2.

Code No.310101				:: 2 ::				Set No.4

7.a)	Explain (i) Manometric head  (ii) Static head (iii) Total head.

b)	The diameters of an impellor of a centrifugal pump at inlet and out let are 20 cm and 40 cm respectively.  Determine the minimum starting speed if it works against a head of 25 m.

8.	Write short notes on the following:

a) Priming of a centrifugal pump

b) NPSH

c) Cavitation

d) Hydro power Installation.

@@@@@