Code No: 310103		FOR 2000 BATCH

Set No.

1

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Note: 1. Answer any ONE question from PART-A and Any THREE   questions from PART-B

- Assume suitable data wherever is necessary

- Uses of IS456-2000 and ONLY Design charts of SP-16 are allowes.

- - -

PART- A (32 MARKS)

1.	Using limit state method design a T-beam of 12.6m span of a live load of 4.0KPa over its 3.5m wide flange. The flange is 100mm thick excluding 50mm thick finishing and flooring.  Design the beam for flexure and shear assuming the material to be used as M20 concrete and Fe415 steel.

Also, Draw the following to a scale:

- Longitudinal section of the beam showing reinforcement details.

- The cross-section of the beam at mid-span showing the reinforcement details.

2.	Design a circular footing for the following data.

(i)	Load transmitted from the column to footing  = 1200kN.

(ii)	The diameter of the column  =  500mm

(iii)	S.B.C of soil	= 	200kN/m2

- Materials used = M20 grade concrete and Fe415 grade steel.

Use limit state method.  Also draw the following to a scale:

- Plan of the footing showing reinforcement details.

- Sectional elevation of the column and the footing showing the reinforcement details.

PART-B (3 X 16 = 48 MARKS)

3.	An R.C. beam has the following data:

(i)	Cross-section = 200mm x 400mm effective depth.

(ii)	Compression reinforcement = 4bars of 16mm 

- Tension reinforcement  = 4 bars of 20mm .

- Materials Used = M20 grade concrete and Fe415 grade steel.

Find the Moment of resistance of the section.

4.	A R.C.C. beam of cross-sectional dimensions 350mm x 625mm depth has 4 numbers of 28mm dia bars.  Design the shear reinforcement for a factored shear force of 350kN.  Use M20 grade of concrete and Fe-415 grade of steel.  Show the reinforcement details.

Contd…2

Code No. 310103				.2.				Set No. 1

5.	Design a reinforced concrete column for an axial load of 3000kN.  Use M20 grade concrete and Fe415 grade steel.  Take effective height of the column as 5m.

6.	A rectangular slab 4.5m x 6m is simply supported at the ends.  Design the slab to carry a superimposed service load of 5kN/m2.  Use M20 grade concrete and Fe415 grade steel.  Assume that the corners of the slab are prevented from lifting.

7.	A rectangular cantilever beam has the following data.

(i) Cross-section	=	35cm X 65cm

(ii) Span		=	4.0m

(iii) Bending moment at the support = 150kN.m out of which 30% moment is due to permanent loads.

(iv) Tension Reinforcement = 4 X 20mm  bars.

- Effective cover = 40mm.

- Material used = M20 concrete and Fe415 steel.

Calculate the short-term deflection at the free end and deflection due to shrinkage of concrete.

###

Code No: 310103		FOR 2000 BATCH

Set No.

2

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Note: 1. Answer any ONE question from PART-A and Any THREE   questions from PART-B

2. Assume suitable data wherever is necessary

3. Uses of IS456-2000 and ONLY Design charts of SP-16 are allowes.

- - -

PART- A (32 MARKS)

1.	Design a simply supported T-beam of span 8m and spaced at 3m centre to centre.  The thickness of slab is 120mm and total load including self weight of beam is 28kN/m.  The materials are M20 grade of concrete and Fe415 grade of steel.  Design the beam for flexure and shear using curtailment rules.  Also draw the following to a scale:

a)	Longitudinal section of the beam showing reinforcement details.

b)	The cross-section of the beam at mid-span showing the reinforcement details.

2.	Design a rectangular isolated sloped footing for a column of size 250mm X 750mm carrying an axial load of 1800kN and reinforced with 8 number 25mm diameter bars.  The safe bearing capacity of soil is 200kN/m2.  The materials are of M20 grade concrete and Fe415 grade steel.

a)	Plan of the column and Footing showing the reinforcement details.

b)	Sectional elevation of the column and Footing showing the reinforcement details.

PART – B (3 X 16 = 48 MARKS)

3. a)	Describe various failure modes of a concrete beam.

b)	Sketch the stress-strain relationship for concrete and steel.

c)	A singly reinforced rectangular beam is subjected to a bending moment of 50kN.m at working loads.  Design the beam for flexure.  The materials are M20 grade concrete and mild steel reinforcement of grade Fe 250.  Use limit state method.

4.	A reinforced beam is simply supported over a span of 5m and carries a UDL of 20kN/m including its self weight.  If the cross-section of the beam is restricted to 275mm X 575mm, determine the areas of tension and compression steel.  Give M20 grade of concrete and Fe415 grade of steel.

Contd…2

Code No. 310103				.2.				Set No. 2

5.	A rectangular beam of size 250mm X 600mm overall depth is subjected to a factored sagging bending moment of 48kN.m; factored shear force of 48kN and a factored torsional moment of 18kN.m.  Design the reinforcement at the section.  The materials are M20 grade concrete and Mild steel reinforcement.

6.	Design a short circular column to carry a load of 1200kN.  The materials are M20 grade concrete and Fe415 grade of steel.  Draw a sketch showing the reinforcement details.  (Take SBC of soil 180kN/m2).

7.	Design a slab over a room 5m X 7m as per IS code.  The slab is supported on masonry walls all round with adequate restraint and the corners are held down.  The live load on the slab is 2000N/m2.  This slab has a bearing of 150mm on the supporting walls.  Use M25 concrete and Fe415 grade steel.

###

Code No: 310103		FOR 2000 BATCH

Set No.

3

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Note: 1. Answer any ONE question from PART-A and Any THREE   questions from PART-B

2. Assume suitable data wherever  necessary

3. Uses of IS456-2000 and ONLY Design charts of SP-16 are allowes.

- - -

PART- A (32 MARKS)

1.	A square column 500m m X 500mm carries an axial load of 1350kN.  Design the column and a square footing for the column.  The safe bearing capacity of the soil is 200kN/m2.  Use M20 grade of concrete and Fe 415 grade of steel.  Design by limit state method and also draw the following to a scale.

a)   Plan of the Column and footing showing the reinforcement details.

b)   Sectional elevation of the column and footing showing the reinforcement details.

2.	A R.C.C. slab has to be provided over a room 4metreX5 metre.  The slab is continuous over two adjacent supports and discontinuous at the other two supports.  The live load on the slab is 3500 N/m2.  Design the slab.  Use M20 concrete and Fe 415 grade of steel. Draw the following to the scale.

a)   Sectional elevation showing the reinforcement details.

b)   Plans showing top and bottom reinforcement details of the slab.

PART-B (3X16=48MARKS)

3.	A T-beam has the following data:

Thickness of Flange    =   100mm

Effective depth	=    600mm

Bending moment	=    200kN.m

Effective Flange width =   1200mm

Thickness of rib	=     300mm

If M20 grade concrete and Fe 415 grade steel is used as material, find the moment of resistance by working stress method.

4.	Design a doubly reinforced beam for the following data:

(i)  Span	=     5.5m,   Simply supported

(ii) Load	=     135 kN/m  375mm (UDL)

(iii)Cross-section    = 240mm deep overall.

(iv) Materials used  = fck  = 20N/mm2  and

fy  =  415 N/mm2

Contd…2

Code No. 310103				.2.				Set No. 3

5.	A cantilever beam has the following data:

a)  	Span = 3m

b)  	Cross-section = 30 cm X 60cm

c)	B.M at the support section = 140 kN.m out of which 50% moment is due       to permanent loads

d) 	Reinforcement   = 4 Nos. – 20mm bars in tension zone.

e)  	Concrete cover    =  40mm

f) 	Materials used = M2O concrete and Fe 415 steel.  Find the short term deflection of the beam and deflection due to shrinkage of concrete.

6.	A reinforced concrete beam section of size 350mm X 400mm effective is subjected to a factored torsion of 6 kNm in addition to a factored moment of

45 kNm and a factored shear force of 80 KN.  Design the torsion reinforcement using M20 grade concrete and Fe415 grade steel.

7.	Write short notes about the following:

a)  Uniaxial and biaxial bending concepts of columns.

b)  Advantages of Limit State Design over Elastic design.

c)  Concepts of bond, anchorage and Development length.

&&&&&

Code No: 310103		FOR 2000 BATCH

Set No.

4

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERING-I (RCC) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Note: 1. Answer any ONE question from PART-A and Any THREE   questions from PART-B

2. Assume suitable data wherever is necessary

3. Uses of IS456-2000 and ONLY Design charts of SP-16 are allowes.

- - -

PART- A (32 MARKS)

1.	A T- beam floor consists of 130 mm thick R.C.C slab monolithic with 250mm wide beams. The floor of the hall 12 meter by 6 meter to the centers of the supporting walls, consists of 3 beams spaced at 3 meters apart. Design an intermediate beam for flexure and shear using M20 grade concrete and Fe 415 grade of steel. Allow a live load of 3500 N/m2 and a dead load of the floor finish may be taken as 500 N/m2. Use limit state method. Draw to a scale:

- Longitudinal section of the beam showing reinforcement details.

- The cross section of the beam at mid-span showing the reinforcement details.

2.	A square column 450mm X 450mm carries an axial load of 1200 kN. Design the column and a  square footing for the column. The safe bearing capacity of the soil is 195 kN/m2. Use M20 grade of concrete and Fe415 grade of steel. Design by limit state method and also draw the following to a scale:

- Plan of the column and footing showing the reinforcement details.

- Sectional elevation of the column and footing showing the reinforcement details.

PART-B ( 3 X16 = 48 MARKS)

3.a)	Explain : Balanced, Under reinforced and Over reinforced sections.

b)	Under what conditions doubly reinforced beams are preferred? Explain.

c)	Explain briefly about stress-strain relationship for concrete and steel.

4.	Determine the maximum permissible live load on a T-beam of 800mm overall depth, 230mm thick web, 2.4m wide effective flange of 100mm thickness, and reinforced with five bars of 20mm diameter. Assume an effective span of 12m. Use M20 grade concrete and Fe 415 grade steel.

5.	An Isolated circular footing is carrying a load of 1000kN transmitted by the column. The column is 450mm in diameter. The safe bearing capacity of the soil is 180kN/m2. Adopt M20 grade Concrete and Fe 415 grade steel.

Contd…2

Code No. 310103				.2.				Set No. 4

6.	Design a simply supported slab to cover a room with internal dimensions 4mX5m and 230mm thick brick walls all around. Assume a live load of 3kN/m2 and a finish load of 1kN/m2. Use M20 grade Concrete and Fe 415 grade steel. Assume that the corners of the slab are prevented from lifting.

7.	Determine the safe load carrying capacity of a column of section 750mmX350mm and reinforced with 12 bars of 20mm diameter. Design suitable transverse ties for the column. Assume M20 concrete and Fe 415 steel.

^^^