Code No.310104

Set No.

1

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

ENVIRONMENTAL ENGINEERING

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	What is Percapita demand? How to determine it?  What factors affect it?

b)	Enlist the different methods of population forecast and state for which type of growing city Geometrical increase method is useful.  By using incremental increase method of population forecast findout the probable population of a town in 2020AD for the data given below:

Year:				1960	1970	1980	1990	2000

Population in thousands:	  26	  29	  35	  43	  47

2.a)	What are the various types of pressure pipes that can be used for water supply scheme?  Give the merits and demerits of the same.

b)	Estimate the hydraulic gradient in a 2m diameter smooth concrete pipe carrying a discharge of 3cumecs by using.

i) Manning’s formula	 ii) Hazen –William’s formula

Assume (i) n = 0.014	(ii) CH = 130.

3.a)	Sketch and describe a medium size river intake.

b)	List the different types of valves provided on water supply pipe lines.  Describe working of any one with a neat sketch.

4.a)	Enumerate the different physical, chemical and biological tests conducted on water sample to decide the suitability for drinking.

b)	Design a coagulation cum sedimentation tank for a town population 80,000 and the average percapita consumption is 135lpcd.  Assume any other additional data necessary clearly stating the same.

5.a)	Describe different systems of sewerage giving the merits and demerits of each.  Also state the situations in which each system can be suitably used.

b)	Describe the laying of a sewer line in a trench.

6a)	What is B.O.D.?  Deduce an expression for B.O.D. with time.  What are the factors on which the deoxygenation constant (k) depends?

b)	Estimate 5-day B.O.D. of sewage sample of 27oC, if its 1-day B.O.D. at 40oC is 310mg/lit.  Assume K at 20oC as 0.1/day.

7.a)	Distinguish between standard rate and high rate trickling filters.

b)	Under what circumstance Septic tanks are most suitable. With the help of neat sketch, describe the construction and operation of a septic tank.

8.a)	Clearly bring out the comparison between activated sludge process and trickling filters.

b)	Write a detailed note on disposal of waste water on to land for irrigation.

@@@@@

Code No.310104

Set No.

2

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

ENVIRONMENTAL ENGINEERING

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Bring out the significance of protected water supply with special reference to water-borne diseases.

b)	Estimate the future population of a community in the year 2031, from the following data by geometrical increase method:

Year:		1971		1981		1991		2001

Population:	11,093		13,751		15,206		19,723

2.a)	Present graphically the variation in daily demand for water.

b)	Explain clearly how you take into account the effect of variation in consumption in the design of various units of water supply system.

3.a)	Explain the working of a rapid sand filter with a sketch.

b)	Design a set of slow sand filters to treat 2 million liters per day of water.  Give the dimensioned sketch and explain its working.

4.a)	Explain the significance of conducting the water quality analysis for the following water quality parameters.  (i) Turbidity	(ii) Chlorides.

b)	Discuss with the help of diagrams, various methods of laying out the distribution system.

5.a)	What special considerations are needed while designing sewer pipes as compared to water supply pipes?  Why is self-cleansing velocity required to be taken into account in sewer designs?

b)	Design a circular sewer with the following data: i) Population: 45000;

ii) Water supply rate: 140 lit/capital/day;

iii) Max.  available slope for laying:1 in 500;

iv) The design to be done for 3 times D.W.F.

6.a)	State and describe four important tests that may be carried out to know the characteristics of sanitary sewage.

b)	Data from an unseeded domestic waste-water BOD test are: 5-0ml of waste in a 300ml bottle, initial D.O. of 7.8 mg/l and 5-day D.O. equal to 4.3 mg/l compute: (i) The BOD and (ii) The ultimate BOD, assuming a K-rate of 0.10/day.

7.a)	Draw a neat sketch to represent a flow diagram of a typical sewage treatment plant and explain the function of each component.

b)	How sedimentation of sewage is done?  Explain in brief the theory of sedimentation. Explain the applications of stroke’s law for sewage sedimentation.

8.a)	Differentiate between standard rate and high rate trickling filter.

b)	Design an oxidation pond for a colony of 4000 population. The sewage flow is

180lpcd. The 5-day BOD is 300mg/lit.Assume any other suitable data required

@@@@@

Code No.310104

Set No.

3

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

ENVIRONMENTAL ENGINEERING

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the various factors that influence the percapita consumption of water.

b)	Mention the water borne diseases and their causative organisms.  How do you ensure their absence in a laboratory sample of water?

2.a)	Explain the river intake with a neat sketch.  Also explain the various factors to be considered in its location.

b)	Explain the functions and location of scour value.

3.a)	Explain the Hardy-cross method and derive the equation for corrected discharge using the method of balancing heads by correcting assumed flows.

b)	Design a rectangular sedimentation tank to treat 3x106 liters of raw water/day.  The detention period may be assumed to be 3.0 hours and depth as 3.0m compare the economical aspect with that of a circular tank for the same capacity.

4.a)	Compare the design and working features of the slow sand filter and rapid sand gravity filter.

b)	A treatment plant treating 1,00,000 m3 of water per day recorded a residual chlorine of 0.2 mg/l after a contact period of 15 minutes.  Calculate the chlorine dosage if 200 kg of bleaching powder containing 30% chlorine is consumed per day.

5.a)	List the common tests conducted on a sample of sewage to determine the mode of treatment in the laboratory.

b)	With the help of a neat sketch indicate the location and explain the salient features of different types of sewers used in a sewerage system.

6.a)	Define B.O.D.  Derive a mathematical formulation to assess first stage BOD.

b)	The 5-day BOD of a waste water is 250 mg/l and the reaction rate constant is 0.172/day.  Find the ultimate BOD of the waste.

7.a)	Describe the principles of working of activated sludge process with a neat flow diagram.

b)	Design an oxidation pond for the treatment of sewage for an industrial town of population 3000 and the sewage flow is 125 l/c/day.  Standard 5 day BOD of the sewage is 350 mg/l.  Depth of tank may be taken as 1.0m and assume BOD loading as 250 kg/day/hectare.

8.a)	What are the various methods of disposal of sewage and describe any one of them.

b)	Design a septic tank for 30 users.  Make suitable assumptions.

@@@@@

Code No.310104

Set No.

4

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

ENVIRONMENTAL ENGINEERING

(Civil Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain the fire demand.

b)	Write in detail about design period, forecast of population and demand while designing water supply schemes.

2.a)	Explain the following with the help of a neat sketch

(i) Expansion joint (ii) Flexible joint.

b)	Water is to be supplied to a town with population 1-5 lakhs at the rate of 120 lpcd from a river 1800m away.  The maximum static head is 32m and duration of pumping is 8 hours a day.  Assume maximum demand 1.5times average demand.  Coefficient of friction for the rising main = 0.0075 and velocity in the rising main = 2.4m/s. Determine the size of the rising main and B.H.P. of the pumps required.

3.a)	Write ISI standards for Domestic water quality.

b)	Give a typical layout plan of water treatment works. Name the different units.  Give the important points to be kept in mind while planning the layout.

4.a)	Write a detailed note on troubles in operation of rapid sand filter.

b)	Calculate the area of filtering media required for treating water by means of rapid sand filters giving the data:  Population: 80,000 Rate of supply: 200lpcd

Maximum demand: 1.5 times the average.  Assume suitable rate of filtration.

5.a)	Distinguish between conservancy and water carriage system.

b)	Assuming that the surface on which rainfalls in a thickly built up residential district is as follows: 40% of the area consists of roofs and pavements with   C1 = 0.80 and 60% of the area consists of lawns and gardens with C2 = 0.2, calculate the coefficient of run off.  If the total area of the district is 2.5 hectares and the maximum intensity of rain is 40mm/hour, what is the total run off of the district.

6.a)	Give the B.O.D. equation and explain all the terms in the equation.

b)	A 25cm dia sewer is flowing full.  It was laid to a slope of 1 in 150.  Calculate the velocity of flow and discharge in the sewer, assuming n = 0.013.

7.a)	Explain the function of a soak pit with a neat sketch.

b)	Design a septic tank for 70 users of a hostel assume the necessary data.  (The daily water supply rate is 140l/c/day).

8.a)	Explain the principle of Activated sludge process with a neat flow diagram.

b)	Design  a single stage Trickling Filter to treat 10,000 m3 of sewage per day having a BOD of 200 mg/l.  The effluent is to have a BOD of 30mg/l.

@@@@@