Code No: 310105

Set No.

1

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

GEOTECHNICAL ENGINEERING

(Civil Engineering)

Time: 3 hours						Max. Marks: 80

Note: - 1.  Answer any Five questions.

2.  All questions carry equal marks.

3.  Assume any data not given suitably.

---

1.a)	Distinguish coarse grained and fine grained soils in respect of particle size, shape, feel and factors influencing their behaviour.

b)	800 cu.m of earth fill is to be constructed at a void ratio of 0.75. How many cubic metre of earth is to be excavated from a borrow pit in which the void ratio is 1.10.

2.a)	Discuss the factors affecting coefficient of permeability of soils.

b)	A soil consists of stiff clay 12m thick over a bed of sand. During the excavation of a trench for a foundation ground water was observed at a depth of 4m below ground surface. The specific gravity of clay is 2.75 and the void ratio is 0.8. If the excavation is to be carried safely to a depth of 9.5m, by how much the water table should be lowered in the vicinity of the trench. Find also the depth of excavation that can be carried out without the danger of the bottom becoming unstable under the uplift pressure of ground water.

3.a)	Clearly bringout the differences in Boussinesq’s and Westergaard’s theories for stress distribution and explain how Boussinesq’s theory is used for determining the increase in vertical stress due to a point load Q applied at the ground surface.

b)	An overhead water tank is supported at a depth of 3.5m by four isolated square footings of 2.1m side each placed in a square pattern with a centre-to-centre spacing of 8m. Compute the vertical stress at the foundation level (i) At the centre of the four footings and (ii) At the centre of one footing. Adopt Boussinesq’s point load approximation. The load on each footing is 650 kN.

4.a)	Discuss the significance of (i) Max.Dry density and (ii) Optimum moisture content in field compaction. State the differences between standard Proctor compaction and modified proctor compaction.

b)	A standard proctor compaction test was carried out and the following values were recorded. The volume of the mould is 994 CC.

| Weight of soil (gm) | 1772 | 1837 | 1860 | 1871 | 1906 | 1865 | 1850 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Water content (%) | 17.5 | 19.0 | 20.0 | 20.8 | 21.8 | 22.4 | 24.2 |

Plot the Dry density vs. moisture content curve and hence find the optimum moisture content and maximum dry density. What is Relative compaction?

(Contd…2)

Code No: 310105                            	-2-                                                	Set No.1

5.a)	Explain the terms (i) Normally consolidated soils and (ii) Over consolidated soils.

State the reasons for soils in the field to become over consolidated.

b)	In a consolidation test, the pressure on a sample was increased from 150kN/m2 to

300 kN/m2. The void ratio after 100% consolidation under 150kN/m2 was 0.945 and that under 300 kN/m2 it was 0.810. The coefficient of permeability of the soil was 24 x 10-6 mm/sec and the initial height of the sample was 20mm. Determine

(i) The coefficient of compressibility	(ii)  The coefficient of volume compressibility	(iii)  Coefficient of consolidation and 	(iv) The time taken in days for 90% consolidation of the layer of this clay, 5m thick in field and soadwiched between an impervious layer beneath and the pervious layer at the top.

6.a)	Discuss the types of shear tests based on drainage conditions and state the field applications for these tests.

b)	The following results were obtained from a consolidated undrained test on a normally consolidated clay. Plot the strength envelope interms of effective stresses and determine the strength parameters

| Sample No. | Cell Pressure (KN/m2) | Deviator Stress (KN/m2) | Pore Water Pressure (KN/m2) |
| --- | --- | --- | --- |
| 1 | 250 | 152 | 120 |
| 2 | 500 | 300 | 250 |
| 3 | 750 | 455 | 350 |

7.a)	Explain the terms (i) Coefficient of compressibility and (ii) Coefficient of volume compressibility.

b)	A Boring log reveals that a thin layer of silty clay exists at a depth of 15m below the natural ground surface. The soil above this layer is a silt having a dry density of 15.5 kN/m3 and a water content of 28%. The groundwater table is found to exist approximating near the ground surface. Triaxial shear tests on the undisturbed silty clay give the following results Ccu=48.3 kN/m2; cu = 13o; Cd1=41.4kNm2;φd1=23∘.  Estimate the shearing resistance of the silty clay on a horizontal plane (i) when the shear stress builds up rapidly and (ii) when the shear stress builds up very slowly.

8.	Write short notes on the following:

- Consistency limits and their importance.

- Characteristics of Grain size distribution curve.

- Unconfined compressive strength.

- Sensitivity.

***     ***     ***

Code No: 310105

Set No.

2

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

GEOTECHNICAL ENGINEERING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

Note: - 1.  Answer any Five questions.

2.  All questions carry equal marks.

3.  Assume any data not given suitably.

---

1.a)	Discuss the soil structure of fine grained soils.

b)	An earthen embankment under construction has a bulk unit weight of 1.6t/m3 (16KN/m3) and a moisture content of 10 percent. Compute the quantity of water in litres required to be added per cubic metre of earth to raise the moisture content to 14% at the same void ratio.

2.a)	Derive an expression for coefficient of permeability in a falling head permeameter test with usual notations.

b)	The water table at a building site is at a depth of 6m below the ground surface. The soil strata has an average liquidity index of 0.75 and a L.L of 60% and P.L of 38%. Assume saturated soil conditions and a specific gravity of solids as 2.74.

Calculate the effective stress at a depth of 9.0 m below ground level. Draw a profile of total, effective and neutral stresses to this depth.

3.a)	Explain how Newmark’s influence chart is constructed. Explain how it is used to find the increase in vertical stress.

b)	A pumping test was made in a pervious soil extending to a depth of 19m where a bed of clay was encountered. The normal ground water table was 1m below the ground surface. Observation wells were located at distances of 4.2m and 8.1m  from the pumping well. At a discharge of 8.8 cum per minute from the pumping well a steady state was attained in 24 hours. The draw down at 4.2m was 2m and at 8.1m was 0.5m. Compute the coefficient of permeability in cm/sec.

4.a)	Briefly describe the different methods of compaction in field and state their suitability to each type of soil.

b)	The max.dry density of a sample by the light compaction test is 1.79 gm/cc at an optimum water content of 16%. Find the air content and the degree of saturation. Take G=2.67.

What would be the corresponding value of the dry density on the zero air void line at O.M.C.

(Contd…2)

Code No: 310105                            	-2-                                                Set No.2

5.a)	Distinguish clearly between consolidation of soils and compaction of soils. Also state the difference between primary consolidation and secondary consolidation in soils.

b)	In an oedometer test a clay specimen initially 25mm thick attains 90 percent consolidation in 10 minutes. In the field the clay stratum from which the specimen was obtained has a thickness of 5.8m and is sandwiched between two sand layers. A structure constructed on this clay experienced an ultimate settlement of 200mm. Estimate the settlement at the end of 100 days after construction.

6.a)	Discuss the Mohr-Coulomb failure theory as applied to soils.

b)	Two partially saturated identical specimens of a compacted soil 4cm in diameter and 8cm high are tested in a triaxial cell under undrained conditions with confining pressures of 100 and 200 kPa respectively. The respective additional axial loads at failure for the two specimens are measured from a proving ring as 0.89 kN and 1.082 kN. The increase in volume of the first specimen at failure is 1.0 ml and it fails at an axial strain of 8%. The second specimen fails at a strain 10% with an increase in volume equal to 1.5ml. Determine the values of apparent cohesion and the angle of shearing resistance for the soil by drawing Mohr circles.

7.a)	What are the assumptions made by Terzaghi in his theory on one dimensional consolidation and discuss their validity.

b)	Determine the average coefficients of permeability in the horizontal and vertical directions for a deposit consisting of three layers of thickness 5m, 1m and 2.5m and having the coefficients of permeability of 3 x 10-2mm/sec, 3 x 10-5 mm/sec and 4 x 10-2 mm/sec respectively. Assume the layers are isotropic.

8.	Write short notes on the following:

- Relative consistency and liquidity index of soils.

- Thixotropy and its importance.

- Relative density of soils.

- The engineering properties of soils and their importance.

***     ***     ***

Code No: 310105

Set No.

3

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

GEOTECHNICAL ENGINEERING

(Civil Engineering)

Time: 3 hours							        Max. Marks: 80

Note: - 1.  Answer any Five questions.

2.  All questions carry equal marks.

3.  Assume any data not given suitably.

---

1.a)	Derive an expression for the bulk density of a soil in terms of specific gravity of soil grains, void ratio, degree of saturation and unit weight of water and hence deduce expressions for dry unit, saturated unit weight and submerged unit weight of soil.

b)	A sample of clay weighing 675 gm was coated with paraffin wax. The combined weight of clay and the wax was found to be 682 gm. The volume was found by immersion in water as 345ml. The sample was then broken open and the water content and specific gravity of solids were found to be 15% and 2.70. Calculate the bulk density of soil, its void ratio and degree of saturation. Take specific Gravity of wax as 0.89.

2.a)	Derive an expression for the discharge per unit time per unit length of a spillway to be obtained from flownets.

b)	The atterberg limits of a clay soil are: liquid limit 53%; plastic limit 31% and shrinkage limit 18% if the specimen of this soil shrinks from a volume of 39.5cm3 at the liquid limit to a volume of 24.2cm3 at the shrinkage limit, calculate the true specific gravity.

3.a)	Explain the two approximate methods used for determining the increase in vertical stress and state their limitations if any.

b)	The contact pressure for a 2m x 2m footing is 2.3 kg/cm2. Using 60o dispersion and also 6312∘ dispersion find out the depth at which the contact pressure will be 0.45 kg/cm2.

4.a)	Discuss the factors affecting compaction of soils.

b)	A sample of soil was prepared by mixing a quantity of dry soil with 10% by mass of water. Find the mass of this wet mixture required to produce a cylindrical compacted specimen of 14.5cm in diameter and 13cm in height and having 6% air content. Find also the void ratio and the dry density of the specimen if G=2.68.

(Contd…2)

Code No: 310105                            		-2-                                                Set No.3

5.a)	Drive the expression for the settlement of a soil layer as S=CC1+e∘.H.log10p+Δpp∘  with usual notations.

b)	A soil of specific gravity 2.65 has a moisture content of 18% when fully saturated. 1.9cm thick sample of this soil tested in a consolidometer shows a compression of 0.05cm when the load is increased from 40 kN/m2 to 80 kN/m2. Compute the compression index of the soil.

6.a)	Discuss the types of shear tests available for determining the shear parameters and state their relative merits and demerits.

b)	A direct shear box test performed on a remoulded sand sample gave the following observations at the time of failure.

Normal load = 0.36kN; shear load = 0.18kN; area of sample = 36cm2.

Determine (i) The angle of internal friction 	(ii) The magnitude and direction of the principal stresses in the zone of failure  and 	(iii) The magnitude of max. deviator stress if a sample of the same sand with the same void ratio were tested in a triaxial test with an alround pressure of 60 kN/m2.

7.a)	State Darcy’s law. To what type of soils this law is applicable. State its limitations.

b)	A saturated soil stratum 5m thick lies above an impervious stratum and below a pervious stratum. It has a compression index of 0.25 and a coefficient of permeability of 3.2 x 10-8 cm/sec. Its void ratio at a stress of 1.5 kg/cm2 is 1.9. Compute  (i) The change in void ratio due to increase of stress to 2.0 kg/cm2 (ii) Settlement of the soil stratum due to the above increase in stress  (iii) The time required for 50% consolidation for which the time factor may be assumed as 0.2.

8.	Write short notes on the following:

a)	Total, effective and neutral stresses.

b)	Field identification of soils as per IS.

c)	Superficial Velocity and Actual Velocity and the relationship between them (derive).

d)	Critical Hydraulic Gradient and Quick sand condition.

***     ***     ***

Code No: 310105

Set No.

4

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

GEOTECHNICAL ENGINEERING

(Civil Engineering)

Time: 3 hours							        Max. Marks: 80

Note: - 1.  Answer any Five questions.

2.  All questions carry equal marks.

3.  Assume any data not given suitably.

---

1.a)	Explain how fine grained soils are classified according to IS procedures.

b)	A compacted cylindrical specimen 50mm dia and 100 mm length is to be prepared from oven-dry soil. If the specimen is required to have a water content of 15% and the percentage air voids of 20%, calculate the mass of the soil and the water required for the preparation of the sample. Take G = 2.69.

2.a)	What are consistency limits. Explain briefly about these limits. State the significance of these limits and how these influence the behaviour of fine grained soils.

b)	An earth dam is built on an impervious foundation with a horizontal filter at the base level near the toe. The horizontal and vertical permeabilities of the soil material in the dam are 3.6 x 10-3 and 1.2 x 10-3 cm/sec respectively. The full reservoir level is 26m above the filter. A flow net constructed for the transformed section of the dam consists of 4 flow channels and 15 equipotential drops. Estimate the seepage loss per 50 m length of the dam.

3.a)	Mention the characteristics of flow nets.

b)	What is quick sand condition and derive an expression for it with usual notations.

c)	A load of 15 kN/m2 is uniformly distributed over a circular area of 5.5m diameter at the ground surface. Calculate the vertical stress at a point P which is at a depth of 5m directly below the centre of the loaded area.

4.a)	Briefly discuss the effect of compaction on the engineering properties of soils.

b)	The following results are obtained standard compaction test performed on a sample of soil.

| Water content (%) | 5 | 10 | 14 | 20 | 25 |
| --- | --- | --- | --- | --- | --- |
| Bulk density (gm/cc) | 1.77 | 1.98 | 2.10 | 2.18 | 2.16 |

Plot the water content, dry density curve and optain the optimum water content and maximum dry density.

(Contd…2)

Code No: 310105                            		-2-                                                Set No.4

5.a)	Describe a method to determine the preconsolidation pressure of soil. Explain its significance.

b)	A 24 mm thick undisturbed sample of a saturated clay is tested in the laboratory with drainage allowed on both faces. The sample reaches 50% degree of consolidation in 45 minutes. If the clay layer from which the sample was obtained is 4.8m thick and is free to drain at both of its faces, calculate the time required for the clay layer to undergo the same degree of consolidation. What would have been the time of consolidation if the clay layer has only single drainage. Assume uniform distribution of consolidating pressure.

6.a)	Mention the uses of pore pressure coefficients in practice.

b)	A specimen of  fine dry sand when subjected to a triaxial compression test failed at a deviator stress of 400kN/m2. It failed with a pronounced failure plane with an angle of 24o to the axis of the sample. Compute the lateral pressure to which the specimen would have been subjected to.

7.a)	Explain clearly how course grained soils are classified using IS: procedures. What is the usefulness of classifying soils.

b)	A moist soil sample has a volume of 465cc in natural state and a weight of 795 gm. The dry weight is 735 gm. Specific gravity of soil grains is 2.68. Determine the void ratio, porosity, water content and the degree of saturation (solve the problem from first principles).

8.	Write short notes on the following:

- Compressibility parameters: compression index and coefficient of consolidation.

- Types of shear tests based on drainage condition.

- How the shrinkage ratio of a soil is equal to the apparent specific gravity.

- Normally consolidated soils and over consolidated soils.

***     ***     ***