Code No: 310107		FOR 1999 BATCH

Set No.

1

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERING –II (STEEL) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

NOTE: 1) Answer One question from PART-A and any three     questions from PART-B

2) Use of I.S. 800 and I.S 875 is permitted.

3) Any missing data may be suitably assumed.

---

PART-A

(Marks: 32)

1. a)	Design the mid-span section of a riveted plate girder simply supported over an effective span of 16m and subjected to a super imposed load of 80kN/m throughout the length.  Design the riveted connections between

(i)	Flange angles and Flange Plates

(ii)	Flange angles and web plate.

b)	Draw the cross section and longitudinal section of the plate girder to a suitable scale showing the intermediate stiffeners and bearing stiffeners (Design of stiffeners need not be done).

2. a)	Design a suitable roof truss supported over an effective span of 18m with pitch of 1/4.  The spacing of the trusses is 4m.  The building is situated in Hyderabad.

b)	Draw the roof truss to a suitable scale showing the joint details.

PART-B

(Marks: 3 X 16 = 48)

3. a)	Draw the various types of riveted Joints.					(6)

b)	A bracket comprising of two plates connecting to both the sides of a column ISMB 300 is carrying a load of 35kN.  The load is acting at 200mm from the centre of the web of the column (at the top level of the plate) and is inclined at 60o to the horizontal.  Design the riveted connection.  The thickness of bracket plates is 10mm.									(10)

4. a)	Write the advantages and disadvantages of welding.				(8)

b)	A 100mm X 10mm plate is to be welded to another plate 150mm X 10mm by the fillet welding on three sides.  The size of the weld is 6mm.  Find out the necessary over lap of the plate, for full strength of the Joint.  The allowable tensile stress in plate is 15MPa and allowable stress in weld is 110MPa.			(8)

5.	A simply supported beam is to support a u.d.l.  of 100kN/m over a clear span of 8m.  Design a plated rolled steel beam if ISMB 500@ 0.869 kN/m and 10mm thick plates are only available.  The compression flange of the beam is laterally restrained.

Contd…2

Code No. 310107				.2.			Set No. 1

6.	A tension member, consisting of 4 ISA 110 X 110 X 10mm is connected to a gusset plate by 20mm diameter rivets as shown in fig(1).  Calculate the permissible tension  in the member when

(i)	No tacking rivets are used

(ii)	Tack riveting is done along C and D only

- Tack riveting is done along A and B only

- Tack riveting is done along A, B, C and D.

7.	A steel column 12m long carries an axial load of 1500kN.  The column is hinged at both the ends.  Design an economical built-up section with double lacing.  Design the lacing also.

###

Code No: 310107	FOR 1999 BATCH

Set No.

2

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERING –II (STEEL) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

NOTE: 1) Answer One question from PART-A and any three     questions from PART-B

2) Use of I.S. 800 and I.S 875 is permitted.

3) Any missing data may be suitably assumed.

---

PART-A

(Marks: 32)

1. a)	Design the central section of welded plate girder simply supported over an effective span of 24m and subjected to a super imposed load of 100kN/m through out the length.  Design

(i)	Connection between flange and web

(ii)	End bearing stiffeners

- Intermediate stiffeners

b)	Draw the longitudinal Elevation and sectional plan of the welded plate girder to a suitable scale.

2. a)	Design a suitable roof truss supported over an effective span of 20m with pitch of 1/5.  The spacing of the trusses is 4.5m.  The building is situated in coastal area.

b)	Draw the roof truss to a suitable scale showing the joint details.

PART – B

(Marks: 3 X 16 = 48)

3. a)	Define the terms

(i) Pitch	(ii) Gauge distance	(iii) Rivet value    (iv) Efficiency of a riveted Joint.										(6)

b)	A bracket plate 12mm thick transmits a load of 200kN at an eccentricity of 250mm to a column section ISHB 250 through 14 Nos-22mm  rivets arranged in two vertical rows 100mm apart.  The pitch of the rivets is 80mm and the load lies in the plane of the rivets.  Check the safety of the Joint.			(10)

4.	A bracket, carrying a load of 100kN at an eccentricity of 120mm is connected to a stanchion by fillet weld, on both the sides of the plate, as shown in the fig(1).

(i)	Determine the size of the fillet weld

(ii)	If 8mm fillet weld is used, determine the depth of the bracket.

(iii)	If 8mm fillet weld is used with a bracket of 250mm depth.  Calculate the resulting stress in the weld.

Contd…2

Code No. 310107				.2.				Set No. 2

5.	Design a laterally unsupported beam for the following data.

Effective Span		= 6m

Super imposed load 	= 50 kN/m

The ends of the beam are restrained against torsion and lateral bending.  Check for web buckling also.

6. a)	Long leg of an ISA 150mm X 75mm X 10mm is connected to a gusset plate by 20mm diameter rivets in two rows.  The gauge distance is 55mm and the staggered pitch is 40mm.  Determine the thickness of the angle which would be sufficient to transmit a pull of 200kN.					(10)

b)	An ISA 60 X 60 X 8mm is connected to a gusset plate by one leg only by 18mm  rivets in one chain line along the length of the member.  Determine the tensile strength of the member.							(6)

7.	A column 5m long is to support a load of 4000kN.  The ends of the column are effectively held in position and direction.  Design the column if rolled steel beams and 18mm plates are only available.

###

Code No: 310107		FOR 1999 BATCH

Set No.

3

III-B.Tech I Semester Supplementary Examinations, May 2003

STRUCTURAL ENGINEEIRNG –II (STEEL) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

NOTE: 1) Answer One question from PART-A and any three     questions from PART-B

2) Use of I.S. 800 and I.S 875 is permitted.

3) Any missing data may be suitably assumed.

---

PART-A

(Marks: 32)

1. a)	Design the mid-span section of a riveted plate girder simply supported over an effective span of 18m and subjected to two concentrated loads of 400kN at 6m from both the ends along with super imposed u.d.l of 50kN/m throughout the length.  Design the riveted connections between

(i)	Flange angles and Flange plates.

(ii)	Flange angles and web plate.

b)	Draw the cross section and longitudinal section of the plate girder to a suitable scale showing the intermediate stiffeners and bearing stiffeners.  (Design of stiffeners need not be done)

2. a)	A built-up column consists of ISHB 400 with one 300mm X 12mm flange Plate on each side.  The column carries an axial load of 2400kN.  Design a gusseted base, if the column is supported on concrete pedestal with a bearing capacity of 4N/mm2.  Design the required connections also.

b)	Draw the plan and two side views of the gusseted base for the above problem to a suitable scale.

PART – B

(Marks: 3 X 16 = 48)

3. a)	Explain the various types of connections used for connecting the structural members.									(6)

b)	Design a riveted Joint as a

(i)	Lap Joint

(ii)	Single cover butt Joint

- Double cover butt Joint

to connect two plates 14mm thick.  Power driven rivets may be used for making the connection.								(10)

Contd…2

Code No. 310107				.2.				Set No. 3

4. a)	Define the following terms

(i) Size		(ii) Effective throat thickness		(iii) Effective length

of the fillet weld.								(6)

- Design the welded connection for the bracket loaded as shown in fig (1).	(10)

5.	A class room 7m X 15m is provided with 120mm thick stone patties over rolled steel beams spaced @ 3m c/c.  A wearing coat of 20mm thick cement concrete is provided over 160mm thick lime concrete.  The effective length of compression flange is equal to the effective span of beam.  Design rolled steel beams.  The ends of beams shall not be free to rotate at the bearings.

Live load			=	3kN/m2

Unit weight of stone patties	=	24kN/m3

Unit weight of lime concrete	=	18kN/m3.

6. a)	Design a tension member consisting of a pair of angles (back to back) and connected by the short legs to (i) the same side of gusset plate  (ii) each side of the gusset plate. The member is to carry a pull of 300kN.			(12)

b)	What is the significance of net sectional area in angle ties.			(4)

7.	Design a built-up column consisting of two channels connected by battens to carry an axial load of 800kN.  The effective length of the column is 5m.

###

Code No: 310107			FOR 1999 BATCH

Set No.

4

III-B.Tech I Semester Supplementary Examinations, June 2003

STRUCTURAL ENGINEERNG –II (STEEL) DESIGN AND DRAWING

(Civil Engineering)

Time: 3 hours							Max. Marks: 80

NOTE: 1) Answer One question from PART-A and any three     questions from PART-B

2) Use of I.S. 800 and I.S 875 is permitted.

3) Any missing data may be suitably assumed.

---

PART-A

(Marks: 32)

1. a)	Design the central section of welded Plate girder simply supported over an effective span of 20m and subjected to a super imposed load of 80kN/m throughout the length.  Design.

(i) Connection between flange and web

(ii) End bearing stiffeners

(iii) Intermediate stiffeners

b)	Draw the longitudinal Elevation and sectional Plan of the welded Plate girder to a suitable scale.

2. a)	A built-up column consists of ISMB 450 with one 250mm  X 12mm flange plate on each side.  The column carries an axial load of 2000kN.  Design a gusseted base, if the column is supported on concrete pedestal with a bearing capacity of 4N/mm2.  Design the required connections also.

b)	Draw the Plan and two side views of the gusseted base for the above problem to a suitable scale.

PART-B

(Marks: 3 X 16 = 48)

3. a)	Explain the different types of failures of a riveted Joint.			(8)

b)	A double riveted double cover butt Joint is used to connect plates of 12mm thick.  Determine the diameter of rivet, rivet value, gauge and efficiency of Joint.	(8)

4. a)	Draw the various types of welded Joints.					(8)

b)	A tie member of a roof truss consists of 2 ISA 90 X 60 X 10.  It is subjected to a pull of 250kN.  The angles are connected to either side of a gusset plate 10mm thick.  Design the welded connection.

5. a)	Write short notes on

(i) Web crippling	(ii) Web buckling					(6)

b)	A beam ISMB 600 @ 122.6 kg/m is simply supported over a span of 8m.  Determine the safe u.d.l the beam can carry, assuming that the beam is laterally unsupported.  The ends of the beam are restrained against torsion and the ends of the compression flanges are unrestrained against lateral bending.		(10)

Contd…2

Code No. 310107				.2.				Set No. 4

6. a)	A double angle tie ISA 150 X 75 X 10mm of a roof truss (short legs back to back) is connected to (i) the same side of a gusset plate (ii) each side of a gusset plate with 18mm diameter rivets, such that each angle is reduced in section by one rivet hole only.  Determine the tensile strength of the member in both the cases.	(12)

b)	What is the significance of net sectional area in angle ties.			(4)

7. a)	A built-up column consists of ISMB 300@ 44.2 kg/m, having two plates, each of 10mm thickness, attached to each flange so as to have equal resistance about either axis.  Determine the load the column can safely carry over an effective length of 4.5m.

b)	Briefly explain the design principles of eccentrically loaded columns.

###