Code No: 310202

Set No.

1

III-B.Tech. I Semester Supplementary Examination, June 2003

ELECTRICAL MEASUREMENTS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a) 	With usual notation derive an expression for the deflecting torque in a PMMC

instrument.

b)	Explain how the PMMC instrument can be employed to measure

(i)	Voltage	(ii)	Current.

2.a) 	What is the principle of working of a repulsion type moving iron instrument?

b)	Explain with a neat sketch the working of such an instrument.

3.a) 	Explain the construction and working of the rotating type sequence indicator.

What is the application of phase sequence indicator. What is the application

of phase sequence indicator at generating station.

b) 	What are the different types of frequency meters available and hence explain with a neat sketch the construction and working principle of reed frequency meter

4.a) 	What are the accessories used with potentiometers.

b) 	What are the applications of potentiometers.

c) 	With a circuit diagram explain salient features of a self balancing

potentiometers.

5.a)	What are the different factors that affect the precision measurement of medium resistances with wheatstone bridge. Explain how their effects are minimized.

b) 	The four arms of a wheatstone bridge are as follows. AB = 1K, BC = R,                   CD = 5K and DA = 1K. A galvanometer is connected between BD and a dc voltage source between AC. The Thevinin source generator voltage E0 is given by 24mV and the galvanometer current is 13.6 Micro Amp. Calculate the value of R.

6.	Describe how high currents and voltages are measured with the help of instrument transformers.Draw diagrams to illustrate your answer.Describe the advantages of instrument transformers as regards extension of  range of current and voltage on high voltage a.c systems.

7. 	Describe the errors in electrodynamometer type of wattmeter.

8.a) 	Explain the construction and  working of a MI type power factor meters .

b) 	Describe the construction and working of weston type synchroscope.

***     ***     ***

Code No: 310202

III-B.Tech. I Semester Supplementary Examination, June 2003

Set No.

2

ELECTRICAL MEASUREMENTS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1a) 	What is a shunt referred to a PMMC instrument. Discuss the design of such shunts in extending the range of an ammeter.

b) 	How temperature effect is corrected in the shunts. Discuss with a neat circuit

diagram.

2.a) 	What are the advantages and limitations of a moving iron instrument?

b) 	The inductance of a moving iron instrument is given by L= (10 + 5 -  2)H.

where  is the deflection in radian from zero position. The spring constant is

12 x 10 -6  Nm / rad. Estimate the deflection for a current of 5 amps.

3.a) 	With a neat figure, explain the construction and working principle of electrical

resonance frequency meter.

b) 	Explain  the term “Standardization of a potentiometer”. Describe the procedure for the standardization of a D.C. potentiometer.

4.a)	Derive an expression for current through the galvanometer connected in wheatstone bridge for a small unbalance.

b)	The four arms of the wheatstone bridge are given by AB = 100, BC = 10,     CD = 4 and DA = R. Find the value of R for no current through the galvanometer. The galvanometer has a resistance of 20and is connected across BD. Across AC there is a source of 10V dc.

5.a)	Write a circuit diagram to explain salient features of self balancing potentiometer.                                                                                           b) 	Explain clearly the construction and working principle of “DRYSDALE” polar 	type ac potentiometer.

6.	Describe the ‘Silsbee’s method ‘for testing of a current transformer.

7. 	Explain the special features incorporated in an electrodynamometer type of wattmeter so that it can be used for low power factor applications.

8.a) 	Describe  the construction and  working of  a moving coil ballistic galvanometer

b) 	Describe the method of experimental measurement of flux density in a specimen of magnetic material using ballistic galvanometer.

***     ***     ***

Code No: 310202

III-B.Tech. I Semester Supplementary Examination, June 2003

Set No.

3

ELECTRICAL MEASUREMENTS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a) 	Explain different types of deflecting systems which are operating in an indicating

instrument.

b)	Discuss the role of controlling system in an indicating instrument.

c)	How are controlling torques produced. Explain with neat sketches the different methods of producing controlling torque.

2.a) 	Compare between attraction type and repulsion type of moving iron instruments.

b) 	What are the errors found in moving iron instruments? Explain also the method of

compensation.

3.a)	With a neat sketch, explain the working of Single Phase Energy motor.

b)	What are the various adjustments that are required to be made in a single phase energy meter, so that it reads correctly? Explain each one of them.

4.a)	With a neat sketch, explain the measurement of resistance using a potentiometer.

b) 	Draw circuit diagram and explain the measurement of power using potentiometer.

5.a)	What are the various limitations of wheatstone bridge for measurement of high and low resistances.

b) 	The four arms of a whetstone bridge are as follows. AB = 100, BC=1000,             CD = 4000 and DA = 400. The galvanometer has a resistance of 100, a sensitivity of 100 mm / Micro Amp. and is connected across AC. A source of 4V dc is connected across BD. Calculate the current through the galvanometer.

6. 	Describe the ‘Variable Mutual Inductance method for testing of a current transformer.

7. 	Describe the method of measurement of reactive power in three phase circuits using single dynamometer type wattmeter.

8.	Write short notes on the following

a)	Loss of energy due to hysteresis.

b)	Synchroscope.

***     ***     ***

Set No.

4

Code No: 310202

III-B.Tech. I Semester Supplementary Examination, June 2003

ELECTRICAL MEASUREMENTS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a) 	Explain primary and secondary method of measurements. How are primary

standards and secondary standards are defined for purpose of measurement.

b) 	Explain deflecting system, controlling system and damping system with reference

to an electrical indicating instrument.

2.a) 	How torque is produced in an electrostatic instrument. Derive an expression for

the torque in terms of relevant quantities.

b) 	What are the different types of electrostatic instruments? Discuss the working of

Gold-leaf electroscope.

3.  	Describe with the help of suitable diagrams how a D.C. potentiometer can be used for the calibration of (a) an ammeter (b) voltmeter and (c) a wattmeter.

4.a) 	With a neat figure, explain the construction and working principle of Weston

frequency meter.

b) 	What is a sequence indicator. Along with the construction explain the working

principle of rotating type sequence indicator.

5.a)	Explain the difference between balance conditions for DC and AC bridges.

b)	An AC bridge circuit working at 1000Hz, have its arms as follows. Arm AB  is 0.2 Micro Farad capacitance. Arm BC is a 500 Ohms resistance; arm CD contains an unknown impedance and arm DA has a 300 Ohms resistance in parallel with a 0.1 Micro Farad capacitor. Find the R and L or C constants of arm CD considering it as a series circuit.

6.	Describe the effect of the following on the characteristics of a potential transformers   (a) burden (VA) of secondary winding circuit (b) power factor of secondary winding circuit  (c) frequency  (d) supply voltage.

7. 	Explain the working of a 3 phase wattmeter.Describe how mutual effects between the two elements of the wattmeter are eliminated.

8.a) 	Write short notes about dial type Synchroscope.

b) 	With a suitable diagram explain the working of Schering and Schmidt Galvanometer.

***     ***     ***