Code No: 310203

Set No.

1

III-B.Tech. I-Semester Supplementary Examinations June, 2003

POWER SYSTEMS-II

(Electrical and Electronics Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Write a short notes on the following types of over head line conductors

(i) ACSR (ii) Bundled conductors (iii) Hollow conductors (iv) AAC conductors.

b)	Find the loop inductance and reactance per km of a single phase overhead line 	consisting of two conductors, each 1.213cm diameter. The spacing between 	conductors is 1.25 meters and frequency is 50Hz.

2.a)	What is symmetrical and asymmetrical spacing of conductors? What is the significance of symmetrical spacing of conductors?

b)	What is Radio Interference? Why it occurs explain.

c)	What is transposition? Explain the method of transposition.

3.a)	Discuss why equivalent pi circuit of a long line is  preferred over the equivalent   	T circuit.

b) 	A three phase 50Hz transmission line is 150 km long and delivers 25MW at 0.85 	power factor lagging and at 110KV. The resistance and reactance of the line per 	conductor per km are 0.3 ohms and 0.9 ohms respectively. The line charging 	admittance is 0.3 x 10 -6 mho per km per phase. Compute by applying the nominal 	pi method the voltage regulation and transmission efficiency.

4.a)	What is skin effect?

b)	A 3 phase, 50Hz, 100km long transmission line delivers a load of 20000KW at 	110KV at 0.9 power factor lagging. The copper conductors of the line are 1.2 cm 	in diameter and are spaced equilaterally, so that the distance between them is 2 m. 	Using nominal pi method, calculate the sending end voltage, current, power 	factor, regulation and efficiency of the line. Neglect the leakage.

5.a)	What is a sag-template? Explain how this is useful for location of towers and stringing of power conductors.

b)	How are the sag and tension of the conductor affected by an extreme fall of 	temperature?

6.a)	What are all the insulating materials used in U.G. cables?  Explain in detail about 	different kinds of insulating materials.

b)	Determine the economical size of a single core cable working on 220 KV, 3 phase 	system.  The maximum permissible stress in the dielectric is not to exceed

250 KV/cm.

Contd…..2

Code No: 310203				-2-				Set No:1

7.a)	Why the improvement of power factor is very important for both consumers and generating stations?

b)	List the various causes of low power factor and explain.

c)	List the power factor improvement equipment and briefly explain.

8.a)	Compare the merits and demerits of various methods of p.f. improvement methods.

b)	A single phase motor connected to a 240V, 50Hz supply takes 20A at a p.f. of 	0.75 (lag). A capacitor is shunted across the motor terminals to improve the p.f. to 	0.9 (lag). Determine the capacitance of the capacitor to be used.

- - -

Code No: 310203

Set No.

2

III-B.Tech. I-Semester Supplementary Examinations June, 2003

POWER SYSTEMS-II

(Electrical and Electronics Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	What factors must be taken into account while calculating the resistance of Overhead  line conductors? How are these factors accounted for?

b)	What is equivalent spacing of a 3-phase line?  What is its significance?

c)	A 3-phase 50km long single circuit 66KV transposed Overhead line has 	horizontal spacing with 3 meters between adjacent conductors and 6 meters 	between outer conductors. The conductor diameter is 2cm. Find the inductance 	per phase.

2.a)	Why transmission line conductors are stranded?

b)	Give important application of Bundled and Hollow conductors.

c)	Why transmission lines are transposed? Explain the procedure of transposition.

d)	A single phase transmission line has conductors 2cm in diameter spaced 3 meter apart. Calculate the inductance per km of the loop.

3.a)	Discuss why receiving end voltage of an unloaded long line may be more than the sending end voltage.

b)	A single phase transmission line 100 km long has the following constants.  Resistance per km is 0.25 ohms and reactance per km is 0.8 ohms.  Susceptance per km is 14 micro mho.  Receiving end voltage is 66 KV.  Assuming that the total capacitance of the line is localized at the receiving end alone, determine sending end voltage, sending end current, regulation, efficiency and supply power factor.  The line is delivering 15MW at 0.8 lagging power factor.  Draw the vector diagram to illustrate your calculations.

4.a)	What is Proximity effect?

b)	A 345 KV 3 phase transmission line is 130km long. The resistance per phase=0.036 ohms/km and inductance per phase is 0.8 mH/km. The shunt capacitance is 0.0112 micro Farad/km. The receiving end load is 270MVA with 0.8 power factor lagging at 325KV. Find the voltage and power at the sending end and the voltage regulation. Use

(i)   Nominal T method

(ii)  Nominal pi method

(iii) ABCD constants.

Compare the result.

Contd…..2

Code No: 310203				-2-				Set No:2

5.a)	The transmission line is designed based on worst probable conditions and not worst possible conditions. Why?

b)	An overhead line has the following data: span length 160metres, conductor dia 	0.95cm, weight per unit length of the conductor 0.65kg/metre. Ultimate stress 	4,250Kg/cm2, wind pressure 40Kg/Cm2 of projected area. Factor of safety 5. 	Calculate sag.

6.a)	What is the relation between the conductor diameter and breakdown potential of a 	cable while voltage of the cable and its overall diameter are fixed?  Prove the 	same.

b)	The capacitance per kilometer of a 3-phase belted cable is 0.25 microfarads 	between the two cores with the third core connected to the lead sheath calculate 	the charging current taken by five kilometers of this cable when connected to a     	3-phase,  50 Hz, 11 KV supply.

7.a)	List and explain any four disadvantages of low power factor.

b)	Derive an expression for most economical power factor for constant              	KVA-	demand.

8.a)	Explain the method of improving power factor by using synchronous condenser. Discuss the merits and demerits of the above method.

b)	A synchronous motor improves the p.f. of a load of 200KW from 0.8 lagging to 0.9 lagging. Simultaneously the motor carriers a load of 60KW. Find

i)  The leading KVAR supplied by the Motor.

ii) The p.f. at which motor operates.

- - -

Code No: 310203

Set No.

3

III-B.Tech. I-Semester Supplementary Examinations June, 2003

POWER SYSTEMS-II

(Electrical and Electronics Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	How can the inductance of a two sub-conductor bundled conductor single phase line be calculated? Derive expressions for GMR and GMD of the arrangement.

b)	A 3-phase transposed line has conductors of diameter 2cm and spaced at distance 	of 3, 5 and 8 meter between the centers. Calculate the inductance per phase per 	km of line length.

2.	Derive an expression for line to neutral capacitance for a 3-phase line when conductors are:

a) Symmetrically placed.

b) Unsymmetrically placed but transposed.

3.a)	Draw the phasor diagram of a medium transmission lines represented by a T 	model and derive the expression for  voltage regulation.

b)	Determine the efficiency and regulation of a 3 phase, 50Hz transmission line 	having resistance, inductance and capacitance of 10 ohms, 0.1 H and 0.9 micro 	farads respectively. The line delivers a load of 35MW at 132 KV and 0.8 p.f. lag. 	Use nominal T method.

4.a)	How do you classify the transmission lines?

b) 	A 220KV, 3 phase transmission line is 40 km long. The resistance per phase is 	0.15 ohms/km and the inductance per phase is 1.3263 mH per km. The shunt 	capacitance is negligible. Use the short line model to find the voltage and power 	at the sending end and the voltage regulation and efficiency when the line is 	supplying a 3 phase load of:

(i)  381 MVA at 0.8 power factor lagging at 220KV.

(ii) 381 MVA at 0.8 power factor leading at 220KV.

5.a)	What is a stringing chart? Explain clearly the procedure adopted for stringing the power conductors on the supports.

b)	A transmission line conductor having a dia of 19.5mm weighs 0.85kg/m. The 	span is 275 meters. The wind pressure is 40kg/m2 of projected area with ice 	coating of 13mm. The ultimate strength of the conductor is 8000Kg. Calculate the 	maximum sag, if the factor of safety is 2 and ice weighs 910kg/m3.

Contd…..2

Code No: 310203				-2-				Set No:3

6.a)	Classify the under ground cables according to various parameters.

b)	A 3-core 11 KV cable supplies a load of 1500 KW at 0.85 power factor lag for 	280 days in a year at an average of 9 hours per day.  The capital cost per 	kilometer of the cable is Rs. 8000a + 20,000.  The resistance per Km of a cable of 	cross-sectional area 1 cm2 is 0.173 ohms.  If the energy loss cost per unit is 	2-paise and the rate of interest and depreciation is 12% calculate the most 	economical current density and the dia of the conductor.

7.a)	What is the importance of power factor in the supply system? What is the effect of low power factor on the generating stations?

b)	Why is unity power factor not the most economical p.f.? Suggest the most 	economical value of power factor for a particular customer by deriving 	appropriate derivation with suitable assumptions. KW of load is maintained 	constant.

8.a)	Explain the method of improving power factor by the method of using phase advancers.

b)	A 3-phase, 50Hz, 400V Motor develops 80HP, the p.f. being 0.75 (lag) and 	efficiency 93%. A bank of capacitors is connected in delta across the supply 	terminals and p.f. is raised to 0.95 (lag). Each of the capacitance units is built of 4 	similar 100Vcapacitors. Determine the capacitance of each capacitor.

- - -

Code No: 310203

Set No.

4

III-B.Tech. I-Semester Supplementary Examinations June, 2003

POWER SYSTEMS-II

(Electrical and Electronics Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Develop an expression for the inductance of a single phase transmission line taking into account the internal flux linkages. Assume the conductors are solid.

b)	Calculate the inductance per km per phase of a 3-phase transposed line. With distance between any two conductors being 4m, 4m and 8 meter respectively.

2.a)	How do we find line to neutral capacitance in a 3-phase system?

b)	The three conductors R, Y and B of a 3-phase line are arranged in a horizontal plane with DRY=1.5m; DYB=2m and DBR=3.5m. Find line to neutral capacitance per km if dia of each conductor is 1.2cm. The conductors are transposed at regular intervals. Also calculate line capacitance per km length.

3.a)	What is a nominal circuit? Find ABCD constants for nominal pi circuit of a transmission line.

b)	A 300km 3 phase overhead line has a series impedance of 11+j106 ohms per phase and a shunt admittance of 1.13 milli mho per phase. The line delivers 250MW at unity power factor and at 275 KV. Find (i) A, B, C, D constants (ii) sending end voltage (iii) power angle.

4.a)	Discuss why Ferranti effect is significant only in medium and long lines?

b)	What are tuned power lines?

c)	The ABCD constants of a lossless three phase, 500KV transmission line are

A=D=0.86+j0		B=0+j130.2		C=j0.002

Obtain the sending end quantities and the voltage regulation when line delivers 	1000MVA at 0.8 power factor lagging at 500KV.

5.	An overhead line has a conductor of cross-section 2.5cm2 hard drawn copper and a span length of 150metres. Determine the sag which must be allowed if the tension is not to exceed one-fifth of the ultimate strength of 4,175Kg/Cm2.

- in still air and

- with a wind pressure of 1.3kg/metre and an ice coating of 1.25cms.

Determine also the vertical sag in the latter case.

Contd…..2

Code No: 310203				-2-				Set No:4

6.a)	What are the limitations of solid type cables?  Explain any other kind of cable to overcome the limitations.

b)	a single core lead covered cable is to be designed for 66 KV to earth.  Its conductor radius is 0.5 cm and its three insulating materials A, B and C have relative permittivities 4,4.5 and 2.5 with maximum permissible stresses of 50, 40 and 30 KV/cm respectively.  Find the minimum internal diameter of the lead sheath.

7.a)	Explain the concept of power factor.

b)	Explain any three causes of low power factor of supply system.

c)	Discuss the disadvantages of low power factor and list methods to improve power factor.

8.a)	What are symmetrical components? Why they are necessary?

b)	A 3-phase unbalanced system currents are read as IR=150A, IY=130A and IB=10A. The phase sequence is RYB. Find all the three symmetrical components for the case.

- - -