Code No. 310204

Set. No.

1

III-B.Tech I-Semester Supplementary Examinations, June 2003

POWER ELECTRONICS

(Electrical Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	Describe the different modes of operation of a thyristor with the help of its V-I characteristics?

2.	Explain series operation of SCR’s and derive resistance used for static voltage 	equalization for a series connected string.

3.	For a single phase voltage controller feeding a resistance load, draw the waveforms of source voltage, gating signals, output voltage.  Describe its working with reference to the waveforms drawn.

4.a)	Explain the operation of a 3-phase fully controlled rectifier with suitable waveforms of voltages and currents and a circuit diagram.

b)	A three-phase full wave controlled rectifier is operated from a three-phase Y connected 230V, 50Hz supply and the load resistance is 10.  If it is required to obtain an average output voltage of 50% of the maximum possible output voltage, calculate the delay angle.  If the leakage inductance of each phase of the input transformer is 2mH, calculate the overlap angle and the drop in the DC output voltage.

5.a)	What is a freewheeling diode?  Draw the circuit diagram of an SCR full wave rectifier with and with out free wheeling diode, and explain the operation of the circuit with the help of necessary waveforms.

b)	Explain the various performance factors of line commutated converters.

6.a) 	What is current limit control of chopper? Explain the operation.

b) 	The speed of a separately excited dc motor is controlled by a chopper. The dc 	supply voltage is 120V, armature circuit resistance is Ra=0.5Ω, armature 	circuit 	inductance is La=20mH, and motor back emf constant is Kb=0.05 	v/rpm. The motor drives a constant torque load requiring an average 	armature current of 20A.Assume that motor current is continuous.

Determine (i) the range of speed control 	(ii) the range of the duty cycle.

Contd..2

Code No. 310204			-2-					Set No.1

7.a)	Discuss with relevant waveforms class a self commutation by resonating the load, employed for thyristor circuits.

b)	For the class C commutation circuit, the DC source voltage Edc=120V and current through R1 and R2 =20A. The turnoff time of both the SCR is 60 µSec. Calculate the value of commutating capacitance for successful commutation.

8. a)	Explain the necessity of Inverters and list out the different applications of the same.

b)	Briefly explain the different types of Inverters with working principle.

###

Code No. 310204

Set. No.

2

III-B.Tech I-Semester Supplementary Examinations, June 2003

POWER ELECTRONICS

(Electrical Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define holding and latching currents.

b)	Describe various modes of operation of thyristor with the help of V-I 	characteristics.

2.a)	Describe the various anode voltage rating as applicable to an SCR.  Indicate these voltage ratings on a relevant voltage waveform.

b)	Describe the dynamic characteristics of SCR.

c)	Discuss how SCRs suffer from unequal voltage distribution across them during their turn on- and then-off process.

3.	Describe the operating principle of single phase to single-phase step up cycloconverter with the help of mid point and bridge type configuration.

4.a)	Draw the circuit diagram of a three pulse converter and obtain an expression for the average load voltage across a resistive load.

b)	A three phase full converter is operated form a three phase 400V, 50Hz supply.  The load resistance is 10 ohms.  Calculate the firing angle for an average output voltage of 60% of the maximum possible mean output voltage.  Calculate also the RMS value of load current.

5.a)	With the help of suitable connection diagram and waveforms describe the behaviour of a 3-phase bridge type fully controlled converter supplying an inductive load.  Derive the equation for the DC voltage ratio and input current distortion factor assuming the load to be purely resistive.

b)	A single phase half controlled rectifier is used to supply power to a load of impedance 5ohm from a 230V, 50Hz ac supply at a firing angle of 60o.

Calculate (i) Average value of voltage and (ii) Load current.

6. 	What is complementary impulse commutation? Describe this type of commutation with a circuit diagram and appropriate waveforms. Derive expressions for current through and voltage across commutating capacitor. Find also the circuit turnoff times for the complementary thyristors.

Contd..2

Code No. 310204			-2-					Set No.2

7.a) 	Discuss the main classification of dc to ac thyristor converters. Which of these is most commonly employed   why?

b) 	Describe the principle DC chopper operation. Derive an expression for its average dc output voltage.

8.a)	Give the difference between principle of operation of series and parallel Inverter circuits.

b)	Distinguish between different methods commonly used for forced commutation in Inverters.

###

Code No. 310204

Set. No.

3

III-B.Tech I-Semester Supplementary Examinations, June 2003

POWER ELECTRONICS

(Electrical Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	What are the necessary conditions for turning on of an SCR.

b)	Define turn on and turn off times of SCR.

2.	Discuss the two-transistor model of a thyristor.  Using this model, describe the various mechanism of turning on a thyristor.

3.	What is a Triac? Describe the different modes of operation of Triac.

4.	The SCR has maximum rms on-state current as 50A.  If this SCR is used in a 	resistive circuit, compute its average on-state current rating for conduction 	angles 	of 30o and 60o in case current waveform is

(i) half sine wave              	(ii) 	Rectangular wave.

5.a)	Distinguish between a half controlled and Fully controlled rectifiers with regard to their input and output performance.

b)	Calculate the average current through a load resistance of 100ohm when it is connected in series with a large inductance across the terminals of a:

(i)	Fully controlled single phase bridge circuit

(ii)	Half controlled single phase bridge circuit.

6.a)	Describe the operation of class E commutation circuit with appropriate waveforms.

b)	For a class D commutation circuit Vs=230V, L=20µH and C=40µF .For a constant load current of 120 A calculate the circuit turnoff times for main and auxiliary thyristors.

7.a)	Describe the operation of voltage commuted chopper with relevant circuit and voltage waveforms.

b) 	A load commutated chopper fed from a 230V, DC source, has a constant load current of 50A.  For a duty cycle of 0.4 and a chopping frequency of 2KHz computer (a) the average output voltage (b) the value of commutating capacitance.

Contd..2

Code No. 310204			-2-					Set No.3

8.a)	Explain the operation of a parallel inverter and mention its merits.

b)	Mention the purpose of feedback diodes in Inverter and condition under which 	those are not required.

###

Code No. 310204

Set. No.

4

III-B.Tech I-Semester Supplementary Examinations, June 2003

POWER ELECTRONICS

(Electrical Electronics Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	Draw thyristor gate V-I characteristics indicating clearly the gate drive limits.  Explain, with the help of these characteristics, the selection of an operating point and the choice of gate circuit parameters.

2.	The SCR has maximum rms on-state current as 50A.  If this SCR is used in a 	resistive circuit, compute its average on-state current rating for conduction 	angles 	of 30o and 60o in case current waveform is (i) half sine wave              	(ii) 	Rectangular wave.

3.	Describe the basic principle of working of a 1 to 1  step down cycloconverter for both continuous and discontinuous conductors for a bridge-type cycloconverter.

4.a)	Explain the principle of working of a three phase fully controlled converter supplying to resistive load and derive an expression for average load voltage.

b)	A three phase full converter bridge is connected to a supply voltage of 230v, 50Hz.  The source inductance is 4mH. The load current of 20A is constant and ripple free.  Determine the overlap angle when the firing angle delay is 20o.

5.a)	What is a semi converter?  Draw two semi converter circuits.

b)	What is a full converter?  Draw two full converter circuits.

6.a)	Describe class B self commutation by an LC circuit employed for a thyristor circuits.

b)	A two thyristor class C turn off circuit is required to be designed for use as a blinker to turn on and off a lamp of constant resistance of 10 from a DC supply of 100V. If the SCR used are converter grade with turnoff time 50 sec. Find the value of commutation capacitor so that the commutation failure may not occur.

Contd..2

Code No. 310204			-2-					Set No.4

7.a)	Draw the power circuit diagram of a current commuted chopper. Explain the working of a chopper by dividing its commutation process interval into well defined modes.

b) 	A step up chopper has input voltage of 220V and output voltage of 660V, if the non-conducting time of the thyristor chopper is 100 µsec, compute the pulse width of output voltage. In case pulse width is halved for constant frequency operation, find the new output voltage.

8.a)	Briefly discuss the different methods of which voltage control can be done externally in an Inverter.

b)	State why the o/p voltage in a parallel Inverter is not a pure sine wave.

###