Code. No.: 310205

Set No.

1

III B.Tech I Semester (Supplementary)Examinations June 2003

ELECTROMECHANICS-III

(Electrical and Electronics Engineering)

Time: 3 hours							     Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.	Describe the constructional features of cylindrical rotor synchronous generators and the purpose of each part.

2.	A 3 – Phase, 50 Hz cylindrical – rotor synchronous machine has the following parameters.

Self inductance per phase = 3.15mH

Armature leakage inductance = 0.35 mH. For this machine, calculate the mutual inductance between armature phase and its synchronous reactance.

3.a)	Describe synchronous impedance method to determine regulation of an alternator for Lagging power factor

b)	A 600V, 60KVA, Single-phase alternator has an effective resistance of 0.2. A field current of 10A produces an armature current of 210A on short-circuit and an e.m.f of 480V on open-circuit. Calculate:

i)   Synchronous impedance and reactance.

ii) Regulation with 0.8 P.f Lagging, and unity P.f Leading.

4.a)	Explain the method of determining the voltage regulation of an alternator by ASA method

b)	An alternator has a synchronous reactance of 20% and negligible resistance calculate its voltage regulation when working at full-load

  - 0.8 P.f Lag

  - Unity P.f and

  - 0.8 P.f Lead.

5.a)	Mention the essential parts of a synchronous motor. Explain neatly with diagrams each one of them.

b)	A 3, 220V, 50Hz, 1500rpm, mesh connected synchronous motor has a synchronous impedance of 4/phase. It receives an input line current of 30Amps. at a leading p.f. of 0.8. Find the line value of the induced emf and the load angle in mechanical degrees. If the mechanical load is thrown off without change of excitation, determine the magnitude of current under the new conditions. Neglect losses.

6.	A 6000 KVA, 5000V, 50Hz, 3 phase alternator with 4 poles and a synchronous reactance of 25% operates an constant voltage and constant frequency bus bars. The moment of inertia of the whole rotating system is 16800 kg-m2. Calculate the time of one complete oscillation for full load and unity power factor.                        								                 Contd….2

Code No.: 310205			-2-				Set.No:1

7.a)	Describe briefly the effect of varying excitation upon the armature current and p.f. of a synchronous motor when input power to a motor is maintained constant.

b)	A 400V, 50Hz, 3, 37.3KW, star connected synchronous motor has a full load efficiency of 88%. The synchronous impedance of the motor is (0.2+j1.6) / phase. If the excitation of the motor is adjusted to give a leading p.f. of 0.9, calculate for full-load (i) the induced emf (ii) the total mechanical power developed.

8.a)	“A pure single phase motor is not a self starating motor” justify the statement

b)	Why cannot a shaded pole motor be made to rotate in the reverse direction?

$$$

Code No.: 310205

Set No.

2

III B.Tech I Semester (Supplementary) Examinations June 2003

ELECTROMECHANICS-III

(Electrical and Electronics Engineering)

Time: 3 hours							     Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.	Discuss the differences in constructional features between cylindrical rotor and salient pole synchronous generators. Also explain why two types of machines are required.

2.	The flux distribution in a salient pole machine is rectangular, the base being two thirds of pole – pitch. The peak value of flux density is 1wb/m2 . Draw one complete cycle of the e.m.f induced in a single - turn coil which has the pitch of five – sixth of full pitch and moves with a uniform velocity of 30m per sec. The armature conductor length is 2m. Calculate the r.m.s. value of the voltage.

3.a)	In brief explain the circuit model of synchronous machine (cylindrical rotor). What is synchronous reactance?

b)	What is meant by regulation of an alternator? Draw the Phasor diagram of a cylindrical rotor alternator supplying Lagging P.f load. Show the positions of induced e.m.f and terminal voltage

4.	Explain the merits and demerits of EMF and MMF method. Explain what are the assumptions made in each case.

5.	Two similar 6000V, 3-phase generators are running in parallel a constant voltage and frequency bus bars. Each has an equivalent resistance and reactance of 0.05 ohms and 0.5 ohms respictively and supplies one half of a total load of 10000KW at a lagging power factor of 0.8, the two machines being similarly excited, If the excitation of one machine is adjusted until the armature current is 438 A and the steam supply to the turbine remains unchanged, find the armature current, the emf and the power factor of the other alternator.

6.a)	Discuss the effect of armature current for change in excitation.

b)	A 2,000V, 3- star-connected synchronous motor has an effective resistance and synchronous reactance of 0.2 and 2.2 respectively. The input is 800KW at normal voltage and the generated line emf is 2,500V. Calculate the line current and power factor.

Contd….2

Code No.: NR-310205			-2-				Set.No:2

7.a)	Define back-emf. Draw the equivalent circuit diagram of synchronous motor. Also deduce the expression for i) excitation voltage ii) synchronous impedance and iii) armature current.

b)	A 3-, Y-connected synchronous motor takes 48KW at 693V (line), the p.f. being 0.8 lag. The induced emf is now increased by 30%, the power input being the same. Find the new current and power factor. Synchronous impedance equals to (0+2j)/phase.

8.a)	Using double revolving field theory explain the torque-slip characteristic of a single phase induction motor and prove that it cannot produce staring torque.

b)	What is the difference between conductively compensated series motor

and an inductively  compensated series motor.

$$$

Code No.:310205

Set No.

3

III B.Tech I Semester (Supplementary) Examinations June 2003

ELECTROMECHANICS-III

(Electrical and Electronics Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.	Describe how the armature windings are arranged in synchronous generators. What is the effect distribution factor on the performance of the generator?

2.	A 3 – Phase, 12 – Pole ,500 r.p.m star connected alternator has 144 slots with 10 conductors per slot. The coils are full pitch and the flux per pole is 0.094 wb. Determine the phase and linee.m.fs. What harmonics do you expect due to slots and why?  What will be the phase voltage if the coils are connected to form a balanced two-phase winding?

3.a)	In brief explain the operation of a 3-phase cylindrical rotor alternator under constant load with variable excitation. Draw the Phasor diagram

b)	A synchronous generator has a direct axis synchronous reactance of 0.8 p.u. and quadrature axis synchronous reactance of 0.5 p.u. It is supplying full load at rated voltage at 0.8 P.f Lagging. Find the open circuit voltage

4.a)	Explain how open-circuit and short-circuit tests are conducted on a synchronous machine

b)	What is an air-gap line? In an alternator, explain why short-circuit characteristic is a straight line where as open-circuit characteristic is a curve.

5.a)	How do you calculate the time constant in case of an alternator?

b)	Show that an alternator running in parallel a constant voltage and frequency bus

bars has a natural time period of oscillation.Deduce a formulae for the time of one

complete oscillation and calculate the value for a 500KVA, 3-phase, 10000V

machine running at 1500 rpm and constant 50Hz bus bars.

The moment of Inertia of the whole moving system is 14112 Kg-m2 and the

steady short-circuit current is five times the normal full load value

6.a)	What will be the effect of increased load with constant excitation? Explain with phasor diagram.

b)	A 75KW, 400V, 4-pole, 3-, star connected synchronous motor has a resistance and synchronous reactance per phase of 0.04 and 0.4 respectively. Compute for full load 0.8 pf lead the open circuit emf/phase and gross mechanical power developed. Assume an efficiency of 92.5%.

Contd….2

Code No.: NR-310205			-2-				Set.No:3

7.a)	What is a synchronous condenser? What is the use of synchronous condenser?

b)	A 500V, 50Hz, 3- circuit takes 20A at a lagging power factor of 0.8. A synchronous motor is used to raise the power factor to unity. Calculate the KVA input to the motor, and its power factor when driving a mechanical load of 7.5KW. The motor has an efficiency of 85%.

8.a)	Explain the constructional details and principle of operation of a split phase 	  	induction motor.  List out its industrial applications.

b)	“ The centrifugal switch of a single phase motor failed to open”.  Explain the after 	effects in the performance.

$$$

Code No.: 310205

Set No.

4

III B.Tech I Semester (Supplementary) Examinations June 2003

ELECTROMECHANICS-III

(Electrical and Electronics Engineering)

Time: 3 hours							     Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

1.	Distinguish between integral slot and fractional slot winding and their merits and demerits.

2.	The field form of a star connected 3-phase machine is given by 100 sin + 30sin 3 + 10 sin5, where  is measured from the interpolar axis. If the stator has 9 slots per pole and coils are full pitched, determine the wave form of phase voltage in terms of Vmax, the amplitude of the fundamental phase voltage.  Determine also the r.m.s values of phase and line voltages. What are the phase and line voltages if the flux is sine distributed with the same total flux per pole?

3.a)	What happens to the value of synchronous reactance if air gap is increased

b)	What is the excitation voltage in case of synchronous machines?

c)	A 30KVA, 440V, 50Hz, 3-phase, Star-connected alternator gave the following 	test data:

4.	Explain the Phasor diagram of salient-pole synchronous machine

- At Lagging P.f

- At Leading P.f

5.a)	How do you calculate the time constant in case of an alternator

b)	Show that an alternator running in parallel a constant voltage and frequency bus

bars has a natural time period of oscillation. Deduce a formulae for the time of one complete oscillation and calculate the value for a 500KVA, 3-phase, 10000V

machine running at 1500 rpm and constant 50Hz bus bars.

The moment of Inertia of the whole moving system is 14112 Kg-m2 and the

steady short-circuit current is five times the normal full load value

6.a)	Define and explain different torques of a synchronous motor?

b)	A 400V, 50Hz, 6-pole, 3-, Y connected synchronous motor has a synchronous reactance of 4 / phase and a resistance of 0.5 / phase. On full-load, the excitation is adjusted so that machine takes an armature current of 60 Amperes at 0.866 p.f. leading. Keeping the excitation unchanged, find the maximum power output. Excitation, friction, windage and iron losses total 2KW.

Contd….2

Code. No.: 310205				-2				Set.No:4

7.a)	Why are synchronous condensers also called as synchronous phase modifiers? Where are these used?

b)	A substation operating at its full load of 1,000KVA supplies a load of power factor 0.71 lagging. Calculate the permissible additional load at this power factor and the rating of synchronous condenser to raise the substation power factor to 0.87 lagging.

8.a)	Show that in a shaded pole motor, the flux in the shaded part of pole segment 	lags behind the flux of the unshaded pole segment both in space and time, 	resulting a rotational torque in the motor.

b)	A capacitor connectected in series with the starting winding of a resistance start 		split phase induction motor.Explain the changes in the performance characteristics

$$$