Code No: 310302

Set No.

1

III-B.Tech. I Semester Supplementary Examinations June, 2003

HYDRAULIC MACHINERY AND SYSTEMS

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Derive an expression for the force exerted by a jet on a stationary curved plate, when the jet strikes the curved plate at one end tangentially when the plate is symmetrical.

b) 	A jet of water of diameter 10 cm moving with a velocity of 30 m/s strikes a curved fixed symmetrical plate at the center. Find the force exerted by the jet of water in the direction of the jet if the jet is deflected through an angle of 120o at the outlet of the curved plate.

2.a)	How do the rotational speed and cavitation affect the selection of a water 
turbine?

b)	What do you mean by water wheels?

3.	A hydroelectric power station requires a turbine producing 3MW at 250 rpm 	under a head of 25m. Calculate the specific speed of the turbine. Estimate the 	runner diameter if the speed ratio is 0.70 and suggest the type of the turbine and 	runner.

4.a)	Why a reciprocating pump is called a positive displacement pump? Where 	reciprocating pumps are suitable?

b)	A single acting reciprocating pump having a bore of 150mm diameter and a 	stroke 	of 300mm discharges 200 litres of water per minute at 40 r.p.m. Neglecting 	losses, 	find:

- Theoretical discharge in litres/min.

ii)   Coefficient of discharge

iii)  Slip of the pump.

5.a)	What are the different components of a centrifugal pump? Show  them with the help of a neat sketch.

b)	What is the pressure at the eye of a centrifugal pump? What is the limitation on 	this pressure?

6.a)	What are the hydraulic losses associated with centrifugal pumps?

b)	The discharge of a centrifugal pump is 0.38 m3 / sec and runs at a speed of   	1500 rpm. The head developed is 15 m. The width of the  impeller at out let is   	5 cm and the diameter is 75 cm.  The manometric efficiency is 0.8. Estimate 	the blade angle at outlet.

Contd…..2

Code No: 310302				-2-				Set No:1

7.a)	Differentiate between Hydraulic Accumulator and a Differential accumulator.

b)	The diameter of the ram of an accumulator is 500mm and the stroke is 759mm.  It 	is loaded with 1.25 MN of total weight.  If the packing friction of the ram is 0.5 	percent of the weight on the ram, calculate the power delivered to the mains, if the 	ram falls steadily at the rate of the stroke in 125 seconds and the pumps are 	delivering simultaneously at the rate of 2 m3 per minute.

8.a)	What are hydraulic systems?  Describe briefly a hydrostatic and hydrokinetic 	system and cite examples.

b)	Describe the working of Hydraulic crane with a neat sketch.

- - -

Code No: 310302

Set No.

2

III-B.Tech. I Semester Supplementary Examinations June, 2003

HYDRAULIC MACHINERY AND SYSTEMS

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Derive an expression for the force exerted by  a jet on a stationary curved plate when the jet strikes the curved plate at one end tangentially when the plate is unsymmetrical.

b) 	A plate is acted upon at its center by a jet of water of diameter 20 mm with a velocity of 20 m/s. The plate is hinged and is deflected through an angle of 15o. Find the weight of the plate if the plate is not allowed to swing.  What will be the force required at the lower edge of the plate to keep the plate in vertical position.

2.a)	What do you mean by inward and outward flow turbines? 		
   b)	Differentiate between impulse and reaction turbines. Give examples.

3.a)	Explain the terms: slip and negative slip with reference to reciprocating pump.

b)	A single acting reciprocating pump has plunger diameter of 200 mm and stroke 	length 300mm. The suction pipe is 100 mm diameter and 8 m long. The pump 	draws water 4 m below the cylinder axis at 30r.p.m. Find the pressure head on the 	piston.

    - in the middle of the suction stroke and

    - at the end of the suction stroke.

Assume f = 0.01 and atmospheric pressure head = 10.3m of water.

4.a)	Draw the out let velocity triangle for forward curved vane of a  centrifugal 	pump.

b)	How does the pressure at the eye of a centrifugal pump vary with  the fluid in 	contact with the impeller?

5.a)	What are the mechanical losses in the working of a centrifugal  pump?

b)	The impeller of a centrifugal pump has outer diameter of 40 cm  and inner 	diameter of 20 cm. The blade angle at outlet is 30. The  speed of the impeller 	is 1450 rpm. The velocity of flow at inlet and out let is same at 2.2 m/ sec. 	Find head developed if manometric   efficiency is 75%, absolute velocity at 	out let and blade angle at inlet.

Contd…..2

Code No: 310302				-2-				Set No:2

6.	A Francis turbine works at a head of 150m while running at 750 rpm. The outer and inner diameter of the runner is 1.4m and 0.85m. The water enters the runner with a velocity of 30 m/s. the outer angle of the guide blades is 10o. Estimate the runner blade angles at inlet and outlet, if the discharge is axial and velocity of flow is constant through the runner. Also calculate the hydraulic efficiency.

7.a)	Differentiate between a Hydraulic ram and a centrifugal pump.

b)	A hydraulic ram is used to lift 4 lps of water to a height of 25m using 72 lps of 	water from a supply tank 3.1m above the ram.  If the delivery pipe is 63.5mm in 	diameter and 155m long, determine the efficiencies of the ram.  Take f=0.014 for 	the pipe.

8.a)	Explain the working of a direct acting lift.

b)	Describe with neat sketch torque converter.

- - -

Code No: 310302

Set No.

3

III-B.Tech. I Semester Supplementary Examinations June, 2003

HYDRAULIC MACHINERY AND SYSTEMS

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)		Derive an expression for the force exerted on a flat vertical plate moving in the    	direction of jet.

b) 	A 15 m/s velocity jet of water 5 cm in diameter strikes perpendicularly a smooth 	flat plate. Determine the force exerted by the jet on the plate if the plate is at rest 	and if it moves in the direction of the jet with a velocity of 5 m/s. Also determine 	the work done in each case  and the efficiency of the jet in the second case.

2.a)	What are guide vanes? 		
   b)	Define flow ratio and speed ratio? 		
   c)	What are the disadvantages of an outward flow turbine?

3.	A model of a Francis turbine one-fifth of full size, develops 30.8 kW at 305 rpm, 	
	under a head of 2.5 m. Find the speed and power of full size turbine operating 	under a head of 6 m.

4.a)	What is an indicator diagram of a reciprocating pump? What information does it 	give?

- A single acting reciprocating pump has a stroke length of 150 mm. The suction 	pipe is 7.5 m long and the ratio of plunger diameter and a suction diameter is 4/3. 	The water vessel in the pump is 2.5 m below the axis of the pump cylinder and  	75 mm diameter pipe connects the sump and the pump cylinder. If the crank is 	running at 75 r.p.m. find the pressure head on the piston:

i)	In the beginning- of the suction stroke

ii)	In the middle of the suction stroke and

iii)	In the end of the suction stroke.

5.a)	Why will the centrifugal pump function only when started at or above minimum 	speed?

b)	Why is the pump called centrifugal pump? How is the centrifugal action  present 	inside the casing.?

6.a)	Explain  the leakage losses in centrifugal pumps?

b)	The impeller tips of a centrifugal pump have radii of 10 cm and 30cm. 	Determine the minimum starting speed at which delivery will commence 	against a static head of 15m.

Contd…..2

Code No: 310302				-2-				Set No:3

7.a)	Derive an expression for the efficiencies of a hydraulic ram.

b)	A low level reservoir is 5m above a certain ram installation.  The discharge from the low level reservoir to the ram is 50 litres per second.  The ram lifts water into a high level reservoir 27m above the ram level at the rate of 5 litres per second through a 60mm diameter pipe 60m long having a coefficient of friction 0.03.  Determine the efficiencies.

8.a)	Differentiate between a fluid coupling and a torque converter.

b)	State and draw the characteristics of fluid coupling and torque converter.

- - -

Code No: 310302

Set No.

4

III-B.Tech. I Semester Supplementary Examinations June, 2003

HYDRAULIC MACHINERY AND SYSTEMS

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Obtain an expression for the force on the inclined plate moving  in the direction of 	the jet .   Also get the expression for the work done per second by the jet on the 	plate.

b) 	A jet of water moving at 12 m/s impinges on a concave shaped vane to deflect the 	jet through 120o when stationary.  If the vane is moving at 5m/s, find the angle of 	jet so that there is no shock at inlet. Also compute the absolute velocity of jet at 	exit both in magnitude and direction. Assume that the vane is smooth.

2.a)	What do you mean by the statement that a turbine is discharged radially at outlet?
   b)	Differentiate between WHP and BHP of a turbine. 		
   c)	In a turbine the relative velocity and the velocity of flow at inlet are equal and 		
	in the same direction. Under what conditions this can occur?

3.	A Kaplan turbine runner is to be designed to develop 10,000 bhp. The net 	available head is 5.50 metres. Assume that the speed ratio is 2.09 and the flow 	ratio is 0.68, and the overall efficiency is 60%. The diameter of the boss is 13rd of 	the diameter of the runner. Find the diameter of the runner, its speed and its 	specific speed.

4.a)	Differentiate between

- A single acting and double acting reciprocating pumps

- A single cylinder and double cylinder reciprocating pumps.

b)	A double acting reciprocating pump running at 50r.p.m is discharging 900 litres 	of water per minute. The pump has a stroke of 400 mm. The diameter of piston is 	250 mm. The delivery and suction heads are 25m and 4m respectively. Find the 	slip of the pump and power required to drive the pump.

5.a)	Explain the difference between  (i) Manometric efficiency and(ii) Over all 	efficiency

b)	What is the function of a casing in a centrifugal pump? How does the 	pressure vary inside the casing?

Contd…..2

Code No:310302				-2-				Set No:4

6.a)	When are two pumps called homologous pumps?

b)	A centrifugal pump impeller has outer diameter of 40 cm and outer width of   	6 cm. The impeller runs at a speed of 900 rpm and the blades are curved 	backward with a blade angle of 30 at the  out let. The discharge delivered is 	0.3 m3 / sec. Determine all the velocities and angles of outlet velocity triangle.  	Sketch the velocity triangle.

7.a)	Describe with the help of a schematic diagram the construction, working and performance of a hydraulic ram.

b)	The diameter of ram of a hydraulic press is 300mm and the diameter of the plunger is 500mm.  If the force applied to the plunger is 500N, determine the weight lifted by the press.

8.a)	Sketch and explain the working of a torque converter.

b)	State the fields of application of hydraulic coupling and torque converter.  	Compare the characteristics of the two.

- - -