Code No: 310304

Set No.

1

III-B.Tech. I Semester Supplementary Examinations June, 2003

DYNAMICS OF MACHINES

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering and Mechatronics.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	What is a friction circle?  Show how many friction axes are there for a link having 	pin joint at its end.

b)	A force of 4,000 N is applied on a piston along the line of stroke of a horizontal 	steam engine.  When the crank is at 60o to I.D.C. calculate the torque on the crank 	shaft; when the length of connecting rod is 0.8 m and length of stroke is 0.4m.

2.	A vertical petrol engine 150mm diameter and 200mm stroke has a connecting rod 	350mm long.  The mass of the piston is 1.6 kg and the engine speed is 1800 rpm.  	On the expansion stroke with crank angle 30o from top dead centre, the gas 	pressure is 750 kN/m2.  Determine the net thrust on the piston.

3.a)	State and explain the following terms

(i)   Coefficient of friction  (ii)  Angle of repose (iii) Angle of friction.

b)	A lorry is moving on a level road at a sped of 36 kmph. Its centre of gravity lies at 	a distance of 0.6m from ground level.  The wheel base is 2.4m and the distance of 	centre of gravity from the rear wheels is 0.9m.  Find the distance travelled by the	car before coming to rest when brakes are applied, (i) to the rear wheels (ii) to the	front wheels & (iii) to all the four wheels.  The coefficient of friction between the 	tyres and the road surface is 0.45.

4.a)	Explain the principle of working of a screw jack and derive an expression for its mechanical advantage.

b)	In a band and block  Brake, the band is lined with 14 blocks, each of which subtends an angle of 200 at the drums  centre.  One end of the band is attached to the fulcrum of the brake lever and the other to a pin 150 mm from the fulcrum. Find the force required at the end of the lever 1m long from the fulcrum to give a torque of 4k N-m.  The diameter of the brake drum is 1m and the coefficient of friction between the blocks and the drum is 0.25.

5.	A proell governor has arms of 300 mm length.  The upper arms are hinged on the axis of rotation, whereas the lower arms are pivoted at a distance  of 35 mm from the axis of rotation.  The extension of lower arms to which the balls are attached are 100 mm long.  The  mass of each ball is 8 kg and the mass on the sleeve is 60 kg.  At the minimum radius of rotation of 200 mm, the extensions are parallel to the governor axis.  Determine the equilibrium speed of the governor for the given configuration.  What will be the equilibrium speed for the maximum radius of  250 mm.

Contd…..2

Code No:310304				-2-				Set No:1

6.	Explain terms (a) Variations in tractive effort  (b) Swaying couple

(c) Hammer 	blow as applied to locomotive balancing. Derive expressions for 	these for two 	cylinder uncoupled locomotive balancing.

7.a)	Define free vibrations with neat sketch.

- A Shaft 1.5m  long, supported in flexible bearings at the ends, carries two wheels each of 60 kg mass.  One wheel is situated at the centre of the shaft and the other at a distance 375mm from the centre. The shaft is hollow, of external diameter 75mm, internal diameter 40mm. The density of the shaft material is 7700 kg/m3.

8.	A marine engine, shaft and propeller are approximately equivalent to the 	following three rotor system. The combined moment of inertia of the engine 	masses is 800 kg.m2, that of 	the flywheel is 320 kg.m2  and that of the propeller 	is 20 kg.m2. The equivalent shaft between the engine masses and the wheel is 	5 cm diameter and 200 cm long and that between flywheel and the propeller 	is 2.5 cm diameter and 200 cm long. Find the frequencies of torsional 	vibration of the system and the positions of the nodes.

- - -

Code No: 310304

Set No.

2

III-B.Tech. I Semester Supplementary Examinations June, 2003

DYNAMICS OF MACHINES

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering and Mechatronics.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	In the figure given below the slider crank mechanism, in which the forces F1 and 	F2 are known.  Determine the torque that may be applied on the crank shaft to 	maintain equilibrium.

F1=1000 N,  F2=800 N,

AB=36 cm, OA=9cm, AS=16 cm

Angle AOB = 450.

2.	A horizontal steam engine running at 240 rpm has a bore of 300 mm and stroke 	600mm.  The connecting rod is 1.25 m long and the mass of reciprocating parts is 	60 kg.  When the crank is 60o past its inner dead centre, the steam pressure on the 	cover side of the piston is 1.125 N/mm2 while that on the crank side is 0.125 	N/mm2.  Neglecting the area of the piston rod, determine: (a). the force on the 	piston 	rod and (b) the turning moment on the crankshaft.

3.a)	Derive an expression for ratio of maximum and  minimum tensions in a   band and block brake.

b)	 Determine the axial force required to engage a cone clutch transmitting 25kW of power at 750r.p.m. Average friction diameter of the cone is 400mm, semi cone angle 100 and coefficient of friction is 0.25.  Also find the width of the friction cone.

4.a)	Define the following terms:

(i)  Friction circle    	(ii)  friction axis of a link

b)	A differential band brake acting on the ¾th of the circumference of a drum of 450mm diameter, is to provide a braking torque of 225Nm.  One end of the band is attached to a pin 100mm from the fulcrum of the lever and the other end to another pin 25mm from the fulcrum on the other side of it where the operating force is also acting.  If the operating force is applied at 500mm from the fulcrum and the coefficient of friction is 0.25, find the two values of the operating force corresponding to two directions of rotation of the drum.

Contd…..2

Code No: 310304				-2-				Set No: 2

5.	A Proell governor has all the four arms of length 250 mm.  The upper and lower ends of the arms are pivoted on the axis of rotation of the governor.  The extension arms of the lower links are each 100 mm long and parallel to the axis when the radius of the ball path is 150 mm.   The mass of each ball is 4.5 kg and the mass of central load is 36kg.  Determine the equilibrium speed of the governor.

6.a)	Explain role of reference plane in balancing masses of rotation in different planes.

b) 	Explain why two balancing weights are required to balance the weights rotating in 	different planes, compared to single balance weight required to balance weights 	rotating in one plane.

7.a)	Derive an equation for the transverse vibration of a uniformly loaded shaft.

b)	A rigid massless bar of length  L  is hinged at its end and carries a spring K2 with mass in at its right end. The bar is also supported by a spring K1 at a distance from the left hinge. Determine the natural frequency of the bar.

8.	The flywheel of an engine driving a dynamo has mass of 200 kg and had a radius 	of gyration of 30 cm. The shaft at the flywheel end has an effective length of 	25cm and is 5cm diameter. The armature mass is 225 kg  and has radius of 	gyration of 25.5 cm. The dynamo shaft has a diameter of 4.375 cm and an 	equivalent length of 20 cm. Neglecting the inertia of the shaft and coupling, 	calculate the frequency of the torsional vibrations and the position of node. The 	modulus of  rigidity for the shaft material is 80 GN/m2.

- - -

Code No: 310304

Set No.

3

III-B.Tech. I Semester Supplementary Examinations June, 2003

DYNAMICS OF MACHINES

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering and Mechatronics.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	Two Isosceles Triangles represent the turning moment diagram of an engine, the 	base of the two triangles being 0 to  and  to 2  the maximum torque being 	1000 N-m. The engine runs at 500rpm.  The fluctuation of speed is not to exceed 	1.5%.  Find the thickness of disc type flywheel require if diameter is 0.5m and 	density of material is 7.5 gm/cm3.

2.	A steam engine 200 mm bore and 300 mm stroke has a connecting rod 625 mm 	long.  The mass of the reciprocating parts is 15 kg and the speed is 250 rpm when 	the crank is at 30o to the inner dead centre and moving outwards, the difference in 	steam pressures is 840 kN/m2.  If the crank pin radius is 30 mm, determine: 	(i)	The force on the crankshaft; and (ii) 	   The torque acting on the frame.

3.a)	Derive an expression for the efficiency of screw jack (a) considering collar friction and (b) neglecting collar friction.

b)	A simple band brake is operated by a lever of length 500mm. The brake drum 	has a diameter of 500mm and the brake band embraces 5/8 of the circumference.  	One end the band is attached to the fulcrum of the lever while the other end is 	attached to a pin on the lever 100mm from the fulcrum.  If the effort applied to the 	end of the lever is 2k N and the coefficient of friction is 0.25, find the maximum 	braking torque on the drum.

4.	Derive an expression for frictional torque of a conical pivot bearing assuming

(a) uniform pressure (b) uniform wear.  How will the expression be modified, if 	the conical pivot is truncated.

5.	The arms of a Porter governor are each 250 mm long and pivoted on the governor 	axis.  The mass of each ball in 5 kg and the mass of the central sleeve is 30 kg.  	The radius of rotation of the balls is 150 mm when  the sleeve begins to rise and 	reaches a value of 200 mm for maximum speed.  Determine the speed range of the 	governor.  If the friction at the sleeve is equivalent of 20 N of load at the sleeve, 	determine how the speed range is modified.

Contd…..2

Code No:310304				-2-				Set No:3

6.a)	Prove that maximum secondary unbalanced forces is 1/n times maximum            	primary unbalanced for n cylinders reciprocating engine.

b) 	For radial engines with an odd number of cylinders prove that the primary force 	may be balanced by attaching single mass of ½ km where ‘k’ is number of 	cylinders and ‘m’ is mass of reciprocating parts.

7.a)	Define forced vibration with neat sketch.

- A beam of length 10m carries two loads of 200 kg  at distances of 3m from each end together with a central load of 1000 kg calculate the frequency of transverse vibration. Neglect the mass of the beam and take  I – 109 mm4 ANDF e=200 kN/mm2.

8.	Three rotors A,B and C having moment of inertia of 2000, 6000 and 3500 kg m2 	respectively and carried on a uniform shaft of 0.4 m diameter. The length of the 	shaft between the rotors A and B is 6m and between B and C is 32m. Find the 	natural frequency of the torsional vibration. The modulus of rigidity for the shaft 	material is 80 	GN/m2.

- - -

Set No.

4

Code No: 310304

III-B.Tech. I Semester Supplementary Examinations June, 2003

DYNAMICS OF MACHINES

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering and Mechatronics.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	The rotor of a turbine installed in a boat with its axis along the longitudinal axis of 	the boat makes 1500 rpm clockwise when viewed from the stern.  The rotor has a 	mass of 750 kg and a radius of a gyration of 300 mm.  If at an instant, the boad 	pitches in the longitudinal vertical plane so that bow rises from the horizontal 	plane with an angular velocity of 1 rad/s, determine the torque acting on the boat 	and the direction in which it tends to turn the boat at the instant.

2.	A vertical single cylinder engine has a cylinder diameter of 250mm and a stroke 	of 450mm.  The reciprocating parts have a mass of 180 kg.  The connecting rod is 	4 times the crank radius and the speed is 360 rpm.  When the crank has turned 	through an angle of 45o from top dead centre, the net pressure on the piston is 	1.05 mN/m2.  Calculate the effective turning moment on the crank shaft for this 	position.

3.a)	Explain the working of a double shoe brake with the help of a line diagram. How do you calculate the brake torque for this ?

b)	Calculate power lost in overcoming friction and number of collars required for the 	thrust bearing whose contact surfaces are 20cms internal radiaus and 15cms in 	internal radiaus.  The coefficient of friction is 0.08. The total axial load is 30kN. 	Intensity of pressure is not to exceed 3.5 bar. Speed of the shaft is 420 r.p.m.

4.a)	Derive an expression for the effort to be applied on a body for moving it down the rough inclined plane when (i) effort applied is horizontal (ii) effort applied is parallel to the plane.  Also derive an expression for the efficiency of the inclined plane.

b)	In a vertical belt transmission dynamometer the diameter of the driving pulley 	rotating at 1500 r.p.m. is 80mm.  The centre distance of the intermediate pulleys 	from the fulcrum is also 80mm  each.  The weighing pan on the lever is at a 	distance as 250mm.  Find the power transmitted when a mass of 20 kg is required 	in the pan, including its on mass.

5.	Porter governor has equal arms each 250 mm long and pivoted on the axis of 	rotation.  Each ball has a mass of 5 kg and the mass of the central load on the 	sleeve is 25 kg.  The radius of rotation of the ball is 150 mm when the governor 	begins to lift and 200 mm when the governor is at maximum speed.   Find the 	minimum and maximum speeds and range of speed of the governor.

Contd…..2

Code No:310304				-2-				Set No:4

6.a)	Distinguish reverse and direct crank methods balancing of radial engines.

b) 	Distinguish balancing of inline engines and radial engines with appropriate examples.

7.a)	Discuss the effect of inertia of the shaft in longitudinal vibrations.

- A vertical shaft 30mm diameter and 1m  long is mounted in long bearings and carries a pulley of mass 10 kg midway  between the bearings. The centre at pulley is 0.5mm from the exist of the shaft. Find  (i) the whistling speed and  (ii) the bending stress in the shaft, when it is rotating at 2000 rpm. Take youngs  modulus of the material of the shaft as 200 GN/m2.

8.	The two rotors A and B are attached to the and of a shaft 500 mm long. The mass 	of the 	rotor A is 300 kg and its radius of gyration is 300 mm. The corresponding 	values of the 	rotor B are 500 kg and 450 mm respectively. The shaft is 70 mm in 	diameter for the first 	250 mm, 120 mm for the next 70 mm and 100 mm 	diameter for the remaining length.The modulus of rigidity for the shaft 	material is 80 GN/m2. Find (a) the position of the ode  (b) the frequency of 	torsional vibration.

- - -