Code No.:310305

Set No.

1

III-B.Tech. I Semester Supplementary Examinations June, 2003

DESIGN OF MACHINE ELEMENTS

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Distinguish clearly between direct stress and shear stress.

b)	A bolt is subjected to a direct tensile load of 18000 N and a shear load of 12000N.

The yield stress of the bolt material in simple tension is 350 Mpa. Use a factor of safety of 3 and poison’s ratio of 0.25. Calculate the diameter of the bolt by:

Maximum shear stress theory and (b) Maximum principal strain theory.

2.a)	Select suitable materials for the manufacture of the following: (i) Machine tool 	spindle  (ii) valve spring  (iii) condenser tubes  (iv) connecting rod.

b)	What are alloy steels? Discuss the effect of adding different alloying elements in 	steel?

3.a)	How to mitigate stress concentration? Explain with examples.

b) 	The maximum pressure of air in the cylinder of a double acting air compressor of 	0.5 m bore is 0.9 MPa. What should be the diameter of the piston rod if there are 	no stress raisers and no column action? Take the factor of safety as 1.75. Infinite 	life is desired. Ultimate tensile strength is 1750 MPa. Take the factor for reversed 	axial loading as 0.85, size factor of 0.85 and machine surface finish factor of 0.78.

4.a)	Enumerate the different types of riveted joints.

b) 	Two plates 16 mm thick are joined by a double riveted lap joint. The pitch of each 	row of rivets is 90 mm. The rivets are 25 mm in diameter. The permissible 	stresses are 140 MPa in tension, 80 MPa in shear and 160 MPa in crushing. Find 	the efficiency of the joint.

5.a)	Determine the required length of a square key if the key and shaft are to be made 	of same material and of equal strength.

b)	Design a cotter joint to withstand an axial load varying from 30kN in tension to 	30kN in compression.  The allowable for the steel used in the joint are 60Mpa in 	tension; 75 Mpa in crushing; 48 Mpa in shear.

6.a)	Describe the design procedure of Socket and Spigot cotter joint.

b)	Design a cotter joint with two gibs to transmit an axial force of 125 kN.  The 	permissible stresses are 165 Mpa in tension; 100 Mpa in shear and 180 Mpa in 	crushing.

Contd…..2

Code No.:310305 				-2-				Set No: 1

7.a)	Explain the effect of keyway on the strength of a shaft.

b)	A shaft is required to transfer 43kW of power at 600rpm.  The outside diameter 	must not exceed 50mm and the maximum shear stress is not to exceed 70N/mm2.  	Find out the dimensions of hollow and solid shaft, which would meet these 	requirements.  Also compare their weights.

8.a)	Give the classification of couplings.

b)	Design a bushed pin type of flexible coupling required to transmit 25hp at 	1000rpm.  The coupling is to be connected between a motor and a centrifugal 	pump each having their shaft diameter of 50mm and 40mm respectively.

- - -

Code No.:310305

Set No.

2

III-B.Tech. I Semester Supplementary Examinations June, 2003

DESIGN OF MACHINE ELEMENTS

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	What is factor of safety? List the important factors that influence the magnitude of 	factor of safety?

b)	A cast iron pulley transmits 15 kW at 350 rpm. The diameter of the pulley is 	520mm and it has four straight arms of elliptical cross section in which the major 	axis is twice the minor axis. Find the dimensions of the arm, if the allowable 	bending stress is 25 MPa.

2.a)	Mention the composition of materials designated a 40C8, 40Cr1, 40C15S12 and 	30Ni4Cr1.

b)	List various advantages of Aluminium and Magnesium alloys giving their 	applications in engineering use.

3.a)	Explain the following methods of reducing stress concentration:

(i)  Removal of undesired material    	(ii)   Added grooves.

b) 	A steel connecting rod is subjected to a completely reversed axial load of 120 kN. Suggest the suitable size of the rod using a factor of safety 1.8. The ultimate strength of the material is 1000 MPa.

Load correction factor		0.7

Size factor			0.85

Surface finish factor           	0.8

4.a)	What is the difference between caulking and fullering? Explain with the help of 	neat sketches.

b) 	A double riveted double cover butt joint in plates 20-mm thick is made with 25 	mm diameter rivets at 100 mm pitch. The permissible stresses are 120 MPa in 	tension, 100 MPa in shear and 150 MPa in crushing. Find the efficiency of joint, 	taking the strength of the rivet in double shear as twice than that of single shear.

5.a)	Differentiate between a flat key and feather key.

b)	Design a cotter joint to withstand an axial load varying from 20kN in tension to 	60kN in compression.  The allowable stresses for the steel used in the joint are 50 	Mpa in tension; 70 Mpa in crushing; 40 Mpa in shear.

Contd…..2

Code No.:310305				-2-				Set No: 2

6.a)	Describe the design procedure of Sleeve and Cotter joint.

b)	Design a cotter joint with two gibs to transmit an axial force of 130 kN.  The 	permissible stresses are 165 Mpa in tension; 100 Mpa in shear and 180 Mpa in 	crushing.

7.a)	Write the comparison between solid shafts, hollow shafts and spindles.

b)	Compute the diameter of a solid shaft which has to transmit 16kW power at 	300rpm.  Ultimate shear stress per shaft material is 350 N/mm2 and factor of 	safety 	for design is 6.  If a hollow shaft replaces the solid shaft, find the 	inside 	and outside diameters if the ratio is 0.5.

8.a)	Write a short note on universal coupling.

b)	Design a solid muff coupling made of cast iron to connect two shafts transmitting 	35kW at 150rpm with a capability of 25% maximum torque greater than the 	mean torque.  The shaft and key are made of mild steel for which permissible 	shear and crushing stress are 30MN/m2 and 80MN/m2 respectively.  Permissible 	shear stress in CI is 15MN/m2.

- - -

Code No.:310305

Set No.

3

III-B.Tech. I Semester Supplementary Examinations June, 2003

DESIGN OF MACHINE ELEMENTS

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Briefly explain shear stress and shear strain?

b) 	Calculate the diameter of the solid shaft to transmit 50 kW at 180 rpm. If the 	angle of twist in a length of 4 meters is not to exceed 0.40. The allowable stress in 	the material is 70 MPa and modulus of rigidity is 84 GPa.

2.a)	How will you designate (i) plain carbon steel with 4% carbon and 0.8% 	manganese,  (ii) Plain carbon steel with a minimum yield strength of 200 N/mm2 	and (iii) Alloy steel with 0.35-0.45 % carbon and 0.9 – 1.2 % chromium.

b)	What are the advantages of copper. Discuss important copper alloys?

3.a)	Explain the following methods of reducing stress concentration:

(i) 	Drilled holes    	(ii)   Using large fillet radius.

b) 	A shaft supported as a simple beam, 0.45 mm long, is made of AIS; 3120 steel. With the shaft rotating a steady load of 8000 N is applied midway between the bearings. The surfaces are ground. Indefinite life is desired with a factor of safety of 1.6 based on endurance strength. What should be the minimum diameter of the shaft if there are no surface discontinuities? Endurance limit is 630 MPa. Size factor is 0.85 and machine surface finish factor 0.87.

4.a)	What do you understand by the term riveted joint? Explain the necessity of such a 	joint.

b) 	A double riveted lap joint is made between 15-mm thick plates. The rivet 	diameter and pitch are 25 mm and 75 mm respectively. If the ultimate stresses are 	400 MPa in tension, 320 MPa in shear and 640 MPa in crushing, find the 	minimum force per pitch which will rupture the joint.

If the above joint is subjected to a load such that the factor of safety is two, find out the actual stresses developed in the plates and the rivets.

5.a)	Sketch the keys:

(i) Wood ruff key    (ii) Kennedy key   (iii) Gib head key.

b)	Design a cotter joint to withstand an axial load varying from 50 kN in tension to 50 kN in compression.  The allowable for the steel used in the joint are 60 Mpa in tension; 75 Mpa in crushing; 48 Mpa in shear.

Contd…..2

Code No.:310305 			-2-				Set No: 3

6.a)	Describe the design procedure of Gib and cotter joint with 2 gibs.

b)	Design a knuckle joint to transmit 150 kN, with permissible stresses in tension; shear and compression are 75 Mpa; 60 Mpa and 150 Mpa respectively.

7.a)	What type of stresses are induced in shafts?

b)	A solid circular shaft is subjected to a bending moment of 3000N-m and a torque 	of 10,000N-m.  The shaft is made of 45C8 steel having ultimate tensile stress of 	700Mpa and a ultimate shear stress of 500Mpa.  Assuming a factor of safety as 6, 	determine the diameter of shaft.

8.a)	Discuss the function of coupling.  Give at least three practical applications?

b)	Design a bushed pin type flexible coupling for connecting two shafts when power 	to be transmitted = is 40 kW

Speed of the shaft = 100rpm

Diameter of the shaft = 50mm.

The bearing pressure in the rubber bush and allowable stress in the pins are to 	be limited to 0.45N/mm2 and 25N/mm2 respectively.  Sketch the coupling.

- - -

Code No.:310305

Set No.

4

III-B.Tech. I Semester Supplementary Examinations June, 2003

DESIGN OF MACHINE ELEMENTS

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering.)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	How will you designate the following varieties of materials according BIS 	system: Grey cast iron with an ultimate tensile strength of 300 N/mm2. Plain 	carbon steel with a minimum tensile strength of 320 N/mm2

b)	Briefly explain different ferrous metals and their properties.

2.a)	State the mechanical properties of metals and explain briefly.

b)	Design a hollow shaft required to transmit 12 MW at a speed of 300 rpm. The 	maximum shear stress allowed in the shaft is 80 MPa and the ratio of inner 	diameter to outer diameter 0.75.

3.a)	Explain the following methods of reducing stress concentration 	 	           	(i) Drilled holes    	(ii)  Using large fillet radius 	(iii)  Added grooves.

b) 	A shaft is made of steel [ultimate tensile strength 700 MPa and yield point 420 	MPa] is subjected to a torque varying from 200N m anti-clockwise to 600 N m 	clockwise. Calculate the diameter of the shaft if the factor of safety is 2 and it is 	based on the yield point and the endurance strength in shear.

4.a)	Explain the following terms in connection with riveted joints

(i)  Pitch   (ii) Back pitch  (iii) Diagonal pitch   (iv) Margin

b) 	A double riveted butt joint, in which the pitch of the rivets in the outer rows is 	twice that in the inner rows, connects two 16 mm thick plates with two cover 	plates each 12 mm thick. The diameter of the rivets is 22 mm. Determine the 	pitches of the rivets in the two rows if the working stresses are not to exceed the 	following limits:

Tensile stress in plates = 100 MPa, Shear stress in rivets = 75 MPa and bearing 	stresses in rivets and plates = 150 MPa.

Make a fully dimensioned sketch of the joint showing atleast two views.

5.a)	Prove that a square key is equally strong in shear and compression.

b)	Design a cotter joint to withstand an axial load varying from 60 kN in tension to 	60kN in compression.  The allowable for the steel used in the joint are 60 Mpa in 	tension; 75 Mpa in crushing; 48 Mpa in shear.

6.a)	Describe the design procedure of Gib and cotter joint with one gib.

b)	Design a knuckle joint to transmit 150 kN, with permissible stresses in tension; 	shear and compression are 75 Mpa; 60 Mpa and 150 Mpa respectively.

Contd…..2

Code No.:310305				-2-				Set No: 4

7.a)	How is the shaft  designed when it is subjected to twisting moment only?

b)	Design a shaft to transmit power from an electric motor to a lathe headstock 	through a pulley by means of a belt drive.  The pulley weighs 200N and is located 	at 100mm from the centre of the bearing.  Diameter of the pulley 200mm.  	Maximum power transmitted is 1.5HP at 120 rpm.  Angle of lap of belt 1800.  	Coefficient of friction between belt and pulley 0.3.  Shock factor in bending 1.5 	shock factor in twisting 2.0.  Allowable shear stress in the shaft 35N/mm2.

8.	Design a marine type flange coupling to transmit 500HP at 150 rpm.  Allowable 	stress for the steel used are 70N/mm2 in tension, 140N/mm2 in compression and 	50N/mm2 in shear.

- - -