Code No: 310401

Set No

1

III-B.Tech. I Semester Supplementary Examinations June, 2003

ELECTRONIC MEASUREMENTS AND INSTRUMENTATION

(Electronics and Communication Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Explain terms : i) Significant figures ii) Conformity.

b)	Discus main differences between accuracy and precision.

2.a)	What is meant by voltmeter sensitivity? Explain its relevance in circuit applications. What is meant by loading effect? What circuit arrangement is done to avoid the same?

b)	It is desired to measure the voltage across the 100K resistor in the circuit given below. Two voltmeters are available for this measurement. Voltmeter 1 with a sensitivity of 1000/V and voltmeter 2 with a sensitivity of 20,000/V. Both meters are used on their 50V range. Calculate i) the reading of each meter ii) error in each reading, expressed as a percentage of the true value.

V

200K

100K

150V

+

-

3.a)	What is trigger level errors and suggest the means to obtain the maximum accuracy.

b)	Explain how to extend the frequency range of a frequency counter using heterodyne frequency converter.

4.a)	Distinguish between asynchronous and synchronous counters

b)	Explain the importance of open collector outputs for interfacing seven segment LED displays.

5.a)	Derive an expression for the sensitivity of a wheatstone Bridge with equal arms

b)	Wheatstone bridge has ratio arms of 1000  & 120 , and it is employed to measure a resistance of 40  connected adjacent to the 120  resistor . The bridge excitation is 5 V dc .Two galvanometers G1 & G2 are available with the following specifications :

G1 	: Sensitivity = 0.008 A / div ,    Resistance  = 50 

G2 	: Sensitivity = 0.0005 A / div ,  Resistance  = 600 

Determine which galvanometer is more sensitive to the unbalance in the bridge.

Contd…2

Code No: 310401				-2-			    	        Set No.1

6.a)	Draw and discuss the spectral displays of various modulations using Spectrum analyzer.

b)	Discuss the following display modes of dual-trace CRO. 				(i)	A and B chopped

(ii)	A and B Alternate

7.a)	Write notes on semiconductor thermometer.

b)      Explain the law of intermediate temperature in thermocouple.

c)      What are Bimetallic thermometers?

8.a)	Explain the operation of magneto elastic accelerometer.

b)	What is the arrangement to measure displacement using Hall effect transducer?

^^^

Code No: 310401

Set No

2

III-B.Tech. I Semester Supplementary Examinations June, 2003

ELECTRONIC MEASUREMENTS AND INSTRUMENTATION

(Electronics and Communication Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Define the following

i)   Accuracy

ii)  Error

iii) Linearity

iv) Precision.

b)	How are random errors are analyzed? Explain with examples.

2.a)	Draw the basic circuit of an a.c. voltmeter using rectifier and d’Arsonval movement.

b)	Convert the given d’Arsonval meter to an a.c. voltmeter whose coil resistance is 250 and full scale deflection is current of 1mA. The applied a.c. voltage is from 0-250V. Calculate ohms/volt of the a.c. voltmeter. Show the circuit arrangement.

3.a)	Explain gating errors and time base errors.

b)	Explain the zero crossing detector and explain how it is used for frequency measurement.

4.a)	Give a block diagram of a digital tachometer and explain briefly.

b)	Explain the operation of a digital phase meter.

5.a)	Explain how a Kelvin’s double bridge  can accurately measure low resistances. Also derive the condition for balance..

b)	A four terminal resistor of approximately 50  resistance was measured by means of a Kelvin’s double bridge having the following component values :

Standard resistance = 100.03 ,  inner ratio arms = 100.31   and 200  , Outer ratio arms = 100.25  and 200  . Resistance of the link connecting the standard and the unknown resistor  = 700 .  Calculate the unknown resistance to the nearest of 0.01.

6.a)	Draw the simplified block diagram of the Sampling Oscilloscope and explain.	Draw the waveforms pertinent to the operation.

b)	Explain the significance of the following Lissajous figures.

(i)	Straight line

(ii)	Ellipse

(iii)	Circle

Contd…2

Code No: 310401				-2-			    	        Set No.2

7.a)	Explain the operation of radiation type temperature measuring devices.

b)      Explain the operation of infrared pyrometer.

c)      Explain the operation of disappearing filament pyrometer.

8.a)	Explain variable area capacitive transducer for measuring linear displacement.

b)	Show the measurement of liquid level using capacitive transducers.

^^^

Code No: 310401

Set No

3

III-B.Tech. I Semester Supplementary Examinations June, 2003

ELECTRONIC MEASUREMENTS AND INSTRUMENTATION

(Electronics and Communication Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Classify errors and explain them.

b)	Suggests methods to minimize and eliminate different types of errors.

2.a)	How a basic d’Arsonval movement is converted in to multi range volt meter. Explain it using neat diagram.

b)	A basic d’Arsonval movement with an internal resistance Rm=50 and full scale current Ifsd=0.5mA is to be connected in to a multi range d.c. voltmeter with voltage ranges of 0-10V, 0-50V, 0-250V and 0-500V. Show the arrangement with the help of neat diagram with values of resistances used.

3.a)	Explain the necessity to measure the period of a input signal.

b)	Draw the circuit and explain how the period is measured for a signal.

4.a)	Distinguish between time and phase measurements.

b)	Explain the operation of a very low frequency comparator system?

5.a)	Discuss the principles of resonance methods used for the measurement of components at high frequencies.

b)	A capacitance of 300pf  produces resonance with a coil at a frequency of (2/)*106Hz, while at the second harmonic of this frequency, resonance is   produced by a capacitance of 60pf. Calculate i) self-capacitance and ii) coil inductance. Neglect the effect of voltmeter capacitance and other stray capacitances.

6.a)	Draw the block diagram of a basic horizontal  amplifier and explain it.

b)	Write short notes on the synchronization of the sweep.

7.a)	What are the different types of nuclear radiations?

- Explain Geiger Muller tube to measure α , β and γrays.

8.a)	Explain the variation of dielectric constant in capacitive transducers for 	measurement of displacement.

b) 	Show the differential arrangement of capacitors to measure 	displacement.

^^^

Code No: 310401

Set No

4

III-B.Tech. I Semester Supplementary Examinations June, 2003

ELECTRONIC MEASUREMENTS AND INSTRUMENTATION

(Electronics and Communication Engineering)

Time: 3 hours						                  Max.Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Bring out main difference precision and error.

b)	How do you explain accuracy? Give examples.

2.a)	What is Ayrton Shunt, describe it with a neat sketch. Specify its applications.

b)	Design a universal Ayrton shunt to provide an ammeter with a current range of 2A, 5A, 10A using a d’Arsonval movement with an internal resistance Rm=50 and full scale deflection current of 1mA.

3.a)	What are the different types of extending frequency range of a frequency counter?

b)	Explain any two methods generally used to extend the frequency range of a frequency counter.

4.a)	Explain the working of decade counter?

b)	What is an electronic counter? How can it be used for the measurement of the following:

i)	Time interval mode		  ii)	Totalising

5.a)	A sheet of Bakelite 4.5 mm thick is tested at 50 Hz between electrodes 0.12 m in diameter. The Schering bridge employs a standard air capacitor C2 of 106 pF capacitance, a non-reactive resistance R4  of 1000  in parallel with a variable capacitance C4  and variable resistance R3. If balance is obtained with C4 = 0.4 F and R2 = 400 W.Calculate the capacitance, PF, and relative permittivity of the sheet.

b)	Explain the differences in balancing dc and ac   bridges.

6.a)	Draw the block diagram of vertical amplifier and explain its working.

b)	Draw the block diagram of dual-trace CRO and explain it.

7.a)	Explain any five types of pressure sensitive primary devices.

b)      Explain spiral type Bourdon tube.

- Explain the operation of moving magnets type transducer.

8.a)	Draw the different forms of metal foil strain gauges and explain their 	principle of operation

b)      What are the materials used in making strain gauges?

c)      What are the different types of strain gauges?

^^^