Code No: 310402

Set No

1

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

ANTENNA AND WAVE PROPAGATION

(Common to Electronics and Communication Engineering, Electronics and Telematics)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Find the magnetic vector potential at (0,0,h) due to a surface current of density Jox A/m flowing in the region x2 + y2  a2 , Z=0. Specialize your answer for

a  . Frequency =w.

b)	At the point p (10, 20, 4) in free space the magnetic vector potential A=6e j0.6 Zweb/m. What is the value of 2A at a frequency of 100 MHz?

2.a)	From basics derive the radiation resistance of a half wave dipole.

b)	Define directivity and find the same for a quarter wave monopole.

3.a)	State principle of pattern multiplication and using this obtain the radiation pattern of a four-element binomial array, d = λ2  = 00.

b)	Explain the effect of earth on the radiation pattern.

4.a)	A line source along the Z-axis has the current distribution I(Z) = Io xep(-KZ) for OZL and I(Z) = 0 otherwise. Find an expression for the far-field and explain the effect of varying k on the radiation pattern.

b)	Draw the sketch of a Rhombic antenna and write its important features.

5.a)	What is the equivalent circuit of a slot cut in the narrow wall of a wave guide. How many types of slot arrays are there? How do you select slot inclination and spacing? Write qualitatively.

b)	Explain the method of illuminating parabolic dish antennas. What is the importance of case grain system?

6.a)	What are the precautions to be taken while making gain measurement of antennas?

b)	How do you measure antenna impedance at high-frequency band?

7.a)	Explain the terms i)	Critical frequency	ii)	Vitual height	iii)	Secant law	iv)	MUF	v)	skip distance	vi)	ordinary and extraordinary waves-as pertinent to sky wave propogation.

b)	Write a note on duct propagation.

Contd…2

Code No: 310402				-2-				        Set No.1

8.	Write short notes on THREE:

a)	Space wave propagation

b)	Yagi-uda antenna

c)	Baluns

d)	Effect of earth’s curvature on space wave

e)	Lense antennas.

^^^

Code No: 310402

Set No

2

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

ANTENNA AND WAVE PROPAGATION

(Common to Electronics and Communication Engineering, Electronics and Telematics)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	State and prove antenna theorems.

b)	Define: effective length, directivity, effective aperture area and radiation resistance with reference to an antenna.

c)	A thin dipole is /15 along. If it has a loss resistance of 1.5. Calculate:

i)   Directivity			ii)  Gain		iii)	Effective aperture		iv) radiation resistance	iv)  beam solid angle

2.a)	What are the advantages and disadvantages of a traveling wave antenna?

b)	Explain the salient features of rhombic antenna and Yagi-Uda antenna.

c)	Explain the principle of operation and applications of a folded dipoles.

3.a)	Derive an expression for the radiation resistance of a square loop antenna.

b)	Find the diameter of a dish antenna that will form a beam having 0.5 half power beam width at 8.2 GHz. Assuming 60% efficiency. Calculate antenna gain and effective aperture.

4.a)	Describe an anechoic chamber. What are its advantages?

b)	With suitable diagram explain gain and half power beam width measurement of an antenna.

5.a)	Obtain an expression for the field strength of a space wave signal.

b)	Describe in detail the structure of an ionosphere and obtain the expression for the critical frequency of ionospheric layer.

6.a)	Show that the directivity of a broadside array of two identical isotropic-in-phase point sources spaced at a distance d is given by

D = 21+1βdsinβd

b)	Distinguish between a broadside array and end fire array.

c)	How is plane wave front obtained in horn antenna and reflector antenna?

7.a)	Find the gain and effective aperture area, effective length of a half wave dipole.

b)	What is traveling wave antenna? Derive an expression for the Efield of a TWA.

Contd…2

Code No: 310402				-2-				        Set No.2

8.	Write notes on:

a)	Shunt fed antennas

b)	Parabolic elements

c)	Factors involved in the propagation of radio waves.

^^^

Code No: 310402

Set No

3

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

ANTENNA AND WAVE PROPAGATION

(Common to Electronics and Communication Engineering, Electronics and Telematics)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Show that the directivity of a half wave dipole is 1.64.

b)	What is Lorentz gauge condition? What is its significance?

2.a)	Define gain, directivity, effective area and beam width of an antenna.

b)	Show that the terminal impedance of the antenna is same whether used as a receiving or transmitting antenna.

c)	The radiation intensity of a particular antenna is given by  (, ) = sin2. Calculate the directivity of the antenna.

3.a)	Explain the concept of pattern multiplication taking binomial array as an example.

b)	Obtain the expression for principle maximum and width of the principal labe of a uniform end fire array of ‘n’ elements.

4.a)	Draw a neat sketch of Rhombic antenna and explain its working.

b)	Compare the performance of an antenna with standing wave excitation and with traveling wave excitation. Give examples.

5.a)	What are parasitic elements? Explain the working principle of Yagi Uda antenna.

b)	What is an optimum horn? Draw a neat sketch of E plane sectoral horn. Discuss the application and advantages of horn.

6.a)	With necessary block diagram. Explain how radiation pattern and gain of an antenna can be measured.

b)	Derive the expression for minimum distance at which antenna pattern should be measured.

7.a)	Explain ‘ground wave propagation’ of radio waves.

b)	Determine the maximum electron density in F2 layer if the maximum usable frequency of the layer for an angle of incidence of 250 is given as 18 GHz.

8.	Write short notes on the following:

a)	Helical antenna.

b)	Effect of earth on radiation patterns.

c)	Slot antenna.

d)	Duct propagation.

^^^

Code No: NR-310402

Set No.

4

III-B.Tech. I Semester Supplementary Examination, June 2003

ANTENNA AND WAVE PROPAGATION

(Common to Electronics and Communication Engineering, Electronics and Telematics)

Time : 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What is retarded potential? Give the expressions for electric and magnetic potentials and explain their significance.

b)	Obtain the relation for magnetic and electric fields due to a small, alternating current element and hence define near field.

2.a)	Distinguish clearly between Near field and Far field of an antenna and derive the expressions necessary from the fundamentals.

b)	An antenna has effective height of 100 m, with a current at base being 400 amps(rms) at 40Hz. Find the antenna efficiency.

3.a)	An array consists of three collinear /4 dipoles with  spacing. Determine gain and beam width of the major lobe between 3dB points in the plane containing the array.

b)	What is a parasitic element? Where are they used?

4.a)	Explain how a rhombic antenna is formed by a V antenna and inverted V antenna.

b)	An endfire array composed of /2 radiatiors with axes at right angle to the line of the array required to have a power gain of 20. Determine the array length and width of major lobe between the nulls. Derive the formula used.

5.a)	Derive an expression for the radiation resistance of a square loop antenna .

b)	Find the diameter of a dish antenna that will form a beam having   0.50 half power beam width at 8.2 GHz. Assuming 60% efficiency, calculate antenna gain and effective aperture.

6.a)	Explain the gain measurement of antenna by direct comparison method.

b)	Explain the absolute method for measuring the gain of an antenna.

7.	Discuss the advantages and disadvantages of communication at ultra high frequencies. How does the field strength of UHF signals depend on the heights of the transmitting and receiving antennas.

8.	Write short notes on

a)	Whistlers		b)	Magneto ionic equation

c)	Fading of radio wave	d)	Super refraction

e)	Line of sight propagation.

$$$