Code No. 310403

Set. No.

1

III-B.Tech I-Semester Supplementary Examinations, June 2003

T V Engineering

(Electronics and Communication Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	What do understand by interlaced scanning? Show that it reduces flicker and conserve bandwidth.

2.	What is meant by equal vertical and horizontal resolution? Derive an expression for the highest modulating frequency in television system and show it is nearly 5MHz in the 625-B monochrome system.

3.a)	Write short notes on silicon diode array vidicon with neat sketch.

b)	Bring out the advantages of Silicon diode array vidicon.

4.a)	What are the various types of Image orthicons used?  Distinguish between them.

b)	Give the basic constructional diagram of a 3-inch, non-field mesh Image orthicon 	and explain its working.

5.a)	Draw the scheme of the visual exciter of TV transmitter and explain the function.

b)	Give the Aural exciter section of the T V transmitter and explain.

6.	Explain the operation of the following with neat sketch

a) Fan Dipole UHF antenna

b) Parabolic antenna

c) Balun.

7.a)	Define hue, saturation ,frequency interleaving and color  sub carrier.

b)	Draw the block diagram of a PAL encoder, and explain its working.

8.a)	List the factors that influence the choice of sub carrier frequency for color TV              system?

b) 	Describe the manner in which the APC circuit  functions to keep the reference             sub carrier frequency constant in the PAL system?

###

Code No. 310403

Set. No.

2

III-B.Tech I-Semester Supplementary Examinations, June 2003

T V Engineering

(Electronics and Communication Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Why is vertical scanning necessary in addition to the horizontal line scanning?

b)	Calculate the time of one horizontal scanning line for

i)  Frames repeated at 60Hz with 520 lines per frame for progressive scanning   without interlacing.

ii)  Frames repeated at 25Hz with 625 interlaced lines per frame.

2.	Justify the choice of 625 lines for TV transmission. Why is the total number of lines kept same in all television systems? What is the significance choosing the number of lines as 625 and not 623 or 627?

3. 	Bring out the differences between the principle of operation and characteristics of different camera tubes.

4.a)	What are the functional parts of Plumbicon video camera?  Explain clearly the working of each part with the help of its constructional diagram.

b)	Explain clearly about photo conductive lab in plumbicon.

5. 	Write briefly on

(a)	Co-channel Interference

(b)	Adjacent channel Interference

(c)	Short interference.

6.a) 	Explain the basic operation of Diode detection with suitable circuit diagram and waveforms, also discuss the choice of RL , C and diode of the circuit.

b) 	Explain the following with circuit diagram and response curve

- Series trap circuit

- Parallel trap circuit.

7.a) 	Why is the  (G-Y) difference signal not chosen for transmission ?  Explain how it             is obtained in the receiver for modulating the corresponding beam  of the picture            tube.

b) 	In which picture tube vertical raster shape correction is not necessary? Why?

8.a)	State the color processing functions carried out by a typical chroma             decoder IC.

b) 	If white light shines on a body and none is absorbed then what is the color of the             body?

c) 	Which color have the greatest brightness. Why?

###

Code No. 310403

Set. No.

3

III-B.Tech I-Semester Supplementary Examinations, June 2003

T V Engineering

(Electronics and Communication Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Why are blanking pulses used in the video signal

b)	Name the two signals transmitted in color television.

c)	A picture has 450 horizontal and 325 vertical picture elements. Calculate the total number of details in the picture.

2.	Draw a picture frame chart showing the total number of active and inactive lines during each field and establish the need for terminating the first field in half line and the beginning the second at the middle of a line at the top.

3.a)	Bring out the drawbacks in vidiocon camera tube.

b)	How do you rectify the following drawbacks in Vidicon camera tube?

(i) Image Lag	(ii) Dark Current	(iii) Image Burn	(iv) Resolution.

4.	Bring out the differences in constructional details of camera tubes of color and monochrome TV?

5.a)	Draw the block diagram of a Monochrome TV transmitter and explain the function of each block.

b)	What is IF Modulation explain the criteria of selection of IF frequency.

6.	What are the different types of Tuners? Explain the electrical design Techniques used for UHF and VHF tuners, discuss merits and demerits.

7.a)	What are the features of Color TV Systems that to make it compatible with              Monochrome systems?

b) 	What is the need for Pin Cushion correction? Explain how it is done?

8.a)	How are the luminance and color difference signals produced in the output of a              color camera?

b) 	Explain the corrections that are normally made to achieve almost distortion less             reproduction of color on the screen of a PIL tube.

###

Code No. 310403

Set. No.

4

III-B.Tech I-Semester Supplementary Examinations, June 2003

T V Engineering

(Electronics and Communication Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define aspect ratio, contrast, brightness and resolution.

b)	How is flicker eliminated by using interlaced scanning?

c)	Show calculations for the horizontal blanking time of 10.2 s as 16 percent of H.

2.	What do you understand by interlace error and how does it affect the quality of the picture? Calculate the percentage interlace error when the second field is delayed by 8s. Retrace time may be assumed to be negligible.

3.a)	Bring out the drawbacks of plembicon.

b)	Suggest remedies for different problems of plembicon.

4.a)	Discuss clearly the constructional modifications of silicon diode vidicon over conventional vidicon and explain the working of silicon diode vidicon.

b)	Give the parts of camera optics and explain their working.

5.a)	Give the block diagram of PAC system of TV transmitter and explain the various differences between Monochrome TV transmitter.

b)	Explain the readability of vestigial side band transmission for TV video transmission.

6.	Draw and explain the various functional blocks of TV receiver.

7.a) 	Explain  the main characteristics of the human eye  with respect to the perception             of colors.

b)	What is Y signal? How it is composed? What are the two major components of              the total color signals?

8.a)	Why is the color signal bandwidth  requirement  much  less  than that of the  Y             signal?

b) 	Draw the block diagram of a NTSC encoder and explain its working.

c) 	Why convergence is required for color picture tubes?

###