Code No. 310405

Set. No.

1

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL COMMUNICATIONS

(Common to Electronics and  Communication Engineering and Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	State and prove Sampling Theorem as applicable to band limited signals.

b)	Find the minimum sampling rate and Nyquist interval for the signals represented by – (i) m(t), and (ii) m2(t), where m(t) = Sin(300πt)πt.

2.	Define the band pass sampling theorem.  Verify the signal recovery for the spectrum X(f) of an ideally sampled signal shown in Fig. for the three cases, when (i) fs = 20KHz (ii) fs = 30KHz and (iii) fs = 40KHz.

X(f)

A

f

-20         -10        0                 10       20	  (KHz)

Sketch and explain the spectrum, bringing out the aliasing effects if any, in each case.

3. a)	What is meant by Inter Symbol Interference (ISI)?  And how it differs from Cross-talk in the PAM?

b)	What is the ideal solution to obtain zero ISI and what is the disadvantage of this solution?

4. a)	Explain with neat block diagram, the operation of a Continuously Variable Slope delta Modulator.

b)	What is the function of a predictor in a DPCM system.

5. a)	The Gaussian distributed random variable with zero mean and unit variance is applied to the input of uniform quantizer.

(i)	What is the probability that the amplitude of the input lies outside the range +/-4V.

(ii)	Using the result of (i) find out the Signal to quantization ratio.

b)	What is the maximum power that may be transmitted without slope overload distortion in DM.

Contd…2

Code No. 310405				.2.				Set No. 1

6. a)	Explain with the neat block diagram

- Coherent binary FSK transmitter and

- Coherent binary FSK receiver.  Also explain single space diagram for coherent binary FSK, system.

b) 	Give the comparison for FSK and PSK system

7.	Explain about the syndrome calculation, error correction and error detection in   (n,k) cyclic codes.

8.	Write note on the following error control techniques.

- Parity check bits

- Block codes

###

Set. No.

2

Code No. 310405

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL COMMUNICATIONS

(Common to Electronics and  Communication Engineering and Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Define and distinguish between PAM and PTM schemes.  Sketch and explain their waveforms for a single tone sinusoidal input signal.

b)	For the modulating signal m(t) = 2Cos 100t + 18 Cos 2000t, determine the allowable sampling rates and sampling intervals.

2.	A band limited signal x(t) has the spectrum shown in Fig.  Under ideal sampling conditions, sketch and explain the spectrum of the sampled signal, when- (i)fs=2Hz (ii) fs = 15Hz and (iii) fs = 25Hz.

Comment on signal recovery and distortion problems.

X(f)

A

F(Hz)

-10               0                  10

3. a)	Explain how the residual effect of the channel is responsible for Inter Symbol Interference.

b)	What is the practical solution to obtain zero ISI?

4. a)	Prove that the mean value of the quantization error is inversely proportional to the square of the number of quantization levels.

b)	What are the problems encountered in Linear delta modulation.

5. a)	In the binary PCM system, find out the minimum number of bits required so that quantizing noise is less than +/-k percent of the analog signal.

b)	What is Slope Overload and Granular noise distortion in DM and how it is removed in ADM.

6. a)	Derive an expression for probability of bit error of a binary coherent FSK receiver

b) 	Explain coherent and non-coherent detection of binary FSK waves.

Contd…2

Code No. 310405				.2.				Set No. 2

7. a)	Draw and explain a decoder diagram for a (7,4) majority logic code whose generator polynomial g(x) = 1+x+x3.

b) 	What are the advantages and disadvantages of convolutional codes over block codes?

8.	Discuss about the following with suitable examples.

a)	Hamming codes

b)	Interlaced codes

###

Code No. 310405

Set. No.

3

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL COMMUNICATIONS

(Common to Electronics and  Communication Engineering and Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Explain the basic principles of sampling, and distinguish between ideal sampling and practical sampling.

b)	A band pass signal has a centre frequency fo and extends from fo – 5KHz to         fo + 5KHz.  It is sampled at a rate of fs = 25KHz.  As fo varies from 5KHz to 50KHz, find the ranges of fo which the sampling rate is adequate.

2.	As applicable to pulse modulations systems, write short notes on the following:

a)	Nonlinear aspects of PTM signals.

b)	Single polarity and double polarity PAM

c)	Sampling Theorem in frequency domain.

d)	TDM aspects of T1 system.

3. a)	In decreasing the ISI, what is the practical solution and what is the transmission Bandwidth requirement?

b)	Explain about three tap preset equalizer.

4. a)	Explain the working of delta modulation system with neat block diagram.

b)	Clearly bring out the difference between granular noise and slow over load error.

5.	A Delta modulator system is designed to operate at five times the Nyquist rate for a signal with 3kHz bandwidth.  Determine the maximum amplitude of a 2kHz input sinusoid for which the delta Modulator does not have slope overload.  Quantization step size is 250mv.  Derive the formula used.

6. a)	What are power spectra?  Explain power spectra of binary PSK and FSK signals along with graphs

b) 	Give the comparision of M-ary digital modulation techniques.

7.	What is an interlaced code? Explain about its capability for correcting errors with suitable example.

8.	Write about the following techniques with suitable diagrams.

- Cyclic codes

- Convolutional codes

###

Code No. 310405

Set. No.

4

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL COMMUNICATIONS

(Common to Electronics and  Communication Engineering and Electronics and Telematics)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Compare the characteristics of PWM and PPM signals.  Explain how the PPM signals can be generated and reconstructed through PWM signals.

b)	Which analog pulse modulation can be termed as analogous to linear CW modulation and why?

2.	A band pass signal has the spectrum shown in Figure. Establish the sampling theorem, when fs = 25KHz, 45KHz, and 50KHz.  Sketch the spectra of ideally sampled signals in each case.  How can the signal be reconstructed?

A	X(f)

f(KHz)

-25         -15        0                 15       25

3. a)	Why the overall transfer function of the duo-binary filter is called as Half-cycle cosine function.

b)	Find the impulse response of duo-binary filter.

4. a)	Explain the working of DPCM system with neat block diagram.

b)	Compare PCM and DPCM system.

5. a)	Derive the relations for signaling rate and transmission Bandwidth in PCM system.

b)	Explain the terms: (i) Sampling	(ii) Quantization

(iii) Nyquist rate    (iv) Quantization error	.

6. a)	It is proposed to implement a matched filter in the form of a tapped-delay line filter with a  set of tap weights {Wk}, K=0, 1, ……K.  Give a signal S(t) of duration T seconds to which the filter is matched, find the value of Wk.

- Show that the input-to-output SNR gain of a matched filter depends on the product of the input signal duration and noise bandwidth.

7.	What is meant by random errors and burst errors? Explain about a coding technique which can be used  to correct both the burst and random errors simultaneously.

Contd…2

Code No. 310405				.2.				Set No. 4

- Briefly discuss about the following error control techniques.

- Linear block codes

- Cyclic codes.

###