Code. No.: 310801

Set No.

1

III B.Tech I Semester (Supplementary) Examinations June 2003

MECHANICAL UNIT OPERATIONS

(Chemical Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

| 1.a) | Give an account of the characterization of solid particles. |
| --- | --- |
| b) | The screen analysis shown below applies to a sample of crushed quartz. The density of the particles is 2650 kg/m3 and the shape factors are: a = 2 and Φs=0.571. For the material between 4 mesh and 200 mesh in particle size calculateDs and Dw. |
|  | Mesh no.; Dpi, mm; Mass fraction, Xi; Mesh no.; Dpi, mm; Mass fraction, Xi. 4; 4.699; 0.0000; 35; 0.417; 0.9616. 6; 3.327; 0.0251; 48; 0.295; 0.9718. 8; 2.362; 0.1501; 65; 0.208; 0.9795. 10; 1.651; 0.4787; 100; 0.147; 0.9853. 14; 1.168; 0.7278; 150; 0.104; 0.9894. 20; 0.833; 0.8868; 200; 0.074; 0.9925. 28; 0.589; 0.9406; Pan; ---; 1.0000. |
| 2.a) | How is the mixing index for a mixture of solids estimated? Explain. |
| b) | What is the effect of mixing time on mixing index for granular solids? Discuss briefly. |
| c) | Write the equation for the rate of mixing in terms of the mixing index Is and thereby derive the equation to calculate the time required for any desired degree of mixing. |

3.a)	What is meant by average size of particles?  State various method of averaging the particle size.

b)	Define the term ‘specific surface’. Obtain an expression for its computation for a mixture of particles.

c)	Modify above expression for cumulative analysis of solid particles.

4.a)	Define and discuss filtration.

b)	Name the broad classifications of liquid-solid filters and discuss them in detail

5.a)	Discuss in detail the principles involved in the phenomena of mixing.

b)	Describe with a neat sketch any of the mixers used for pastes and plastic masses.

6.a)	Explain in detail MSMPR crystallizer.

b)	Describe population density function.

Contd….2

Code. No.: 310801				-2-				Set. No: 1

7.a)	Describe with neat sketches the types of solid- solid mixers used in common.

b)	With neat diagram, show the details of a turbine agitated tank high lighting the dimensions in terms of the impeller diameter.

8.a)	Explain the principle and operation of centrifugal separators.

| Concentration (kg water/kg solid) | 5 | 4.2 | 3.7 | 3.1 | 2.5 |
| --- | --- | --- | --- | --- | --- |
| Rate of sedimentation (mm/sec) | 0.17 | 0.1 | 0.08 | 0.06 | 0.042 |

b)	A slurry containing 5 kg of water per kg of solids is to be thickened to a sludge containing 1.5 kg of water per kg of solids in a continuous operation. Sedimentation tests yielded the following results. 								Calculate the minimum area of a thickener to effect the separation of 0.6 kg of solids per second.

$$$

Code. No.: 310801

Set No.

2

III B.Tech I Semester (Supplementary) Examinations June 2003

MECHANICAL UNIT OPERATION -

(Chemical Engineering)

Time: 3 hours							     Max. Marks: 80

Answer any Five questions

All questions carry equal marks

- - -

| 1.a) | Write a brief note on standard screen series and screen analysis |
| --- | --- |
| b) | The screen analysis shown below applies to a sample of crushed quartz. The density of particles is 2650 kg/m3 and the shape factors are: a = 2 and Φs=0.571. For the material between 4 mesh and 200 mesh in particle size calculate Dw, and Ni for the 150/200 mesh increment. |
|  | Mesh no.; Dpi, mm; Mass fraction, Xi; Mesh no.; Dpi, mm; Mass fraction, Xi. 4; 4.699; 0.0000; 35; 0.417; 0.0210. 6; 3.327; 0.0251; 48; 0.295; 0.0102. 8; 2.362; 0.1250; 65; 0.208; 0.0077. 10; 1.651; 0.3207; 100; 0.147; 0.0058. 14; 1.168; 0.2570; 150; 0.104; 0.0041. 20; 0.833; 0.1590; 200; 0.074; 0.0031. 28; 0.589; 0.0538; Pan; ---; 0.0075. |
| 2.a) | What are the distinctive properties of particulate masses? |
| b) | Explain how free flowing solids flow out of bins. |

3.a)	Give general considerations for the selection of a machine for size reduction of a feed to a product of desired size.

b)	Discuss the effect of moisture content and size of feed particles on communition.

c)	Suggest machine for communition of

- big lumps of hard rock ,and

- fibrous materials.

4.a)	Explain screening.

b)	Discuss with suitable sketches the functioning of different types of screens.

5.a)	Differentiate between constant pressure and constant rate filtration.

b)	The following filtration data were obtained on a 5% by weight of CaCo3 slurry in water using a pressure differential of 4.14 x 104 N/m2 gauge ratio of wet cake is 1.35.  the area of filtration is 0.186 m2 calculate the specific cake resistance and filter medium resistance.

| Filtrate collected (kgs) | 0 | 2.275 | 4.55 | 6.825 | 9.08 | 11.37 | 13.65 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time interval (minutes) | 0 | 0.7 | 0.52 | 0.65 | 0.75 | 1.0 | 10.08 |

Contd….2

Code. No.: 310801				-2-				Set. No: 2

6.a)	Mention the specific application of the following

i)	Change-can mixers

ii)	 Two-arm kneader

b)	Show by means of sketches, the ‘velocity’ and ‘flow patterns’ in agitating liquid with different types of agitators.

7.a)	Explain the problems confronting the designer of an agitating equipment.

b)	Explain the difference between agitation and mixing.

c)	Name the different types of impellers commonly used for agitation of fluids.

8.a)	What is growth rate of crystallization?

b)	Explain ‘∆ L’ law of crystal growth.

$$$

Code. No.: 310801

Set No.

3

III B.Tech I Semester (Supplementary) Examinations June 2003

MECHANICAL UNIT OPERATION -

(Chemical Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

| 1.a) | State the similarities and differences between the properties of solid particles and fluids. |
| --- | --- |
| b) | How are particulate solids classified depending on their flow properties? Give examples. |
| c) | Write in detail about bin storage and bulk storage. |

| 2.a) | Develop an expression for determining the specific surface Aw and number of particles Nw of a mixture of particles. |
| --- | --- |
| b) | The screen analysis shown below applies to a sample of crushed quartz. The density of particles is 2650 kg/m3 and the shape factors are: a = 2 and Φs=0.571. For the material between 4 mesh and 200 mesh in particle size calculate Aw in square millimeters per gram and Nw in particles per gram. |
|  | Mesh no.; Dpi, mm; Mass fraction, Xi; Mesh no.; Dpi, mm; Mass fraction, Xi. 4; 4.699; 0.0000; 35; 0.417; 0.0210. 6; 3.327; 0.0251; 48; 0.295; 0.0102. 8; 2.362; 0.1250; 65; 0.208; 0.0077. 10; 1.651; 0.3207; 100; 0.147; 0.0058. 14; 1.168; 0.2570; 150; 0.104; 0.0041. 20; 0.833; 0.1590; 200; 0.074; 0.0031. 28; 0.589; 0.0538; Pan; ---; 0.0075. |

3.a)	Define the term ‘reduction ratio’.

b)	Give general values of reduction ratio for crusher, grinder, and ultrafine grinders.

c)	What is choke crushing? How is it different from free crushing?

d)	Mention the situations where wet grinding is preferred over dry grinding.

4.	Write short notes on.

a)	Ideal and actual screens.

b)	Capacity and effectiveness of screens.

Contd….2

Code. No.: 310801				-2-				Set. No: 3

5.a)	State and explain "screen effectiveness".

b)	One tonne per hour of dolomite is produced by a ball mill operating in closed circuit with a 100 mesh screen.  The screen analysis (weight) is given below Calculate the screen efficiency.

| Mesh | 35 | 48 | 65 | 100 | 150 | 200 | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Feed | 7.07 | 16.6 | 14.02 | 11.82 | 9.07 | 7.62 | 33.8 |
| Oversize | 13.67 | 332.09 | 27.12 | 20.7 | 4.35 | 2.07 | 0 |
| Undersize | 0 | 0 | 0 | 2.32 | 14.32 | 13.34 | 70.02 |

6.a)	Give a neat sketch of kneader mixer and explain its working and uses.

b)	What are the various types of mixers for free-flowing solids.

7.a)	Explain the term "vortex formation" in agitated tank with neat diagram.

b)	Write short notes on " blending and mixing scale".

c)	Describe with a neat sketch the working of a continuous mixing machine.

8.a)	Explain the process of crystal growth.

b)	Describe the individual and overall growth coefficients in mass transfer operations related to crystallization.

$$$

Code. No.: 310801

Set No.

4

III B.Tech I Semester (Supplementary) Examinations June 2003

MECHANICAL UNIT OPERATIONS

(Chemical Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

| 1. | Write the distinguishing features of the following: a) angle of internal friction and angle of repose. - differential and cumulative analysis of particle sizes. - mixers for dry powders and mixers for pastes in general. |
| --- | --- |
| 2.a) | How does DN differ qualitatively from the volume mean diameter Dv? |
| b) | The screen analysis shown below applies to a sample of crushed quartz. The density of particles is 2650 kg/m3 and the shape factors are: a = 2 and Φs=0.571. Calculate the arithmetic mean diameter DN for the –4 mesh to +200 mesh fraction of the material analyzed in the table given below. |
|  | Mesh no.; Dpi, mm; Mass fraction, Xi; Mesh no.; Dpi, mm; Mass fraction, Xi. 4; 4.699; 0.0000; 35; 0.417; 0.0210. 6; 3.327; 0.0251; 48; 0.295; 0.0102. 8; 2.362; 0.1250; 65; 0.208; 0.0077. 10; 1.651; 0.3207; 100; 0.147; 0.0058. 14; 1.168; 0.2570; 150; 0.104; 0.0041. 20; 0.833; 0.1590; 200; 0.074; 0.0031. 28; 0.589; 0.0538; Pan; ---; 0.0075. |

3.a)	What are various objectives of size reduction in chemical industry?

b)	Name four ways, which are commonly used, for communication of solids.   Discuss each one of them with reference to product size and equipment?

c)	What do you understand by stages of reduction? Explain them with example.

4.a)	Discuss in detail about the principles of cake filtration.

b)	What are different types of filtration and discuss?

5.a)	Give a neat sketch of a kneader mixer and explain its working and uses.

b)	Discuss the importance of Froude number in determining the power consumption of an agitator

c)	Describe a suitable equipment for mixing miscible liquids.

6.a)	Explain in detail the different mechanisms involved in blending of solids.

b)	Draw the Rushton plot for mixing and describe its application in the selection of mixing devices.

Contd….2

Code. No.: 310801				-2-				Set. No: 4

7.a)	What is secondary nucleation? Explain fluid shear nucleation.

b)	Explain the phenomenon of contact nucleation.

8.a)	Discuss the principle of centrifugal filtration.

b)	A rotary drum filter with 25% submergence is to be used to filter a concentrate aqueous slurry of CaCO3 containing 250 kg of solids per cubic metre of water.  The pressure drop is 500 mm Hg. If the filter cake contains 50% moisture (wet basis), calculate the filter area required to filter 10 kg / min of slurry when the filter cycle time is 5 min. Assume that filter medium resistance is negligible.  The temperature during test cycle is 20oC.

Given		:	0	 =	1.2 x 1011 m/kg ; s = 0.25

 	 = 	0 (p)s

For water		:	 = 1 CP ;  = 980 kg/m3

Density of calcium carbonate = 250 kg/m3

$$$