Code No.:310803

Set. No.

1

III-B.Tech I-Semester Supplementary Examinations, June 2003

HEAT TRANSFER

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	The chill wind, which is experienced on a cold windy day is due to increased heat transfer from exposed human skin to atmosphere. Consider a layer of fatty tissue (k= 0.2 W/mK) 3mm thick whose interior surface if kept at 360C. On a calm day the convection heat transfer co-efficient at outer surface is 25 W/m2K but with 50kmph wind it reaches 65 w/m2 K.  If the ambient temperature is –150C in both cases.

a)	What is the ratio of heat loss per unit area of skin for the calm day to that for windy day?

b)	What will be the skin outer surface temperature for calm and windy day?

c)	What temperature would the calm air have to attain to produce the same heat loss as on the windy day?

2.	A certain material has a thickness of 30cm & k of 0.04 W/mC.  At a particular instant in time the temperature distribution with x, the distance from the left face is T = 150 x2 – 30x, where x is in meters.  Calculate the heat flow rate at x = 0 &    x = 30cm. Is the solid heating up or cooling down?

3. a)	How are the local and average convection coefficients for flow past a flat plate are related? Derive the relationship.

b) 	Water at 75C flows through a 0.005 m diameter tube with a velocity of 1m/s. If the tube wall temperature is 25C, make calculations for the heat transfer coefficient. Use the correlation,

St = 0.023 Re 0.2 Pr – 0.667.

The thermo-physical properties of water are:

Thermal conductivity is 0.647 W/(m.K)	Viscosity is 1.977 kg/h.m

Density is 1000 kg/m3				Specific heat 4.187 kJ/(kg.K)

4. a) 	Explain Buckingham- theorem method of dimensional analysis giving an example.

b) 	Distinguish between laminar and turbulent flow in a physical sense.

5.	Write short notes on:

a) 	Critical temperature drop

b)	Transition boiling

c)	Leidenfrost point

d)	Nucleate boiling.

Contd…2

Code No. 310803				.2.			 	Set No. 1

6. a)	Define monochromatic and total emissive power. How is the later related to the absolute temperature?

b)	Describe how the monochromatic emissive power varies with the wave length for emissions from a black body?

c)	At what wave length is the black body monochromatic emissive power the maximum?

7. a) 	Derive the relationship between effectiveness and number of transfer units for a parallel flow heat exchanger.

b)	Determine the heat transfer area required for a heat exchanger constructed from a 2 cm OD tube to cool 26,000 kg/hr of an alcohol solution (Cp =3.81 kJ/kgK) from 90 to 50oC, using H2O which enters at 20oC and leaves at 40oC and for the following arrangements: (1) co current shell and tube (2) counter current shell and tube heat exchanger, alcohol in shell and H2O through the tubes. (Assume correction factor for LMTD = 0.95). Assume that the overall heat transfer coefficient based on the outer tube area is 830 kJ/hrm2K.

8. a) 	What is meant by economy in evaporation and how does it vary with multiple effect evaporation?

b)    Explain in detail about Wet Condenser.

- What are the advantages and disadvantages of multipass heat exchangers over single pass heat exchangers?

d)	Explain the functions of  baffles in shell and tube heat exchangers?

###

Code No.:310803

Set. No.

2

III-B.Tech I-Semester Supplementary Examinations, June 2003

HEAT TRANSFER

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	The cross section of a storm window is as shown. How much heat will be lost through window measuring 1.83m x 3.66m on a cold day when inside and outside air temperature are 295 and 250K respectively?  hi = 20, ho=15 w/m2k.  What will exist across each glass panes?  What will be the average temperature of the air between the glass panes?

Air

Space

0.8 cm

Window glass-0.32 Cm thick

Compare the heat loss through the storm window with the same conditions existing except that the window is a single pane of glass 0.32 cm thick.

2.	A spherical storage tank of 1000mm ID and 6mm thick is covered by an insulation of 20mm thick.  The k of tank wall is 30 times greater than that of the insulation.  It is planned to increase insulation thickness by another 10mm.  In such case what would be the percentage reduction in the heat losses after the additional insulation? Derive the equation used.

3. a)	How is the coefficient of cubical expansion for liquids evaluated? Derive the expression for the same.

b) 	Air at 25C flows normally to 0.03m outer diameter water pipe with a velocity of 1m/s. If the surface temperature of the pipe is 75C, estimate the heat transfer per unit length. Use the correlation Nu = 0.683 (Pr)0.333(Re)0.466. The relevant properties of air are:  Thermal conductivity is 0.0282 W/(m.K). Kinematic viscosity is 1.795  10-5  m2/s. Pr is 0.698.

Contd…2

Code No. 310803				.2.			 	Set No. 2

4. a)	What is a dimensionless number? How and why they are used in heat transfer?

b) 	Differentiate bulk mean temperature and film temperature.

c) 	The sloping roof of a house receives energy by radiation from the sun. The roof surface reaches a steady uniform temperature of 60C, and the ambient air temperature is 32C. For roof dimensions of 9 m by 5.5 m, and a convection heat-transfer coefficient of 11 W/ (m2.K), determine the heat transferred by convection to the air.

5. a)	Write a short notes on condensation of superheated vapors.

b)	Explain the effect of non-condensable gases on rate of condensation.

6.	Define Lambert’s cosine law of radiation and prove that the intensity of radiation is always constant at any angle of emission for a diffuse surface.

7.	A solution is to be concentrated from 10 % to 50% solids in a single effect evaporator. Steam is available at 2 bar absolute. The vapour space inside the evaporator is at 100 mm Hg vacuum. The feed rate to the evaporator is 25,000 kg/hr. Overall heat transfer coefficient is 2840 W/m2oC. Specific heat of solution is 3700 J/KgoC. Calculate the area of  heating surface required if the feed is at 50oC.

8. a) 	With neat diagram explain 2-4 shell and tube heat exchanger and indicate the various parts.

b) 	Discuss the operation and advantages of a multiple effect evaporator.

###

Code No.:310803

Set. No.

3

III-B.Tech I-Semester Supplementary Examinations, June 2003

HEAT TRANSFER

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	A steel locker is used for storing currency notes, which get damaged at the temperature +1800C.  Suggest a suitable insulation (by making necessary assumptions) that will keep the notes safe even in case of fire having broken out.  The flame temperature may assumed to be 20000C.  ksteel = 42 W/mK & ki = 0.8 W/mK.

2.	As spherical tank for storing liquid O2 on the space shuttle is to be made from stainless steel of 0.8m OD & t = 5mm. The BP &  of fusion of liquid O2 are 90 K & 213 kJ/kg respectively.  The tank was installed in a large compartment whose temperature is to be maintained at 240K.  Design a thermal isolation system that will maintain O2 losses due to boiling below 1.0 kg/day.

3. a)	What do you mean by convection? How do you differentiate free and forced convections?

b) 	For a certain forced convection process, the following correlation applies:          Nu = 0.031(Re)0.75(Pr)0.30. Workout the percentage change in the rate of heat flow per degree temperature difference when the original coolant is replaced by another fluid having viscosity equal to two-thirds that of the original coolant. Assume that the other fluid variables and configuration remain the same.

4. a)	Give examples for liquid metals. What are their applications in heat transfer?

b) 	Differentiate plug flow and fully developed flow for flow through tubes.

c) 	In a condenser shell, water flows through 100 thin walled circular tubes (inner diameter 22.5mm and length 5m). The mass flow rate of water is 65 kg/s. Water enters at a temperature of 22C and leaves at 28C. Predict the heat transfer coefficient associated with the water flow.

Properties of water at bulk temperature are as follows:

Specific gravity         :    1.0

Thermal conductivity :   0.6054 W/MK

Specific heat 		:   4.178 KJ/KgK

Viscosity 		:   0.893  10-3 NS/m2.

5. a)	Write a short notes on pool boiling of saturated liquid.

b)	Explain the effect of pressure on maximum boiling heat flux and critical temperature drop.

Contd…2

Code No. 310803				.2.			 	Set No. 3

6. 	State and explain the following laws relating to thermal radiation and temperature of a radiating body:

(a)	Plank’s law

(b)	Stefan Boltzman law

(c)	Wien’s displacement law.

7.	Estimate the increase in heat dissipation rate which could be obtained from a cylinder wall by using 1 pin shaped fin per sq. cm, each having a diameter of    0.5 cm and a height of   2.5 cm. Assume that the heat transfer coefficient between the surface of the cylinder wall or a fin and the surrounding air is 5.25 kJ/hr m2oC, the cylinder wall is at 320oC and the air at 25oC. The wall and the fin are made of aluminium (K = 755 kJ/hr m oC). Assume that the fin is of finite length with an insulated end.

8. a)	With a neat diagram explain the construction and working of any one Compact heat exchanger.

- Discuss in detail the various factors involved in heat transfer analysis in packed beds.

###

Code No.:310803

Set. No.

4

III-B.Tech I-Semester Supplementary Examinations, June 2003

HEAT TRANSFER

(Chemical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	Liquid NH3 at –150C is stored under pressure in an insulated Vessel of ID 15m. Insulation thickness is 50cm outer surface temperature of insulation is 100C.  Calculate the rate and cold loss if the thermal conductivities of insulation at 100C & –150C are 0.12 and 0.11W/mK respectively.

2.	A hot chamber has an 8 cm thick inner layer of fire brick (k = 1.04W/m0C) and a 13 cm outer layer of ordinary brick ( k = 0.69 W/m 0C. The inside & out side temperatures are 400 & 750C, respectively. Considering that the outer surface temperature is too high, it is decided to apply a 5 cm thick layer of plaster on the outer surface. On doing so, the outer skin surface temperature reduced to 600C & the rate of heat loss decreases by 250 W/m2 of the wall area. Calculate the k of the layer of plaster.

3. a)	Differentiate between Reynolds analogy and Prandtl analogy. What is the significance of the Colburn j-factor?

b) 	A horizontal cylinder 0.025 m diameter and 0.5 m long is suspended in water at 20C. Calculate the rate of heat transfer if the cylinder surface is at 60C. Use the following correlation:

Nu = 0.53 (Gr.Pr) 0.25

The relevant physical properties of water at the mean film temperature 40C are:

Thermal conductivity is 0.63 W/(m.K)	Viscosity is 2.35 kg/h.m

Density is 992 kg/m3				Pr is 4.3

4. a) 	What is Prandtl number? Explain its physical significance in detail.

b) 	Explain how film coefficient is evaluated for heat transfer by forced convection in laminar flow, turbulent flow, and transition flow of fluids through circular tubes.

5.	Derive zuber’s analytical expression for the peak heat flux in nucleate boiling.

6. a)	Explain in detail how the combined heat losses by conduction, convection and radiation can be estimated.

- Give three practical examples for the above situation.

c)	Write a note on radiation in film boiling.

Contd…2

Code No. 310803				.2.			 	Set No. 4

7. a)	A solution containing 2.5 kg salt per 100 kg of solution is evaporated to 15% solids concentration inside an evaporator. Feed temperature is 20oC and the boiling point in the evaporator is 104oC. It requires 2260 kJ to evaporate 1 kg of water from the boiling solution whereas steam gives only its latent heat of vaporization, 2200 kJ/kg. How many kg of steam is required per kg of feed?  Given:

Heat capacity of feed solution = 4.10 kJ/kgoC

Heat capacity of the solution  = 3.77 kJ/kgoC.

b)	Draw a neat sketch of falling film evaporator and explain its principle of operation and applications.

8. a)	Discuss the various factors which have an effect on heat transfer capacity of an Evaporator and on its economics.

b)	How are heat exchangers classified? and sketch the temperature distribution in fluids in condensers and evaporators?

###