Code No: 310804.

Set No.

1

III-B.Tech  I Semester Supplementary Examination - June 2003

MASS TRANSFER OPERATIONS - I

(Chemical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	What is the effect of temperature and pressure on the diffusivity of gasses?

b)	Determine the diffusivity of carbon monoxide through a gas mixture in which the 	mole fractions of each component are yO2 = 0.20, yN2 = 0.70, yCO = 0.10. The gas 	mixture is at 298K and 2.0625 x  105 N/m2 total pressure.

DCO-O2 = 0.185 x 10-4 m2/s at 273K, 1.01325 x 105 N/m2.

DCO-N2 = 0.192 x 10-4 m2/s at 288K, 1.01325 x 105 N/m2.

2.a)	How are mass transfer operations different from mechanical unit operations? 	Explain giving examples.

b)	Give two examples of equimolar counter diffusion in a gaseous system.

c)	In an oxygen-nitrogen gas mixture at 1.14575 x 105 N/m2 and 350C, the 	concentration of oxygen, at two planes 5 mm apart are 20% and 30% by volume 	respectively. Calculate the amount of oxygen diffused in 3 hours when eqimolar 	counter diffusion takes place. The diffusivity is 0.185 x 10-4 m2/s.

3.	What are local and overall mass transfer coefficients? Describe their significance 	and the relation between the coefficients.

4.	For a bubble cap column, explain briefly:

i)	Relative froth density.

ii)	Holdup of liquid.

iii)	Residence time of vapor and liquid.

iv)	Flooding.

v)	Foaming.

( Contd…2 )

Code No: 310804.			.. 2..				Set No: 1

5.	Ammonia is to be removed from a mixture of 20% NH3 and 80% air by counter 	current scrubbing with water at 1 atm and 200C. The scrubbers is to be designed 	to remove 99% of ammonia in entering gas. Calculate the minimum water rate 	and the number of theoretical plates necessary for absorption if 1.2 times the 	minimum water rate is used for a gas rate of 3500kg/hr.

Equilibrium data:

Partial pressure 	12	18	32	50	70	166

of  NH3, mmHg

gms NH3 per 		0.2	0.3	0.5	0.75	1	2

10 gms H2O

6.a)	Explain wet bulb temperature and adiabatic saturation temperature with all related 	equations.

b)	Explain about the humidity chart.

7.	Wet air with a dew point of 350C is compressed from 1 atmosphere pressure to 	10-atmosphere pressure in a compressor. After compression only 25% of original 	water remains in the air. Compute

i)	The final temperature of wet air after compression

ii)	How much original air (in volume) is required to make 10m3 of the 			compressed air?

iii)	The relative humidity of compressed air.

8.a)	Explain the terms: equilibrium moisture, critical moisture, falling rate period, 	constant rate period.

b)	Explain with a neat diagram the working of a counter current rotary drier.

~ ~ ~ ~

Code No: 310804.

Set No.

2

III-B.Tech  I Semester Supplementary Examination - June 2003

MASS TRANSFER OPERATIONS - I

(Chemical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	For a binary mixture of A and B show that DAB = DBA for equimolar counter 	diffusion.

b)	A mixture of He and N2 gas is contained in a pipe at 298K and

1.01325 x 105 N/m2 total pressure, which is constant throughout. At one end of 	the pipe at point 1 the partial pressure pA1 of He is 0.60795 x 105 N/m2  and at the 	other end 0.2m, pA2 = 0.20265 x 105 N/m2.  Calculate the flux of He at steady 	state if DAB of He-N2 mixture is 0.687 x 10-4 m2/s.

2.a)	What is the effect of temperature and pressure on the diffusivity of liquids?

b)	Estimate the liquid diffusion coefficient of ethanol, C2H5OH, in a dilute solution 	of water at 100C. The atomic volume of C, H and O are 14.8 x 10-3, 3.7 x 10-3 m3 	and 7.4 x 10-3 m3/ kg mole respectively. At 100C, the viscosity of a solution 	containing 0.05 mole of alcohol per 10-3 m3 of water is 1.45 x 10-3 kg/m.s, B = 	2.6 for water.

3.	Discuss the following Mass Transfer Theories:

i)	Film Theory.

ii)	Boundary Layer Theory.

iii)	Penetration Theory.

iv)	Two Film theory.

4.	Explain in detail:

i)	Liquid Hold up in Packed Towers.

ii)	Mass Transfer in Packed Towers.

iii)	End effects and Axial mixing.

( Contd…2 )

Code No: 310804.			.. 2..				Set No: 2

5.	Ammonia is to be recovered from its mixture with air by scrubbing with pure 	water in a multistage continuous counter current apparatus. 99% of ammonia 	(mole basis) present in the inlet mixture, of initial composition 10% by weight, 	is to be recovered. Determine the minimum rate of water flow required and the 	number of practical stages if Murphree stage efficiency based on gas composition 	is 80% for 1.6 times the minimum water flow rate.

Equilibrium data:

Partial pressure 	12	18	32	50	70	166

of  NH3, mmHg

gms NH3 per 		0.2	0.3	0.5	0.75	1	2

10 gms H2O

Temperature and pressure of operation are 200C and 1 bar respectively.

6.	What are the most important methods for determining humidity? Explain any 	three methods in detail.

7.	Air at a temperature of 300C and a pressure of 750mmHg has a percentage 	humidity of 60. Calculate the pressure to which the air must be compressed at 	constant temperature in order to remove 90%of the water present.

8.a)	With the help of a typical drying rate curve for constant drying conditions explain 	the terms: Constant rate period, falling rate period, equilibrium moisture content 	and critical moisture content.

b)	Write a short note on spray drier with a neat diagram.

~ ~ ~ ~

Code No: 310804.

Set No.

3

III-B.Tech  I Semester Supplementary Examination - June 2003

MASS TRANSFER OPERATIONS - I

(Chemical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.	Derive the continuity equation for a binary mixture by considering mass flux 	through a differential control volume.

2.a)	Draw the concentration profiles in terms of pressure versus distance for the 	following two cases:

i)	Steady state diffusion of A through non diffusing B and

ii)	The steady state equimolar counter diffusion.

also write the equations for both the cases.

b)	Oxygen (A) is diffusing through carbon monoxide (B) under steady state 	conditions, with the carbon monoxide nondiffusing. The total pressure is

1 x 105 N/m2 and the temperature is 00C. The partial pressure of oxygen at two 	planes 2mm apart is, respectively, 13,332 and 6,666 N/m2. The diffusivity for the 	mixture is 1.87 x 10-5 m2/s. Calculate the rate of diffusion of oxygen in k mol/s 	through each square meter of the two planes.

3.	Describe the diffusion between phases and show how equilibrium distribution 	curve varies as a function of temperature and pressure taking a system of liquid 	water and a gaseous mixture ammonia and air?

4.	It is desired to eliminate traces of H2S from effluent gases of a topping still, so 	that they can be conveniently used as a fuel.

The original concentration of 2.5% by volume of H2S is to be reduced to 0.2% by 	volume, by scrubbing the gas mixture with hot nonvolatile hydrocarbon oil in a 	multistage continuous counter current equipment. Determine the number of ideal 	stages required if the plant is operated using 1.5 times the minimum liquid rate. 	Henry’s law is applicable for this system and the constant is 0.25 atmospheres per 	mole fraction.

5.a)	Explain the relation of adiabatic saturation temperature to wet bulb temperature.

b)	A sample of air is to be humidified and warmed. Describe two possible methods 	of achieving this by using the humidity chart and the equipment to be employed 	for the two methods.

( Contd…2 )

Code No: 310804.			.. 2..				Set No: 3

6.	A mixture of benzene and dry air at a temperature of 300C and a pressure of 	760mmHg is found to have a dew point of 150C. Calculate:

i)	Percentage volume of benzene.

ii)	Moles of benzene per mole of air.

iii)	Weight of benzene per unit weight of air.

Data: Vapor pressure of benzene at 300C = 120mmHg and at 150C = 60mmHg.

7.a)	Define the following:

i)	Moisture content both dry and wet basis.

ii)	Bound and unbound moisture.

b)	How are equipment used for drying classified? Explain the working of a tray drier 	with a neat diagram.

8.	Write short notes on:

a)	Venturi Scrubbers.

b)	Wetted wall towers.

c)	Spray Towers.

d)	Liquid distribution and factors that affect liquid distribution.

~ ~ ~ ~

Code No: 310804.

Set No.

4

III-B.Tech  I Semester Supplementary Examination - June 2003

MASS TRANSFER OPERATIONS - I

(Chemical Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	State and explain Fick’s first law of diffusion.

b)	Explain the phenomenon of molecular diffusion with the help of the kinetic theory 	of gases. Give an example.

c)	 Distinguish between molecular diffusion and eddy diffusion.

2.a)	Applying Fick’s law of diffusion, derive an equation for steady state radial 	diffusion through a solid cylinder of inner and outer radii a1 and a2 respectively 	and of length l. State clearly the assumptions made.

b)	Hydrogen gas at 2.0265 x 105 N/m2 and 250C, flows through a pipe made of 	unvulcanized neoprene rubber, whose ID and OD are 25 and 50 mm, 	respectively. The solubility of the hydrogen is reported to be

0.053 m3 (STP)/ (m3) (1.01325 x 105 N/m2 ), and the diffusivity of hydrogen 	through the rubber to be 1.8 x 10-10 m2 /s. Estimate the rate of loss of hydrogen by 	diffusion per meter of pipe length.

3.	Explain in detail with appropriate diagrams, the experimental techniques to 	measure mass transfer coefficients and diffusivities.

4.a)	Define Tray Efficiency?

b)	Write short notes on:

i)	Point Efficiency.

ii)	Murphree Efficiency.

iii)	Entrainment.

iv)	Overall tray Efficiency.

( Contd…2 )

Code No: 310804.			.. 2..				Set No: 4

5.	Ammonia is to be recovered from NH3-N2 mixture (initial composition 10% by 	weight) in a counter current equipment using pure water as the absorbing 	medium. The entire operation is carried out under isothermal conditions with 	temperature and pressure being 200C and 1atm. abs. respectively. The equipment 	is to be designed for 95% recovery by weight. The gas mixture enters at a rate of 	3000kg/hr. Determine the minimum water rate required and the number of ideal 	stages required if the operation is carried out using a water rate 30% grater than 	the minimum.

The equilibrium data for this system at 200C is given below:

Partial pressure 	12	18	32	50	70	166

of  NH3, mmHg

gms NH3 per 		0.2	0.3	0.5	0.75	1	2

10 gms H2O

6.a)	Define   	i) Molar humidity   ii) Relative saturation   iii) Adiabatic saturation 	temperature.

b)	Derive an expression to show the relation between wet bulb temperature and gas 	humidity.

c)	Explain any two methods to determine humidity.

7.	Air at a temperature of 600C, a pressure of 745mmHg and a percentage humidity 	of 10 is supplied to a drier at a rate of 1500m3. Water is evaporated in the drier at 	a rate of 25Kg/hr. The air leaves at a temperature of 350C and a pressure of 	742mmHg. Calculate:

i)	Percentage humidity of air leaving the drier.

ii)	Volume of wet air leaving the drier per hour.

8.a)	Draw a typical drying rate curve for batch drying under constant drying 	conditions and explain various regions in it.

b)	Write a short note on drum driers with a neat diagram.

~ ~ ~ ~