Code No: 310805

Set No.

1

III - B.Tech. I Semester Supplementary Examination, June 2003

PROCESS INSTRUMENTATION

(Chemical Engineering)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the following.

(a) Span (b) reproducibility (c) sensitivity  (d) static error (e) measuring lag

2.	Explain in detail the desirable properties of industrial thermocouples.

3.	Why PH ion concentration is measured.  Explain the most common industrial method of measuring PH.  Find the relation between composition and PH value.

4.a)	What are the common manometric fluids used in the manometers?

b)	Explain in detail the working of a simple U tube manometer?

5.a)	What precautions must be taken while measuring pressure of hot condensate gases or steam and the gage is installed above the main line?

b)	With a neat diagram explain the working of a thermal conductivity gage for the measurement of pressure.

6.a)	What is the direct measurement of liquid level?

b)	Describe various methods of direct measurement of liquid level.

7.a)	When is venturi preferred to orifice?  Explain about different types of venturimeters with figures.  How is venturimeter specified?

b)	A venture meter is to be installed to measure flow rate o water.  The maximum flow rate is expected to be 50m3/h at 250 C.  Calculate A) the throat diameter of venturimeter (b) the power to operate the meter at full load the pressure recovery is 90 percent of the differential pressure?

Data:  Diameter of pipe line= 10cm

Density of water       =  998 kg/m3

Pressure differential across venturiment = 1.2 m of Hg

Venturi coefficient = 0.98

8.a)	In an industry, it is required to continuously record the viscosity of a liquid.  Suggest a suitable viscometer.  Describe its working and construction with a figure.

b)	Describe the working of a cone and plate viscometer with a figure.  What is the main advantage of such viscometer over that meters?

&&&&&

Code No: 310805

Set No.

2

III - B.Tech. I Semester Supplementary Examination, June 2003

PROCESS INSTRUMENTATION

(Chemical Engineering)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the following.

(a) Accuracy	(b) drift (c) dead-zone	 (d) hysteresis 	(e) dynamic error

2.	Explain the characteristics of most commonly used industrial thermocouples.

3.	Why are density and specific gravity measured in industry?  Give its uses with examples.  Explain the methods of their measurement.

4.a)	What is the device employed to measure the atmospheric pressure?

b) 	What is its accuracy?

c) 	With a neat diagram explain its working.

5.a)	Write a note on pressure switches.

b) 	With a neat diagram explain how a Purged System operates for the measurement of pressure of Viscous process fluids

6.a)	Write the relation between head and float displacement.

b)	Explain differential pressure manometer for measuring liquid level.

7.a)	Describe the construction of a venturimeter with a figure showing clearly the pressure tap connections.   Explain the salient features of the working of meter.

b)	To measure flow rate of ail flowing through a 40cm pipe, a venturimeter having athroat diameter of 15cm is installed.  The inlet pressure and temperature are 140 kpa absolute and 200C respectively.  The pressure at the throat is 119 kpa absolute.  Calculate the mass flow rate of air

Data: C0=0.96,  R=286.7 J/Kg k

=1.4.	Molecular weight of air = 29

The table for expansion factor Y is given below

p/p1		Venturi throat 	dia / pipe dia.

0.3		0.4		0.5

0.15		0.915		0.914		0.910

8.a)	It is desired in a plant to continuously monitor the viscocity of a liquid for controlling a process.  Suggest a suitable visometer for the purpose and describe its operation and construction with a figure.

b)	Write short notes on the following with figure.

(i)  Lobed impeller meter			(ii) Tilting trap meter.

&&&&&

Code No: 310805

Set No.

3

III - B.Tech. I Semester Supplementary Examination, June 2003

PROCESS INSTRUMENTATION

(Chemical Engineering)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Present the dynamic response of first order type Instruments to

(a) step change   (b) linear change   and  (c) sinusoidal change.

2.	Why lead wires are preferred as connecting wires in thermocouples. Explain.

3.	How many methods are there that use differential pressure for measurement of viscosity.  Explain on such methods that gives continuous indication.

4.a)	What do you understand by the term Temparature correction Factor in progress measurement?

b)	Explain with a neat diagram the working of an inclined the manometer.

5a)		What devices are employed as Pressure standards

b) 	With a neat diagram explain the working of a Diaphragm seal in the measurement of pressure of corrosive liquids.

6.	Describe  the bubbler  system used  for pressure measurement in open vessels, with a neat diagram.

- An orifice meter equipped with flange taps is to be installed to measures the flow rate of crude oil to a cracking unit.  The oil is flowing at 380C through a 10cm pipe.  The maximum flow rate of oil is 80m3/h.  Mercury is used as manometer liquid and glycol is used as sealing liquid in the leads.  Calculate a) the diameter of orifice and b) power loss.

Data:  Density of mercury = 13600 kg/m3

Density of glycol   = 1100  kg/m3

Reading of the manometer = 0.76m

Discharge coefficient = 0.61.

The power loss and  are correlated by equation power loss = m+c.

It is given that =0.4, power loss is 0.81 and =0.6,

Power loss = 0.62.

8.a)	It is required to continuously measure the rate of feeding of coal to a boiler.   Mention the names of a few weighing meters.  Describe the operation of one of them, with a figure.

b)	Write short notes on the following with figures.

(i)  Nutating disk meter	(ii)	Rotary vane meter.

&&&&&

Code No: 310805

Set No.

4

III - B.Tech. I Semester Supplementary Examination, June 2003

PROCESS INSTRUMENTATION

(Chemical Engineering)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Present the dynamic response of first order type Instruments to

(a) step change   (b) linear change   and  (c) sinusoidal change.

2.	Describe the differences in Peltier heat and Joule heat. Why is the Joule heat not 	important in theory of thermocouples.

3.	Describe the methods of measurement of moisture in paper, textiles and Lumber.  Explain the principle.

4.a)	Differentiate gauge pressure, absolute pressure and vaccum.

b)	With a detail the working of an  Enlarged leg Float U tube manaomenter.

5.a)	What should be done while measuring the pressure of a corrosive material and the gage is not designed for that?

b)	With a neat diagram explain the working of a Force balance type pneumatic differential pressure transmitter.

6.a)	What are the two methods for measuring density ?

b)	Explain  liquid level method  for measuring specific gravity with a neat diagram.

7.a)	Natural gas having a density of 0.6kg/  and a  viscosity of 0.011X10-3 NS/m2 is flowing through a 15cm pipe.  The gas is at 370C and 1.5 X 105 N/M2 at upsteam tap. A standard shard edged orifice of 5cm diameter, with flange type taps is installed in the pipe to measure the flow rate.  The manometer reads 1.2 m of water at 250 C.  The ratio of specific heats for natural gas is 1.3.   Calculate the rate l flow of gas though the line in m3/h based on pressure of 1.03X105 N/m2 and a temperature of 250C.  Density of water may be taken as 1000 kg/m3.

b)	Discuss the variation of discharge coefficient of an office meter with Reynolds number and ration of office to pipe diameter with the help o sketch.

8.a)	State the basic working principle of quantity meters.

b)	Name are two kinds of quantity meters.  Explain the principle of operation of each of them.

c)	Describe working and construction of reciprocating piston meter and sealed drum meter.

&&&&&