Code No.:311003

Set No

1

III-B.Tech. I-Semester Supplementary Examinations June, 2003

INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	Explain the concepts of contacting and non-contacting type of length measurements.

2.	What is protection comparator? Show a sketch to illustrate the principle and give the type of works for which this instrument is specially suitable.

3.a)	What is translational velocity?

b)	How will you measure it?

c)	List out different methods to measure it.

d)	Describe the basic principle of any two methods.

4.a)	An accelerometer has a Seismic mass of 0.05kg and a spring constant of 3 x 103 N/m maximum measurement. Maximum mass displacement is ±0.02m(before the mass hits the stop). Calculate Maximum measurable acceleration and natural frequency.

b)	A seismic instrument has a natural frequency of 4 hz and a damping ratio of 0.66. 
          	If the system is excited by a frequency 6Hz, determine the error due to the   	proximity of excited frequency with natural frequency of the instrument.

5.	With the help of neat sketch explain the operation of strain gage load cell. Derive the expression for output voltage.

6.	A torsion bar is made of mild steel and has a diameter of 25mm. The shear modulus of mild steel is 80GNm2. Calculate the value of shear sensitivity. Suppose a torque of 104N−m is applied to the bar, calculate the value of shear strain angle. If the bar is notched to 15 mm diameter, what is the value of shear sensitivity and the shear angle for the above torque?

7.a)	Distinguish between density and specific gravity.

b)	Suggest some methods that may be used to measure specific gravity.

8.a)	What is Psychrometer and explain about sling Psychrometer?

b)	Explain briefly about aspirating Psychrometer.

^^^

Code No.:311003

Set No

2

III-B.Tech. I-Semester Supplementary Examinations June, 2003

INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	Describe in detail about absolute and relative length measurement situations.

2.a)	Why parallel light must be used when projecting cylindrical objects in a project?

b)	In a pres twitch fluid gauge , the diameter of cone is D and the diameter of tube is d. show that its magnification approximately is given by D²/ 13d².

3.a)	Mention the devices used for measuring speed.

b)	Give its classification.

c) 	Explain the basic principle of the simplest type mechanical tachometer.		              d) 	Explain the concept of measurement of speed by means of centrifugal force.

4.a)	A disc mounted on the shaft of a machine has 12 pattern points. The number of 
          	flashes projected on the disc by a Stroboscope is 6000 in a minute. Find the speed 
          	of the machine if the disc  appears stationary and has single image of 12 points.

b)	A stroboscope is directed at a rotating disc having five equi spaced radial lines on 
         	it. The highest flashing  frequency at which a true pattern is observed is 2000 
         	flashes  per minute. Give the other two flashing frequencies which would produce

i) a  5 line pattern and 	ii) a 10 line pattern.

5.	Explain the measurement of force using transducers

a)	L.V.D.T.

b)	Piezoelectric transducer.

6.	A strain gage is bonded to a steel beam 0.25m long and has a cross-sectional area of 0.4×10−3square meter. Young’s modulus of elasticity for steel is 207 GNm2. The strain gage has an unstrained resistance of 240 and a G.F of 2.20.when the load is applied, the gauge’s resistance changes by 0.013 . Calculate the change in length of the steel beam and the amount of force applied to the beam.

7.a)		What are the reasons for consistency variations.

b)		Explain the variable which will affect the consistency measurement.

8.a)	Explain briefly the calculation humidity of mixture from dew-point      temperature.

b)	Describe about aluminium oxide hygrometer.

^^^

Code No.:311003

Set No

3

III-B.Tech. I-Semester Supplementary Examinations June, 2003

INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	Describe in detail about standards and calibrations of length?

2.a)	Explain the difference between a comparator and measuring instrument. State the fields of applications of comparators.

b)	Explain with a neat sketch any type of comparator  you know and show clearly the advantages of magnification adopted.

3.a)	How will you measure rotational speed by using electric tachometer?

b)	Give its classification.

c)	Explain the basic principle behind the same.

4.a)	The speed of a shaft rotating at 2800 rpm is measured using a stroboscope.        	The stroboscope dial is slowly turned  from setting of 4320 rpm to 1400 rpm         	corresponding to flash rate of 96 to 24 per second.

i)Indicate the speed settings which give single, double and triple steady images

ii)what is the observation when the flashing rate is 50 per second.

b)	In order to measure a speed higher than flashing rate of stroboscope, the flashing 
rate was gradually reduced from the highest, noting all the speeds at which single 
image is observed of a mark on the rotating object. These speeds are 20000, 5000, 12000 and 10000 rpm, calculate the actual speed.

5.	Explain the following methods of force measurements

(a) Electromagnetic balance method	(b) Hydraulic load cell

(c) Pneumatic load cell.

6.	A dynamic force-measuring device can be represented by a second order system, with an effective mass of 200g, damping ratio of 0.7 and an undamped natural frequency of 10HZ.

a)	For a unit step force input, find the peak over shoot and the time taken for the output to be with in 5% of its steady state value.

b)	If the force to be measured were to increase linearly with time, find the %error after 5sec.

7.a)		Explain briefly about viscosity relationships.

b)		What is kinematics viscosity?

c)		Explain the reasons for consistency measurement.

8.a)		What are the various methods of measurement of relative humidity?

b)	Explain briefly about electrical hygrometers.

^^^

Code No.:311003

Set No

4

III-B.Tech. I-Semester Supplementary Examinations June, 2003

INDUSTRIAL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours						                    Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	Mention the different methods of length measurement for industrial use and explain the same.

2.	What are different methods of measuring angles? State the accuracy of each method suggested.

3.	How will you measure relative velocity? Discuss briefly about the basic Principle of different methods to measure the relative velocity.

4.a)	An LVDT is used in an accelerometer to measure seismic mass displacement. The 
          	LVDT and signal conditioning outputs are  0.31mv/mm with a ±20mm core           	displacement. The spring constant  is 240N/m and the core mass is 0.05kg. Find

i)	Relation between acceleration in m/s2  and the output voltage.

ii)	Maximum acceleration measurable, and

iii)	Natural frequency

b)	An accelerometer gives an output of 14mv per g where g = 9.81ms-2.               	Design signal conditioning that provides

- A velocity signal scaled at 0.25v/ms-1 and

ii)	Determine the gain of the system and feedback resistance ratio.

5.	Draw and explain the circuit of a digital frequency meter. What are the different methods used for high frequency determination.

6.	For the measurement of torque, four 120  resistance strain gauges having gauge factor 2.0, are mounted on a shaft, 5 cm in diameter, at 45 to the axis of the shaft and are connected in a bridge configuration to get maximum sensitivity.  For the shaft material, the Young’s modulus is 2.0 x 1011 N/m2 and Poisson’s ratio is 0.3.  Speed of the shaft is 1500 rpm. Excitation voltage to the bridge is 4.0 V.  If the output voltage of the bridge is 3.5 m V, determine

(a) torque transmitted; 	(b) power transmitted.

7.	Explain the following terms:

(a) Fluidity.  			(b) Kinematics viscosity.

(c) Specific viscosity. 		(d) Relative    viscosity.

Contd…2

Code No.:311003 			-2-				          SetNo.4

8.a)	Explain how resistive transducers can be used for measurement and control of  	relative humidity. Describe its advantages and disadvantages.

b)	Describe the theory and working of a sling psychrometer used for the     measurement of relative humidity. What are its limitations?

^^^