Code No: 311004

Set No.

1

III-B.Tech  I-Semester Supplementary Examination - June 2003

PROCESS CONTROL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Write the differential equations of the following liquid level process and hence 	derive the transfer function of the system.

m                                    u2

h2

C2		R2

u1

h1	  R1

C1

b) 	Say whether the above system is interacting or Non-interacting. Justify your

answer.

c) 	What are the effective time constants of this liquid level process?

2.a) 	Differentiate between a “Servo operation” and “Regulator operation”.

b)	Give a typical example for servo operation and explain.

c)	Explain the regulator operation with an example.

3.a) 	Discuss relative advantages and disadvantages of the proportional, integral and derivative control actions.

b) 	Discuss  the  characteristic  effects  of  a  proportional  controller  on  the  closed  loop  response  of   a  process.

4.a) 	Explain  with  analytical  expression,  the  concept  of   a  pure  controller  mode  which is  anticipatory  in  nature.  List  its  characteristics.

b)	Why  a  pure  derivative  is  not  preferred?  Draw  the  derivative  controller  output  for  the error  given  below.

e

+

_

t

(Contd…2)

Code No: 311004.			..2..				Set No: 1

5.a)	Explain the following terms as applicable to system evaluation with 	necessary graphs.

(i) Stability		(ii) measure of quality

b)     Discuss the quarter – amplitude criterion to evaluate the response.

6.	Explain the reflections problems in

a)  TTL integrated circuits.							b) C-MOS Integrated circuits.

7.	Explain the following Etching techniques with sketches.

a)	Simple batch – production etching.

b)	Continuous – feed etching.

c)	Etching with open – loop regeneration.

8.	a) 	Explain the principles of cascade controller tuning.

b) 	State the fundamental difference feed forward and conventional feed back 			control.

c) 	Why is a process model required for feed forward controller?

~ ~ ~ ~

Code No: 311004

Set No.

2

III-B.Tech  I-Semester Supplementary Examination - June 2003

PROCESS CONTROL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Derive the process differential equations and the transfer function for the 	following two capacity level process.

m

h2				h1

C2		R2			         R1

C1	                    q0

b) 	This level process is an interacting system – say “TRUE” or “ FALSE”.

Support your answer with valid reasons.

c) 	Define the system time constants of the above process.

2.   (a) Differentiate between a controlled variable and a manipulated variable.

- Explain one process control loop with a neat diagram.

- List all the variables involved in this loop.

- Name the controlled variable and the manipulated variable in this loop.

3.a) 	what is meant by differential gap and how it is related to the performance of the final control element?

b) 	Define proportional band. Explain the relation between proportional gain, proportional band and offset error.

c) 	With a neat sketch explain the integral controller mode action.   Summarize the characteristics.

(Contd…2)

Code No: 311004.			..2..				Set No: 2

4.a)	Describe  the  output  of  a  three  mode  controller  for  an  assumed  error  variation.  Discuss its features.

b)	Specification  of  a  PI  controller  are  Kp=7,  Ki= 2 sec-1  and  PI(0) (controller  output  at  t=(0)=24%.  Plot  the  controller  output  as  the  function  of  time  for  the  given  error.

e

1

0	    1	        2 			t

5.a)	Describe the following types of disturbances with relevant sketches.

(i) Transient        (ii) Set point changes       (iii) Load changes.

b)	Write short notes on the following errors:

(i) IAE     (ii) ISE      (iii) ITAE

6.	Write about PCB layout check related

a)	General Consideration.

b)	Mechanical considerations.

c)	Electrical considerations.

7.a)	Explain Etching with closed – loop regeneration.

b) 	What are the steps taken to minimizing pollution during the Etching Process?

8.a)	Explain the closed loop characterization of cascade control system.

b)	Explain the primary and secondary loop gain adjustment in a cascade control 	system.

~ ~ ~ ~

Code No: 311004

Set No.

3

III-B.Tech  I-Semester Supplementary Examination - June 2003

PROCESS CONTROL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Write differential equation for the following single capacitance process.

m

h

C		R

b) 	The above single capacitance level process has a normal operating head of 1.2 	meter and a normal 	value of outflow of 3375 cubic centimeters per second. The 	cross section area of the vessel is 0.54 square meter. If the resistance to the 	flow is parabolic, determine the time constant of the system.

c) 	Show that the time constant of the above process is proportional to the time 	required to change the fluid in the vessel.

2.	Differentiate between a continuous process and a batch process. Support your 	answer explaining with one continuous process and one batch process with neat 	diagrams.

3.a) 	Explain the principle of a discontinuous controller mode. Mention few applications.

b) 	Distinguish between multi position controller mode and floating control mode with neat sketches.

(Contd…2)

Code No: 311004.			..2..				Set No: 3

4.a)	Discuss   the   relative   advantages   and   disadvantages   of  various  composite  control  modes.

b)	A  three  mode  controller has  Kp=2.5,  Ki=2.5 sec-1 , Kd=2sec  and  PI(0) (controller  output  at  t=0 ) =30%.  Plot  the  controller  output  as  function of time  for the  following  error.

error %

2

0		 2	          4		      t

5.a)	What are the studies required in a control loop of a process control 	system?

b)	What are the three different control actions that are mostly used in the process 	control system?  How do these actions take place?

6.a)	Write about component placing and mounting in PCB.

b)	Explain about cooling requirements and packaged density related to PCB.

7.a)	Briefly discuss different trends in etching machine designs.

b) 	Briefly compare the different Etchants.

8.a)	What is the fundamental characteristics of cascade control.

b) 	Explain the basic principles of cascade control with one industrial application.

~ ~ ~ ~

Code No: 311004

Set No.

4

III-B.Tech  I-Semester Supplementary Examination - June 2003

PROCESS CONTROL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours							Max.Marks:80

Answer any five questions

All questions carry equal marks

---

1.a)	Derive the transfer function H(S)/Q(S) for the liquid level system shown below. 	The resistances are linear. H and Q are deviation variables. Derive the transfer 	function of the system.

Q  m3/min

A=2 m2

h

R1=2					    R2=5

- What modification you have to make in the transfer function if a disturbance U occurs in inflow.

- Write the expression for the response if the above system experiences a step change in Q.

2.a)	What is called a “Dead End System”?  Explain one such system with neat 	diagram.

b)	Write the differential equation of this system and get the transfer function.

3.a) 	Explain with neat sketches, the characteristics of two position and three position controller modes.

b) 	What is cycling? Suggest a suitable modification to overcome cycling.

(Contd…2)

Code No: 311004.				..2..				Set No: 4

4.a) 	Briefly discuss the characteristics of proportional control mode. Mention the type of processes for which proportional controller is most suited.

b) 	A proportional controller has a gain of 3. Plot the controller output for the error given below if

Po = 50%. (Po – controller output with no error)

-3

2

0

1        2      3         4

5.a)	Write the equations for the proportional, integral and derivative actions and 	explain their operations.

b)	What are the stability methods adopted to PID controller for optimum 	settings.

6.	Explain about signal conductor layout for

a)	High frequency amplifiers / Oscillators.

b)	Precision differential amplifiers.

7.	Explain the following:

a)	Limitations of tin – lead plating.

b)	Roller tinning.

c)	Centrifugal tinning.

d)	Hot – air leveling.

8.a) 	State the necessity of split range control for a process.

b) 	Explain the principles of split range control for multiple inputs by taking simple 	examples.

~ ~ ~ ~